00000-003FF  Interrupt Vector Table (256 double words):

00000 INT 00H  Divide by Zero Interrupt Handler

00004 INT 01H  Single Step Interrupt Handler

00008 INT 02H  Non-Maskable Interrupt (Memory Parity or I/O Error)

0000C INT 03H  Breakpoint

00010 INT 04H  Arithmetic Overflow Handler

00014 INT 05H  Print Screen

00018 INT 06H  Reserved - Invalid instruction code

0001C INT 07H  Reserved - Coprocessor not present

00020 INT 08H  Timer Interrupt Routine (18.21590 /sec)         IRQ00 \

00024 INT 09H  Keyboard Service Routine                        IRQ01  |

00028 INT 0AH  VGA Retrace Interrupt (and AT Slave Interrupts) IRQ02 Master

0002C INT 0BH  Serial Device 2 Service Routine                 IRQ03  Int.

00030 INT 0CH  Serial Device 1 Service Routine                 IRQ04 Contr.

00034 INT 0DH  XT Hard Disk Interrupt Routine                  IRQ05  |

00038 INT 0EH  Diskette Interrupt Routine                      IRQ06  |

0003C INT 0FH  Parallel Port Service Routine                   IRQ07 /

00040 INT 10H  Video Services

00044 INT 11H  Equipment Check

00048 INT 12H  Memory Size Check

0004C INT 13H  Diskette and Hard Disk I/O

00050 INT 14H  RS-232 Service Call

00054 INT 15H  System Services Calls (XT Cassette Tape Drive I/O)

00058 INT 16H  Keyboard Call

0005C INT 17H  Printer I/O Call

00060 INT 18H  Basic ROM Entry (startup)

00064 INT 19H  Boot Loader: Implement System (IPL) from disk

00068 INT 1AH  Time of Day Call

0006C INT 1BH  Keyboard Break Address (Ctrl-Break)

00070 INT 1CH  User Timer Interrupt

00074 INT 1DH  Monitor ROM Ptr: for 6845 Video Inititialization(Video Parm Tbl)

00078 INT 1EH  Disk Control Table  Pointer (Disk Parameter Table)

0007C INT 1FH  Alphanumeric Character Pattern Table Pointer

00080 INT 20H  DOS Terminate Program

00084 INT 21H  Microsoft DOS Function Calls

00088 INT 22H  DOS Terminate Address (not a callable function)

0008C INT 23H  DOS Ctrl-break Exit Address (not a callable function)

00090 INT 24H  DOS Fatal Error Exit Address (not a callable function)

00094 INT 25H  DOS Absolute Disk Read

00098 INT 26H  DOS Absolute Disk Write

0009C INT 27H  Terminate and Stay Resident (control passes to COMMAND.COM)

000A0 INT 28H  Idle Loop, spooler waiting; issued by DOS when waiting

000A4 INT 29H  CON Device Raw Output Handler

000A8 INT 2AH  3.x Network Communications

000AC INT 2BH-2DH  Reserved for DOS

000B8 INT 2EH  Execute DOS Command  (undocumented)

000BC INT 2FH  Print Spool Control(Multiplex Interrupt)(MS-DOS instalble Srvcs)

000C0 INT 30H-31H Internal Use

000C8 INT 32H  Reserved for DOS

000CC INT 33H  Microsoft Mouse Driver Calls

000D0 INT 34H-3EH  Reserved for DOS

000FC INT 3FH  Used by LINK to manage overlay segments

00100 INT 40H  Fixed disk/Floppy Disk Handler

00104 INT 41H  ROM pointer; Fixed Disk Parameters

00108 INT 42H  EGA: Org. Video Vector Screen BIOS Entry

0010C INT 43H  EGA: Initialization Parameters

00100 INT 44H  EGA: Graphics Character Patterns

00114 INT 45H  Reserved

00118 INT 46H  AT: Pointer to Second Fixed Disk Parameters

0011C INT 47H  Reserved

00120 INT 48H  PCjr Cordless Keyboard Xlat Routine

00124 INT 49H  PCjr Non-Keyboard Scan Code Xlat Table

00128 INT 4AH  AT, PS/2 User Alarm Routine

0012C INT 4BH-4FH  Reserved

00140 INT 50H  Periodic Alarm Interrupt from Timer

00144 INT 51H-59H  Reserved

00168 INT 5AH  Cluster Adapter BIOS-entry Address

0016C INT 5BH  Cluster Boot

00170 INT 5CH  NETBIOS Entry Point

00174 INT 5DH-5FH Reserved

00180 INT 60H-66H  Reserved for user program interrupts

0019C INT 67H  EMM: Expanded Memory Manager Routines

001A0 INT 68H-6BH  Unused

001B0 INT 6CH  System Resume Vector

001B4 INT 6DH-6FH  Unused

001C0 INT 70H   Real Time Clock                                IRQ08 \

001C4 INT 71H   LAN Adapter (and IRQ2 hardware replacement)    IRQ09  AT

001C8 INT 72H   Reserved                                       IRQ10 Slave

001CC INT 73H   Reserved                                       IRQ11  Int.

001D0 INT 74H   Mouse Interrupt                                IRQ12 Contr.

001D4 INT 75H   80287 NMI Error BIOS Redirected to INT 2       IRQ13  |

001D8 INT 76H   Fixed disk controller                          IRQ14  |

001DC INT 77H   Reserved                                       IRQ15 /

001E0 INT 78H-7FH  Unused

00200 INT 80H-85H  Reserved for BASIC

00218 INT 86H  AT: NetBIOS relocated INT 18H

0021C INT 87H-F0H  Reserved for BASIC Interpreter

003C4 INT F1H-FFH  Reserved for User Program Interrupts

003FF-00300  Used for power-on and initial boot stack

00400-004FF  BIOS Data Area: (2 byte words)

00400   4 words COM1: to COM4: port addresses

00408   3 words LPT1: to LPT3: port addresses

0040E   1 word  LPT4: address except PS/2 Reserved

00410   1 word  Equipment flag

bits: 15-14 number of LPTs attached

13 internal modem (CVT) or reserved

12 joystick

11- 9 number of COMs

8 unused (jr: DMS chip present)

7- 6 number of disk drives

5 1=80x25 0=40x25 screen

4 1=color 0=monochrome

3- 2 00=64K chips; 11=256K chips (PC,XT,AT)

reserved, pointing device   (PS/2)

1 math coprocessor installed

0 IPL disk installed

00412   1 byte  Init Flag; Reserved (CVT self-test status)

00413   1 word  Memory size in K bytes

00415   1 byte  Reserved

00416   1 byte  Reserved

00417   2 bytes Keyboard monitor flag bytes 0 and 1:

bit  7 ins lock          7 ins pressed

6 caps lock         6 caps pressed

5 num lock          5 num lock pressed

4 scroll lock       4 scroll pressed

3 alt pressed       3 pause locked

2 crtl pressed      2 sysreq pressed

1 <shift pressed    1 <alt pressed

0 >shift pressed    0 >alt pressed

00419   1 byte  Alternate keypad entry

0041A   1 word  Keyboard buffer head pointer

0041C   1 word  Keyboard buffer tail pointer

0041E  32 byte  Keyboard buffer

0043E   1 byte  Drive recalibration status

bit  7 Interrupt flag

6-4 reserved

3 Recalibrate drive 3

2 Recalibrate drive 2

1 Recalibrate drive 1

0 Recalibrate drive 0

0043F   1 byte  Motor Status

bit  7 Currently reading or writing

6 reserved

5-4 00 drive 0 selected

01 drive 1 selected

10 drive 2 selected

11 drive 3 selected

3-0 Drive 3-0 Motor On status

00440   1 byte  Motor Control time-out counter

00441   1 byte  Diskette Status Return Code

00H - No error

01H - Invalid diskette drive parameter

02H - Address Mark not found

03H - Write-protect error

04H - Requested Sector not found

05H - reserved

06H - Diskette Change Line active

07H - reserved

08H - DMA overrun on operation

09H - Attempt to DMA Across a 64K boundary

0AH - reserved

0BH - reserved

0CH - Media Type not found

0DH - reserved

0EH - reserved

0FH - reserved

10H - CRC error on diskette read

20H - General Controller failure

40H - Seek operation failed

80H - Diskette drive not ready

00442   7 bytes Diskette Drive Controller Status Bytes (NEC)

00449   1 byte  CRT_MODE

bit 7 text 80x25 mono on mono card

6 graphics 640x200 mono on color card

5 graphics 320x200 mono on color card

4 graphics 320x200 on color card

3 text 80x25 color

2 text 80x25 mono on color

1 text 40x25 color

0 text 40x25 mono on color card

0044A   1 word  CRT_COLS Number of columns (80)

0044C   1 word  CRT_LEN  Length of Regen Buffer in bytes

0044E   1 word  CRT_START Starting Address in Regen Buffer

00450   8 words Cursor Position on each of eight pages

00460   1 word  CURSOR_MODE top-bottom line of cursor (Cursor Type)

00462   1 byte  ACTIVE_PAGE index

00463   1 word  ADDR_6845 Base Address for 6845 Display Chip

3B4H for monochrome; 3D4H for color

00465   1 byte  CRT_MODE_SETting for 3x8 Register (3B8H MDA; 3D8H CGA)

00466   1 byte  CRT_PALLETTE Setting Register (3D9H) on Color Card

00467   1 dword Temporary storage for SS:SP during shutdown

(PS/2: Pointer to reset code with memory preserved.)

0046B   1 byte  Flag to indicate interrupt

0046C   2 words Timer counter (Timer Low, Timer High words)

00470   1 byte  Timer overflow (24 hour roll over flag byte)

00471   1 byte  Break key state (bit 7=1 if break key pressed)

00472   1 word  Reset flag word:

1234 bypass mem. test    4321 preserve memory

5678 sys suspend         9ABC manufact. test

ABCD sys POST loop (CVT)

00474   1 byte  Hard Disk status or Reserved for ESDI Adapter/A

00H - No error

01H - Invalid Function Request

02H - Address Mark not found

03H - Write Protect error

04H - Requested Sector not found

05H - Reset Failed

06H - Reserved

07H - Drive Parameter Activity Failed

08H - DMA overrun on operation

09H - Data boundary error

0AH - Bad Sector Flag detected

0BH - Bad Track detected

0CH - Reserved

0DH - Invalid number of sectors on format

0EH - Control Data Address Mark detected

0FH - DMA Arbitration Level out of range

10H - Uncorrectable ECC or CRC error

20H - General Controller failure

40H - Seek operation failed

80H - Time out

AAH - Drive not ready

BBH - Undefined error occurred

CCH - Write fault on selected drive

E0H - Status error/Error Register 0

FFH - Sense operation failed

00475   1 byte  Number of hard disk drives

00476   1 byte  Fixed Disk Drive Control byte (PC XT)

00477   1 byte  Fixed Disk Drive Controller Port (PC XT)

00478   4 bytes LPT1: to LPT4: time-out byte values (PS/2 has no LPT4:)

0047C   4 bytes COM1: to COM4: timeout byte values

00480   1 word  Keyboard buffer start pointer (word)

00482   1 word  Keyboard buffer end pointer (word)

00484   1 byte  ROWS Video Character Rows - 1

00485   1 word  POINTS Height of character matrix-bytes per character

00487   1 byte  INFO byte:

bit   7  Video mode number (of INT 10H funct.0)

6- 5  Size of video RAM 00 - 64K 10 -192K

01 -128K 11 -256K

4  reserved

3  1 - video subsystem is inactive

2  reserved

1  1 - video subsystem on monochrome

0  1 - alphanumeric cursor emulation enabled

00488   1 byte  INFO_3 byte:

bit 7   Input FEAT1 (bit 6 of ISR0 (Input Status Reg.)

after output on FCR1 (featue control reg.)

6   Input FEAT1 (bit 5 of ISR0)

after output on FCR1

5   Input FEAT0 (bit 6 of ISR0)

after output on FCR0

4   Input FEAT0 (bit 5 of ISR0)

after output on FCR0

3   EGA Config. switch 4 (1=off)

2   EGA Config. switch 3

1   EGA Config. switch 2

0   EGA Config. switch 1

00489   1 byte  Flags

bit 7 bit 4 Alphanumeric scan lines:

0 0  350 line mode

0 1  400

1 0  200

1 1  reserved

6  1 - display switching enabled

5  reserved

3  1 - default palett loading is disabled

2  1 - using monochrome monitor

1  1 - gray scale is enabled

0  1 - VGA is active

0048A   1 byte  DCC Display Combination Code table index (VGA)

0048B   1 byte  Media Control

bit 7-6 Last diskettee drive data rate selected

00 - 500Kb per second

01 - 300Kb per second

10 - 250Kb per second

11 - reserved

5-4 Last diskette drive step rate selected

3-0 reserved

0048C   1 byte  Hard Disk Status Register

0048D   1 byte  Hard Disk Error Register

0048E   1 byte  Hard Disk Interrupt Control Flag

0048F   1 byte  Combination Hard Disk/Floppy Card (bit 0=1)

00490   1 byte  Drive 0 media state byte

00491   1 byte  Drive 1 media state byte

00492   1 byte  Drive 2 media state byte

00493   1 byte  Drive 3 media state byte

bit 7-6 Diskettee drive date rate

00 - 500Kb per second

01 - 300Kb per second

10 - 250Kb per second

11 - reserved

5 Double stepping required

4 Media established

3 Reserved

2-0 Drive/Media State

000 - 360Kb diskette/360Kb drive not established

001 - 360Kb diskette/1.2Mb drive not established

010 - 1.2Mb diskette/1.2Mb drive not established

011 - 360Kb diskette/360Kb drive established

100 - 360Kb diskette/1.2Mb drive established

101 - 1.2Mb diskette/1.2Mb drive established

110 - Reserved

111 - None of the above

00494   1 byte  Drive 0 track (cylinder) currently seeked

00495   1 byte  Drive 1 track (cylinder) currently seeked

00496   1 byte  Keyboard mode state and type flags

bit  7  Read ID in progress

6  Last character was first ID character

5  Force Num Lock if read ID and KBX

4  101/102 keyboard installed

3  Right Alt key pressed

2  Right Ctrl key pressed

1  Last code was E0 Hidden code

0  Last code was E1 Hidden code

00497   1 byte  Keyboard LED flags

bit  7  Keyboard transmit error flag

6  Mode indicator update

5  Resend receive flag

4  Acknowledgment received

3  = 0 reserved

2-0  Keyboard LED state bits

00498   1 word  Offset address to user wait complete flag

0049A   1 word  Segment to user wait complete flag

0049C   1 word  User wait count, microseconds low word

0049E   1 word  User wait count, microseconds high word

004A0   1 byte  Wait active flag

bit   7 wait-time elapse and post flag

6-1 reserved

0 INT 15, AH=86 wait has occurred

004A1   1 byte  LANA DMA channel flags

004A2   1 byte  LANA 0 status

004A3   1 byte  LANA 1 status

004A4   1 dword Saved hardfile interrupt vector

004A8   1 dword BIOS Video Save Table and overrides

Save Pointer Table:

dword 1 Address of Video Parameter table

Video Parameter table

1 byte  Value for CRT_COLS

1 byte  Value for ROWS

1 byte  Value for POINTS

1 word  Value for CRT_LEN

4 bytes Values for Sequencer regisers 1-4

1 byte  Value for Miscellaneous Output Register

25 bytes Values for CRTC registers 0-18H

20 bytes Values for Attribute Controller Regs. 0-13H

9 bytes Values for Graphics Controller Regs. 0-8

dword 2 Address of Dynamic Parameter Save Area (EGA, VGA)

Parameter Save Area

16 bytes Current contents of Graphics Controller Pallette Regs.

1 byte  Current contents of Graphics Controller Overscan Reg.

239 bytes reserved

dword 3 Address of Alphanumeric Character Set Override

Alphanumeric Character Set Override

1 byte  Length in bytes of each char. definition

1 byte  Character generator RAM bank

1 word  Number of characters defined

1 word  First character code in table

1 dword Address of character definition (font) table

1 byte  Number of displayed character rows

n byte array Applicable video mode values

1 byte  0FFH end of list of modes

dword 4 Address of Graphics Character Set Override

Graphics Character Set Override

1 byte  Number of displayed character rows

1 word  Length in bytes of each character definition

1 dword Address of character definition (font) table

n byte array Applicable video mode values

1 byte 0FFH end of list of video modes

dword 5 Address of Secondary Save Pointer Table (VGA)

dword 6 = 0:0 reserved

dword 7 = 0:0 reserved

Secondary Save Pointer Table

word  1 Length of Secondary Save Pointer table in bytes

dword 2 Address of Display Combination Code table

Display Combination Code Table

1 byte  Numberof entries in table

1 byte  DCC table version number

1 byte  Maximum display type code

1 byte  reserved

word array Valid display combinations

0,0 entry  0 No displays

0,1 entry  1 MDPA

0,2 entry  2 CGA

2,1 entry  3 MDPA + CGA

0,4 entry  4 EGA

4,1 entry  5 EGA + MDPA

0,5 entry  6 MEGA

2,5 entry  7 MEGA + CGA

0,6 entry  8 PGC

1,6 entry  9 PGC + MDPA

5,6 entry 10 PGC + MEGA

0,8 entry 11 CVGA

1,8 entry 12 CVGA + MDPA

0,7 entry 13 MVGA

2,7 entry 14 MVGA + CGA

2,6 entry 15 MVGA + PGC

dword 3 Address of second Alphanumeric Character Set Override

Second Alpha Mode Auxillary Character Generator Table

1 byte  Bytes/Character

1 byte  Block to load

1 byte  Reserved

1 dword Pointer to a font table

n byte  Mode values for this font

1 byte  0FFH end of mode values

dword 4 Address of User Palette Profile table

Palette Profile Table (VGA only)

1 byte  Underlining 1 - enable in all alphanumeric modes

0 - enable in monochrome alpha modes

-1 - disable in all alpha modes

1 byte  reserved

1 word  reserved

1 word  Number of Attribute Controller registers in table

(Internal palette count 0-17, 17 normal)

1 word  First Attribute Controller reg. number

(Internal palette index 0-16, 0 normal)

1 dword Address of Attribute Controller reg. table

(Pointer to internal palette)

1 word  Number of video DAC Color registers in table

(External palette count 0-256, 256 normal)

1 word  First video DAC Color register number

(External Palette index 0-255, 0 normal)

1 dword Address of video DAC Color register table

(Pointer to external palette)

n byte array Applicable video mode values for this font

1 byte 0FFH end of list of video modes

dword 5 reserved

dword 6 reserved

dword 7 reserved

004AC   8 bytes Reserved

004B4   1 byte  Keyboard NMI control flags (CVT)

004B5   1 dword Keyboard break pending flags (CVT)

004B9   1 byte  Port 60 single byte queue (CVT)

004BA   1 byte  Scan code of last key (CVT)

004BB   1 byte  Pointer to NMI buffer head (CVT)

004BC   1 byte  Pointer to NMI buffer tail (CVT)

004BD  16 bytes NMI scan code buffer (CVT)

004CE   1 word  Day counter (CVT and after)

004D0  32 bytes Reserved

004F0  16 bytes Application program communication area

00500-005FF DOS Data Area:

00500   1 byte  Print screen status flag

1 = printer active, 0FFH = printer fault

00501   3 bytes Reserved for BASIC and POST work area

00504   1 byte  Single-drive mode status byte: 0=drive A, 1=drive B

00505  10 bytes Reserved POST work area

00510  18 bytes Reserved for BASIC

0050F   1 bytes BASIC Shell Flag =2 if current shell

00510   1 word  BASIC segment address storage set with DEF SEG

00512   1 dword BASIC int 1Ch clock interrupt vector

00516   1 dword BASIC int 23h ctrl-break interrupt vector

0051A   1 dword BASIC int 24h disk error interrupt vector

0051B   5 bytes BASIC dynamic storage

00520   2 bytes DOS dynamic storage

00522  14 bytes Used by DOS for diskette initialization (see INT 1EH)

00530   4 bytes Used by MODE command

00534 204 bytes Reserved for DOS data

00600 256 bytes Reserved for DOS

00700 -         I/O drivers from xIO.SYS (Disk data, Console handler,

Serial Ports drivers, Parallel Port drivers, Null driver)

00847         xIO.SYS IRET for interrupts 1, 3, and 0FH during POST

MS-DOS  kernel from xDOS.SYS: Interrupt handlers and routines

MS-DOS  disk buffer cache, FCBs and installable device drivers

MCB (Memory control block, 16 bytes, paragraph aligned)

1 byte  4Dh Signature byte of intermediate MCB

1 word  Segment of owner program (0 if transient program)

1 word  Paragraphs in block

11 bytes reserved

Resident portion of COMMAND (Int 22, 23, 24 handlers)

(Transient portion, batch processor loaded into high memory)

MCB of original environment

Original Environment (COMSPEC=xxxx, PATH=yyyy, etc.)

MCB for first resident program

PSP (Program Segment Prefix) 256 bytes: Hex offsets:

0  CDh,20h (Int 20h terminate program handler)

1  Segment for end of allocation block

4  Reserved

5  Long call to DOS function dispatcher

0A  Previous Int 22h vector

0E  Previous Int 23h vector

12  Previous Int 24h vector

14  Reserved

2C  Segment of local environment block

2E  Reserved

5C  First File Control Block

6C  Second File Control Block

80  Command tail and default Disk Transfer Area

First resident program

MCB for Environment copy #1

Environment copy #1

MCB for second resident program

PSP for second resident program

Second resident program

MCB for Environment copy #2

Environment copy #2

MCB for current transient program, Last MCB:

16 bytes: 5AH, seg. of prog., paragraph size to xxxxxx.

4Dh and 5Ah are ASCII 'MZ', the initials of Mark Zbikowski,

who, along with Aaron Reynolds, were the developers of DOS.

'MZ' is also used as the first two bytes in an EXE file.

PSP for transient program

Start of Transient Program

xxxxx-yyyyy   User's transient program, memory allocated to zzzzz

zzzzz-end     Possible user resident program loaded into high memory

(end is A0000 for MS-DOS 2.x, 9FC00 for 3.x)

Extended BIOS Data Area: POST in PS/2 allocates memory for

data in high memory below 640K (A0000H) in (n) Kbytes.  The

word value at 00413H (memory size) is decremented by (n) by

this allocation.  The first byte in the Extended BIOS

Data Area is initialized to (n).  To access this segment,

issue Int 15h, AH=C1h (return Extended BIOS Data Area

Segment Address).  To determine if an Extended BIOS

Data Area is allocated, use Int 15h, AH=C0h (Return

System Configuration Parameters).

A0000-AFFFF   Start of EGA and VGA graphics display RAM Modes 0Dh and above

B0000-B3FFF   Start of MDPA and Hercules graphics display RAM

B4000-B7FFF   Reserved for Graphics display RAM

B8000-BBFFF   Start of CGA Color graphics display RAM

BC000-BFFFF   Reserved for Graphics display RAM

C0000-C3FFF   EGA BIOS ROM

C4000-C5FFF   Video adapter ROM space

C6000-C63FF   256 bytes of PGA communication area

C6400-C7FFF   Last 7Kb of video adapter ROM space

C8000-CBFFF   16K of hard disk BIOS adapter ROM space \

CC000-CFFFF                                           |

D0000-D7FFF   32K cluster adapter BIOS ROM            |Expansion Cards

D8000-DBFFF                                           |

DC000-DFFFF   Last 16Kb of adapter ROM space          /

E0000-EFFFF   64K expansion ROM space (AT, PS/2)

F0000-F3FFF   System Monitor ROM

F4000-F7FFF   System Expansion ROMs

F8000-FBFFF     "

FC000-FEFFF   BIOS ROM; BASIC and simple BIOS

FF000-FFFEF   System ROM

FFFF0-FFFF3   Hardware boot far jump vector

At reset or power up, program execution begins at

this address (this is a feature of the  80x86, not

DOS or the BIOS).  The standard ROM reads the boot

sector from disk in A: or hard disk if A: not read.

The boot record resides on track 0, sector 1, side 0

of every formated diskette, or on the first sector

of the DOS partition on hard disks.

The boot code is loaded at 0:7C00H.  (This is 1K less

than 32K, half of the original IBM PC 64K.) Control

is transferred to the boot code.  A hard disk boot code

checks for a disk partition table.   If an active disk

partition is found, the DOS boot code is loaded.  The

MS-DOS boot loader searches for xxxBIO.SYS as the

first file in the root directory.  xxxDOS.SYS

must be the second file in the root.  If they are

found, then xxxBIO.SYS is loaded, which in turn,

loads xxxDOS.SYS.

xxxBIO.SYS is the interface module to the ROM BIOS,

which performs the low-level interface to devices.

When initialization is completed, DOS code is relocated

downward and entered at its first byte.

xxxDOS.SYS initializes its interrupt handlers and

then uses the EXEC function to run COMMAND.COM.

Early in the POST (power on self test) the absolute

addresses 0C0000H through 0C7FFFH are scanned in 2Kb

blocks in search of valid video adapter ROM.  If found,

POST executes a far call to byte 3 of the ROM to allow

power-on video initialization.

Similarly, addresses 0C8000H through 0DFFFFH are

scanned in 2Kb blocks for valid adapter ROM modules.

Valid adapter ROM have 55H,AAH as the first two bytes.

The third byte gives the length of 512b blocks (up

to 7FH) in the ROM.  The ROM is checked by summing

each byte modulo 100H.  The sum must be 0 for a valid

ROM.

FFFF5-FFFFC   ROM date in ASCII form. E.g.: '09/02/86'

FFFFD         Reserved

FFFFE-FFFFF   IBM Model Bytes:

PC   PC   PC   PC   PC    PC   PC  PS/2 PS/2 PS/2 PS/2

XT   jr   AT  AT3x9 XT/2  CVT  30  50   60   80

------------------------------------------------------

Model bytes FF   FE   FD   FC   FC    FB   F9   FA  FC   FC   F8

Submodels         0       0-1  0-1     0    0    0   4    5  0-1

Revisions       1-2            0-1   1-2    0    0   0    0    0

COMPACT PC  XT  DP  HP110

------------------  -----

Model bytes         2D  9A  FE     B6

100000-15FFFF 384K RAM on AT, PS/2 system board

15FFFF-FDFFFF Extended RAM for the 80286.  Additional memory in 8086

machines use bank switching of a segment of memory

within the 1 Meg address range of the 8086.  This is

called expanded memory.  The standard interface to

expanded memory is through the EMM (Expanded Memory

Manager) at Int 67h.  The 80286 can directly address

16 Megabytes of memory.

FE0000-FEFFFF Reserved

FF0000-FFFFFF 64K ROM BIOS on AT, PS/2

01000000-FFFFFFFF Extended RAM for the 80386. The 80386 can address up

to 4 Gigabytes of physical memory.