HP DeskScan II v2.9 Information

This document contains information about HP DeskScan II that has been updated after the regular documentation was printed.

Topics discussed in this document include:

* Upgrading from an earlier version of DeskScan

* The DESKSCAN.INI file

* The HPDS23.INI file

* Scanning for ColorSmart printer drivers

* Scaling images scanned with a transparency adapter

* Making sure that TWAIN file versions are correct

* How to install DeskScan II with Norton Desktop

* DeskScan II and OS/2

* DeskScan II and OLE

* Removing DeskScan II from your computer

Upgrading from an earlier version of DeskScan

The Setup program for DeskScan II automatically upgrades earlier versions of DeskScan software.

Version 2.9 includes an HP ScanJet driver for ASPI- and CAM- compatible SCSI cards.    This driver is for use ONLY with the HP ScanJet 6100C, 5p, 4c, 4p, 3c, IIcx, IIc, 3p or IIp scanners.    This driver will not function correctly with the original ScanJet (HP9190A) or ScanJet Plus scanners.

If you already have an ASPI or CAM SCSI card, the new driver provides the advantages of using one less interface card slot and using less expansion ROM memory space.

When you upgrade, the existing DESKSCAN.INI file is saved as DESKSCAN.BAK.    If you have calibrated print paths that you want to save from the previous version of DeskScan, you can recover them from the DESKSCAN.BAK file. (See the "DESKSCAN.INI file" section below for more information on recovering old calibration files.)

The DESKSCAN.INI file

This is the DeskScan II "initialization" file.    It stores DeskScan's default settings and any custom settings you specify to DeskScan as you run the program.

This file MUST reside in the DESKSCAN directory (the directory where you installed DeskScan II).

This file is not intended for modification by any program other than DeskScan.    However, if you have created custom calibrations for your printers or applications in an earlier version of DeskScan, you can copy them to the new DESKSCAN.INI file.

To do this, open the old version (DESKSCAN.BAK) and the new version (DESKSCAN.INI) as ASCII files in a text editor.    Copy all the lines under the [Calibrations] section in the .BAK file under the [Calibrations] section in the .INI file.    Don't leave any blank lines after the [Calibrations] line.

If you installed the new version of DeskScan II over the old version, all the calibration files will be available.    If you have installed DeskScan II into a new directory, you must copy all the file names called out under the [Calibrations] section from the old directory to the new.

The HPDS23.INI file

This file resides in the WINDOWS directory and is used so that the HP DeskScan II TWAIN data source can find the main DeskScan II program file.    It maintains the directory name where you installed DeskScan II.

If this file becomes corrupted, DeskScan II displays an error message. To restore the file, copy the original version from the DESKSCAN directory to the WINDOWS directory.

If you rename the DESKSCAN directory (the directory where you installed DeskScan II), run DeskScan II by double-clicking the icon in the DeskScan II Program Manager Group BEFORE trying to run DeskScan II through TWAIN.    This will allow DeskScan II to update the path in the HPDS23.INI file.

New Scanning Default Gamma 2.2

In order to better support industry standards for gamma correction, the new industry standard for gamma correction on personal computers is 2.2, replacing the old default of 1.8. Select this calibration file from within the Printer Calibration section of the Custom Print Path dialog box; this is also the new ColorSmart default (see below). This standard (known as sRGB) provides better compatibility with default settings for computer monitors and printers.

DeskScan II still provides the ability to define a gamma of 1.8 using the Gamma 1.8 calibration (formerly Default Gamma 1.8). Use this calibration file if you want backward compatibility with all of your old images.

Scanning for ColorSmart printer drivers

DeskScan II includes a special calibration file to help you achieve better colors when printing to your color printer using a ColorSmart printer driver.    Select this calibration file from within the Printer Calibration section of the Custom Print Path dialog box.

If the colors on your print-out do not closely match the original, try to create a custom print path for your printer and select "ColorSmart" in the Printer Calibration combo box.

If your printer is a 300 dpi device, we suggest that you enter "300" in the "Drawings and Halftones" text box and "100" in the "Photos" text box.

If you have a 600 dpi printer, we suggest that you enter "600" in the "Drawings and Halftones" text box and "150" in the "Photos" text box.    Remember to name and "Add" your new print path before leaving the dialog.

For general information about how to create a custom Print Path for your printer that can be selected from the "Path" combo box on the DeskScan II front panel, please refer to chapter 4 in the DeskScan II User's Guide.

Scaling images scanned with a transparency adapter

The HP ScanJet 6100C, 4c, 3c and IIcx scanners support scanning from a transparency adapter (the /T).

Under some circumstances while scaling an image, minor defects common to all CCD scanners may become more noticeable.

If you are confronted with this problem, scan the image at 50, 100, 150, 200, 300, 400, 600, or 800 dpi.    Then, transform the image to the desired resolution by "resampling" it with an image editor such as Corel PHOTO-PAINT or Adobe Photoshop.

This will reduce or remove any CCD-related defects.

Making sure that TWAIN file versions are correct

It is important to the operation of DeskScan II that old versions of the DeskScan TWAIN data source files do not attempt to launch DeskScan II.

The best way to do this is to delete old TWAIN data source files from your system, or move them to a safe place as specified below.

*	To move the old files to a safe place, create a directory in the WINDOWS directory called

TWAINOLD.    Copy any file named HPDSxx.DS from the WINDOWS\TWAIN directory into

WINDOWS\TWAINOLD.    Then delete the HPDSxx.DS file from the WINDOWS\TWAIN

directory.

*	To delete old versions, look into the WINDOWS\TWAIN directory.    Delete any file named

HPDSxx.DS.

NOTE:    If you have an older version of DeskScan II on your system, the default TWAIN data source will be updated to use the new 32-bit DeskScan.    The older version HPDSxx.DS in the WINDOWS\TWAIN directory will be renamed to HPDSxx.OLD.

How to install DeskScan II with Norton Desktop

If you are running MS Windows using the Norton Desktop as the shell, HP recommends that you    install DeskScan II using the default directory, C:\DESKSCAN.    If you intend to install to C:\DESKSCAN, you don't need to read the rest of this section.

If you want to install to a different directory, you MUST change the shell to PROGMAN.EXE before installing DeskScan II!    (There is a compatibility issue between the DESKSCAN II setup and Norton Desktop.)

Follow these steps to change the shell:

1.    From Windows, run NOTEPAD.EXE and open the SYSTEM.INI file (which is in the \WINDOWS directory).

2.    In the [boot] section of the SYSTEM.INI file, you will find a line that reads: shell=ndw.exe.

Make a copy of this line and insert a ";" (semicolon) as the first character in one of the two lines.

3.    Change the second copy to read: shell=progman.exe

4.    Save the file, close NOTEPAD, and restart Windows.

After DeskScan II has been successfully installed, you can switch back to Norton Desktop by following these steps:

1.    From Windows, run NOTEPAD.EXE and open the SYSTEM.INI file (which is in the \WINDOWS directory).

2.    In the [boot] section, find the copy of the line with the semicolon that includes "shell=ndw.exe".    Remove the semicolon.

3.    Find the copy of the line you made that includes "shell=progman.exe". Delete it.

4.    Save the file, close NOTEPAD, and restart Windows.

DeskScan II and OS/2

DeskScan II is not currently supported under OS/2.    However, call HP's Automated Support Access Program at 1-800-333-1917 (or 1-208-344-4809) for information allowing use with OS/2.

You can also check the HP Peripherals forum on CompuServe or the Internet to find the latest information.

At any CompuServe prompt type,

"Go HPPER"

To connect via the Internet, use either the alias,

ftp-Boi.external.hp.com

or the IP address, 192.6.71.2.    Login as 'anonymous'.    Use your Internet alias as the password (e.g. 'yahoo@lab.hp.com').

DesksScan II and OLE

Memory Usage    The use of OLE requires a lot of memory, which may cause problems if you don't have enough RAM in your system.    One symptom indicating a memory problem is that an application may report that it is unable to find a selected server even though the server application is correctly registered.    (Check the registry to make sure.)    What is really happening is that there isn't enough memory to load the server and the associated system DLLs and the application assumes the failure was due to registration problems.

File Size  Extra data is stored with a scanned image when it's embedded in an OLE object. The extra data is roughly the size of the image, which may be quite large. If file size is a concern, you might prefer using methods other than OLE to store your images in documents. Try using TWAIN, paste from the clipboard, files, etc.

Windows 95/Windows 98    You might encounter problems updating a DeskScan object embedded in a WordPad document. You can avoid this problem by using an alternative method to embed images in a WordPad document as follows: run DeskScan standalone and save an image as a .BMP file; save it to the Desktop. An icon representing the image will show up on the Desktop. Drag this icon into the WordPad window and drop it into the WordPad document. You can double click on the image to edit it. (Note that the image doesn't have to be saved to the Desktop. You can drag an image icon from any directory into a WordPad document.)

Window NT 3.5    The following problem may occur if you're running Client Service for NetWare and Windows NT 3.5. Some container applications may lock up if you close the container while you have an active embedded DeskScan object. To avoid this problem, close the image editor before closing the document that contains the embedded image.

Windows NT 3.51    Write may lock up while trying to activate an embedded DeskScan object.

Windows NT 4.0    If Paintbrush (not PHOTO-PAINT) is your default image editor, an embedded DeskScan object may appear to be corrupted when you create it using Insert Object.    The object is not really corrupted.    To fix the object's appearance, double-click on the object, make a slight change to it, and it will update correctly.

Corel PHOTO-PAINT    If you've installed the Corel PHOTO-PAINT5 software, you can embed Corel PHOTO-PAINT objects directly into your OLE documents by selecting a Corel PHOTO-PAINT object in the Insert Object dialog. This starts the Corel server, which asks you if you want to scan an image. If you decide to scan, you can use DeskScan to scan an image that becomes embedded as a Corel PHOTO-PAINT object.    This functionality is not supported in Corel PHOTO-PAINT6.    You must select DeskScan from the Insert Object dialog.

Removing DeskScan II from your computer

You can run UNINSTAL.EXE to remove DeskScan II from your system.    NOTE:    Be sure DeskScan II is in the same directory where you installed it.    If you have moved DeskScan to a different directory or drive, UNINSTAL.EXE will not work properly.    To manually uninstall the DeskScan II application from your computer follow the steps below.

NOTE:	If you have other HP ScanJet software installed on your system, you should skip the ScanJet Drivers section and go to the DeskScan section.

ScanJet Drivers:

a) Delete the following files in your WINDOWS\SYSTEM directory if they exist:

HPSCNMGR.DLL

HPSCNTST.DLL

HPSJ32.DLL

HPSJ16.DLL

2) Windows 95/Windows 98 only:

Delete SCANJET.INF from the WINDOWS\INF directory

Delete the following files in your WINDOWS\SYSTEM directory if they exist:

HPSJCLAS.DLL

VHPSCAND.VXD

HPSJ1695.DLL

REG32.DLL

HPSJRFSH.VXD

DeskScan II:

1.    Delete the following files from your WINDOWS directory:

HPDS23.INI

2.    Delete the following file from your WINDOWS\TWAIN_32 directory:

HPDS23.DS

3.    Delete all of the files in the directory where DeskScan II was installed.

4.    Remove the directory where DeskScan II was installed.

5.    From the Windows Program Manager, delete the Program Manager Group where DeskScan II was installed.    The DeskScan II icons will be removed automatically.

DeskScan II has been successfully removed from your computer.