���������������������������������������������������������������������ͻ

� CMOSRAM2.EXE:                                                       �

�                                                                     �

� THE INDISPENSABLE GUIDE TO THE CARE AND FEEDING OF THE AT'S         �

� INTERNAL CMOS CONFIGURATION RAM AND ITS BACKUP BATTERY POWER SOURCE �

�                                                                     �

� BY THOMAS MOSTELLER  COPYRIGHT 1995                                 �

���������������������������������������������������������������������ͼ

I. INTRODUCTION

Inside (almost) each and every one of the 47.2 billion IBM AT

clones sold daily there lives an elegant solution to the problem of

configuring a flexible yet easy to use computer system: the internal

CMOS configuration RAM.

When IBM designed the AT they listened to their customer's

grumblings enough to know that one of the major stumbling blocks for

the average PC and XT user was figuring out how to set the motherboard

DIP switches.  Business users also complained about not having a built

in real time clock.  (I guess the sales racked up by companies like AST

on their multifunction boards with clocks didn't hurt any either!)  So

now all AT's also have battery backed up clocks.  IBM solved both

problems with one IC - a clock calendar chip made by Motorola

(MC146818) which includes 50 bytes of user addressable non volatile

RAM.  This same chip is also used by many clone makers, but other

configurations which perform identically are possible.

So now, instead of flipping DIP switches to configure your

computer, the AT uses software to store the machine's configuration.

This software takes the form of a setup program provided by the BIOS

manufacturer, either resident in the BIOS ROM or in a disk based

utility called something like SETUP (or CONFIG, etc.).  True Blue IBM

users will find it on their Advanced Diagnostics Diskette.  The idea

is that with this program, you can quickly install or remove

equipment from the computer without having to search for the

documentation.  In reality, the success of this approach depends on

the quality of the software.  Most of the programs I've seen are

pretty bare bones, but passable.

And what kind of information is stored in this RAM?  Quoting from

IBM's Personal Computer AT Technical Reference Manual - an excellent

way for those who feel their bank account is $125 too heavy to correct

the situation - the following information is stored in the CMOS RAM:

1. Clock information

2. Diagnostics status byte

3. Shutdown status byte

3. Diskette drive type byte - drives A and B

4. Fixed disk drive byte - drives C and D

5. Equipment byte (# of diskette drives, display adapter and math

coprocessor)

6. Low and high base memory bytes

7. Low and high expansion memory bytes

8. A checksum byte to confirm the validity of the stored data

In addition, there are several bytes of memory whose use IBM does

not specify, but instead marks them as reserved.  Perhaps they are

there to allow DOS to smash the 640K barrier with an additional 3 bytes

of slow memory.

II. THE SNAG

Well, this all sounds very nice and possibly even remotely

interesting but probably fits in the same category as general

relativity theory: Interesting But Of Little Use In Everyday Life (move

over, Albert).  Ah, but here's the rub: much like the IRS, it can cause

you grief without warning.

Think about it: what's been the problem with every battery

operated toy you've ever had?  That's right, (unless you broke it

first) eventually the batteries died!  And there's little more useless

than a toy with dead batteries - including your AT.

How long can you expect the battery life to be?  It's difficult to

say; many different battery types are in use.  However, two kinds

dominate the marketplace:  the lithium cell and the alkaline battery

pack.  (A few manufacturers use a rechargeable NICAD pack and claim

infinite life.  If you buy one of these, write to the manufacturer and

demand complete test results.)  The batteries are being used at a very

low current drain, so their lifetime is essentially equal to what the

manufacturer calls the shelf life.  This number varies widely, but 2 to

3 years is probably a typical number.  Your mileage may vary.

What would the symptoms of a dead battery be?  Again, it varies

among BIOS manufacturers, but most of them do a check of the CMOS RAM

at power up.  If the check bounces, the boot process will generally

halt with an error message.  BIOS's with disk based setup programs will

then assume some kind of default state, usually a lowest common

denominator type of setup with 512K of RAM and 1 floppy drive; those

with internal setup programs often just halt.  Now you have to

recognize that this machine - which worked fine yesterday but is now

mysteriously broken - only needs a new battery to be back on its feet.

Otherwise, you're in for an expensive service call and some downtime.

Oh yes, it does need one other thing - it needs to be

reconfigured!  I hope you remember how many cylinders and heads your

hard disk has...  Well, it should be in the documentation - if only

you could find it.  And the diskette with the SETUP program?!  Or

maybe you don't really know that much about computers so the dealer

you bought it from configured it for you and maybe THAT GUY WENT OUT

OF BUSINESS 2 YEARS AGO 'cause you bootlegged Lotus and WordPerfect

from your brother in law instead of buying it from the dealer and

maybe YOU DON'T EVEN KNOW WHAT KIND OF DISK DRIVES ARE IN THE

MACHINE and maybe YOU DON'T KNOW HOW TO SETUP YOUR MACHINE because

you've NEVER DONE IT BEFORE...  Well, no time like the present to

learn!  Will there be snags the first time you try it?  Of course

there will be! Will there be a crucial job whose deadline comes and

goes while you pore over reams of documentation as the rest of the

free world sleeps soundly?  Of course there will be!

A gloomy picture isn't it?  But don't worry, be happy, now there's

III.  HOPE FOR THE DAMNED: CMOSRAM2.EXE

CMOSRAM2.EXE is a program whose sole purpose in life is to

minimize the pain of silicon senility, premature or otherwise.  It's

the updated version of the CMOS_RAM program mentioned in prominent

magazines, and which has saved the bacon of many a user.  CMOS_RAM

was not capable of handling most 386, 486 and above type systems.

Now, CMOSRAM2 is available to protect almost any system on the

market.

How does it do that?  Simple!  By storing the contents of your

CMOS RAM in a file while the computer is up and running, it can be

quickly and easily restored when the unthinkable - but inevitable -

occurs.  Here's how you do it:

Make up what I call a "disaster" floppy.  This is a bootable

floppy disk (consult your DOS manual or the little kid next door if

you don't know how to do that) with no - or minimal - AUTOEXEC.BAT

and CONFIG.SYS files.  Leave out most of the memory resident stuff -

they might confuse the issue.  Users of STACKER or DOUBLESPACE might

have to include them in the CONFIG.SYS to avoid an error message.

However, most error messages can be ignored, you just need to get to

the DOS prompt to run CMOSRAM2.  A DOS system disk is usually

created by typing FORMAT A: /S for users of DOS 5 or greater.  Try

booting from this floppy to ensure that it works.

Oh, and one more thing should be included on your disaster disk

(flourish of trumpets): CMOSRAM2.EXE!  Now, to set up your disaster

disk for use: with the disaster floppy in the diskette drive, simply

type CMOSRAM2, press return, and after a short but delightful

introductory screen the program will present you with 5 choices:

1: About the CMOS RAM

2: Save the contents of your CMOS RAM to a disk file

3: Restore the CMOS RAM from the disk file

4: Set the CMOS clock

5: Exit

Pressing 1 brings up a series of help screens, which basically

repeat the text in this document in a shorter and less witty fashion.

Perhaps you will prefer this.

Option 2 is the one you'll use first to save the CMOS RAM's

contents.  As long as there is around 500 bytes of available space on

the disk, a file called CMOS.RAM will be written and checked for

validity.

The third option is the one employed to bring your system back

from the dead.  As long as the program can find the CMOS.RAM data file

in the current subdirectory, RAM restoration will take place very

quickly (less than one second).  Everything should be copasetic now,

except that the time will be set to whatever time it was when you saved

the RAM.  No problem, that's what option 4 is for.

Option 4 allows the setting of the clock to the current date and

time.  This option does not affect the current DOS time, so the change

will not take effect until you reboot.  You can use this option

whenever you have to change the date or time, such as going on and off

daylight savings time.  Curiously, although the MC146818 has a daylight

savings time function, DOS does not use it.  It also has a day of the

week function, but DOS works that out on its own.  Go figure.

One caveat: the routines I wrote for setting the time and date

will work for any BIOS that follows the IBM guidelines.  I have tested

the program on machines with Phoenix, Award, and AMI BIOS's, and it

works on all of them.  However, it would be possible to design a

hardware/software system for which this program would not work

correctly.  However, it should not damage the system in any way.

Also, versions of DOS greater than 5.0 set both the DOS system clock

and date and the hardware clock and date when you type TIME or DATE.

Earlier versions only set the DOS system clock and date, not those

in the battery backed up clock and calendar.  That's why option 4 is

included in CMOSRAM2.

Do I have to explain the fifth option?

IV.  DEALING WITH DISASTER

So it's finally happened - your AT won't boot up.  Does it give

you a message relating to an invalid configuration, or anything else

that makes you think it could be the CMOS RAM?  A typical example would

be:

Invalid configuration information - please run SETUP program

Strike the F1 key to continue

Your next move depends on your machine.  Units with ROM based

setup programs usually won't proceed any further no matter what you do.

You must press a key, or combination of keys, to enter the setup

routine and configure the system to at least recognize the A: floppy

drive.  Don't bother with the rest of the information; running CMOSRAM2

as explained in the next paragraph will take care of that as long as

the machine will boot.  Just exit the setup menus and save the new

CMOS values.  The opening setup screen should tell you how to do

this.  After saving, the machine will reboot.

Computers with disk based SETUP programs have it easier.  These

machines usually allow the A: floppy to work no matter what the

contents of the CMOS RAM are, so just whip out your disaster floppy and

run CMOSRAM2.  Select option 3, and then set the time and date with

option 4.  If no error messages result, open the drive A: door and

reboot the machine with CTRL-ALT-DEL.  You should be up and running

that fast!  You can continue to use the computer in this state just

as if the battery were working properly.  The values in the CMOS

memory will survive a CONTROL-ALT-DELETE warmboot, or even a press

of the RESET button.  However, if you power down, the above process

must be repeated.

But now you're faced with replacing the battery.  This means you

or a technically oriented friend upon whom you can impose are going to

have to open up the machine and get a replacement for the battery.  The

more conservative among us might be well advised to make a hard disk

backup before opening the machine.  If you just know how to use a

screwdriver, and how to replace the batteries in a flashlight, many

times you can do the work yourself.  Computer hardware knowledge is

usually not required.  A hint: in computers built before 1988 or so,

the batteries are either stuck to the back wall of the computer in a

holder or on the motherboard in a "coin cell" holder.

However, in some cases it still may be worthwhile to take your

computer to an authorized repair station.  An example of such a case

would be if opening the computer required a tool you've never seen

before, or if you really are just all thumbs with hardware.  Also,

newer systems usually have the battery soldered to the motherboard.

Repair of systems like this should only be done by a qualified

repair technician (ie, a person who owns a soldering iron and

screwdriver).  At least CMOSRAM2 will allow you to limp along until

you can arrange to have the machine repaired on the most

advantageous schedule for you.

A WORD OF WARNING:

Do I really have to remind you to unplug the computer before

opening the case?  Please do.  Also, never touch anything on a printed

circuit board without touching a large metal object first.  This is not

a good time to wear your knit sweaters... anything with "static cling"

is a bad idea!

If your machine uses 4 alkaline AA cells, you can just run to the

corner convenience store and pick up replacements.  Be sure to get a

reliable alkaline brand, such as Eveready, Duracell, Radio Shack, or

Kodak.  Look for a good expiration date.  If the batteries simply slip

into the holder, put a sturdy rubber band around the holder to hold

them securely in case of rough handling.

Those with lithium batteries are in for a little rougher time

locating a replacement.  Luckily, our friends at Radio Shack have a

rather nice selection of replacement batteries, including some which

look as though they should do the job for most models.  If not, the

Radio Shack catalog says they will special order any battery, although

I don't know how good they are.  As a last resort, you may have to

contact your manufacturer.

ANOTHER WORD OF WARNING:

NEVER EVER EVER carry a lithium battery loose in your pocket or in

a purse.  When shorted - such as with coins or keys - lithium batteries

are an explosion and burn hazard.  These cells pack a hefty wallop, and

while current types have an over-pressure venting system DON'T TRY IT!

Put the cell in an envelope, wrap tape around it, or figure out some

other way to insulate it.  Leave the new cell in its protective

packaging until you're ready to use it.  Also, be sure to dispose of

the old cell per the manufacturer's instructions.

Another thought to share with you:  you can't ease the replacement

process the next time by buying two sets of batteries and keeping one

aside.  Since the battery's lifetime is essentially equal to its shelf

life, when the one in use quits the other will be at the end of its

life also.  Buy a fresh one.

Once the battery is in place, go through the ritual outlined above

one last time, and then you're back in business.

V. WAIT, THERE'S MORE

Well, it seems that CMOSRAM2 has made the world safe for truth,

justice, and the American way.  By taking a few minutes to prepare

before disaster strikes, I say BEFORE DISASTER STRIKES, a potentially

expensive service call can be handled by anyone with a basic knowledge

of tools and the inside of a computer.

However, one problem with a program like CMOSRAM2 is that it's

easy to use it and forget about it.  After all, it could be several

years until you need it.  In that time, you could easily change your

hardware and forget to update CMOS.RAM, defeating the program's purpose

and increasing your hassles.

Enter CHKCMOS2.  Just install this program in your AUTOEXEC.BAT

file along with a copy of CMOS.RAM in the same directory.  This is

done as follows:

1.  Create a directory called CMOSRAM2.  (MD CMOSRAM2)

2.  Copy the files CMOSRAM2.EXE and CHKCMOS2.EXE to this directory.

3.  Run CMOSRAM2 as outlined for the disaster disk, and use option 2

to save the CMOS contents to a file.

4.  Add the following lines to your AUTOEXEC.BAT file using DOS

EDIT:

CD\CMOSRAM2

CHKCMOS2

CD\

Don't bother to add CMOSRAM2 to your PATH statement.  There's

no need to use up precious space in your path, which is limited to

127 characters.

This program helps you two ways.  First, it checks two self

test status functions of the MC146818 IC to ensure that its data is

valid and that the battery voltage is within the allowable limits.

If either of these checks fails, a warning is generated.  In theory,

this should serve notice that the battery is on its last legs in

time to replace it.  However, in the real world, batteries often

fail suddenly, so don't count on knowing in advance.

The other function of CHKCMOS2 is to make sure that the contents

of the file CMOS.RAM match what's in the CMOS RAM itself, with the

exception of the time and date.  This will remind you to update

CMOS.RAM when you make a hardware change.  Also, it will absolutely

find any CMOS RAM failures which go undetected by the IC's self test

features.  Note that CHKCMOS2 is not memory resident, and it does

not have to be "loaded high" or any other fancy trick to work.

However, keep in mind that both CMOSRAM2 and CHKCMOS2 both only look

for CMOS.RAM in the same directory as they reside.  If you have

CMOS.RAM in another directory, both CMOSRAM2 and CHKCMOS2 will fail

with an error message.

VI. THE BOTTOM LINE

Now that you've seen what CMOSRAM2 and CHKCMOS2 can do for you, we

come to the knotty problem of what its worth.  These programs are like

insurance; it's easy to miss their value in day to day life.  However,

when things go bad, you'll be mighty glad you have them!

The programs have been tested thoroughly and they really work.

When you do need them, if you've followed the instructions above and in

the help screens, they will quickly restore your system's integrity and

get you up and running again with a minimum of hassle.  Also, the less

you know about computers, the greater the value of the programs.  They

save you the time and hassle of figuring out all the parameters of the

SETUP program.

In order to bring the benefits of CMOSRAM2 and CHKCMOS2 to you at

the lowest possible cost, they are distributed under the shareware

concept.  Try them, and if you feel that the programs are useful to

you, a payment of $5.00 (cheap insurance) would be appreciated to:

Thomas Mosteller

1872 Rampart Lane

Lansdale, PA 19446-5051

Compuserve ID# 72637,173

The $5.00 fee entitles you to permanently use a copy of

CMOSRAM2.EXE you obtained from a third party - in other words, not

me.  I cannot supply unregistered copies of CMOSRAM2.EXE and

CHKCMOS2.EXE.  If that's what you want, I recommend you get the

program from any shareware vendor.  I can supply registered copies

if you send $10 to the above address.  Be sure to state whether you

want a 5 1/4 or 3 1/2 disk in your letter.  Anyone who does not

specifically state what size they want will be sent a 3 1/2 inch

disk.

In addition, although I retain all copyrights to the program,

you are free to copy it, pass it along to friends, upload it to

bulletin boards, etc., as long as you don't modify it any way.

Always distribute the files CMOSRAM2.EXE, CHKCMOS2.EXE, and

CMOSRAM2.DOC as an unmodified set.  Also, you may not charge more

than a $5.00 fee for distributing the program in disk form.

Fair enough?

VII.  AND NOW, INTRODUCING KNOWSAVE!

KNOWSAVE is the easy to use commercial version of CMOSRAM2

which won the coveted "Mondo Cool" Award in Computer Shopper

magazine.  (I'm still waiting for the T shirt though.)  KNOWSAVE has

the following features:

1. Automated installation program.  No editing of your AUTOEXEC.BAT

or any other files.

2. Printed manual with vinyl slip case you'll be proud to display in

your home.

3. Improved error checking.

4. Optional command line interface for batch file operation.

5. Easy to use interface featuring - can you stand it - color!

KNOWSAVE is available for $29.95 for a single machine.  Multiple

user discounts are available.  US shipping and handling is included

in that price.  Foreign countries add $6.00.  US currency only

please, no shells or beads accepted.  We do, however, take VISA,

MasterCard, and Discover.  Include your card number and expiration

date with your order to:

Tellerware

1872 Rampart Lane

Lansdale, PA  19446-5051

FAX your order in at (215) 368-5072!

Please state disk size, 5 1/4 or 3 1/2 inch.  If you don't tell us

what disk size you want, a 3 1/2 inch disk will be sent.