SimpTerm - V1.00

Simple Terminal Emulator

Copyright 1995, B&B Electronics Mfg Co

PO Box 1040

707 Dayton Rd

Ottawa, IL 61350

release: 3/9/95

1.0  General Information

1.1   Disclaimer of Warranty

1.2   License

1.3   Introduction

2.0  Screen Layout

2.1   Main Menu

2.2   Terminal Screen(s)

2.3   Setup Window

2.4   Handshake Line

3.0  Users Guide

3.1   Port Menu

3.11   Port | Open

3.12   Port | Close

3.13   Port | Setup

3.14   Port | Change

3.15   Port | Copy Settings

3.16   Port | Summary

3.17   Port | Exit

3.2   Control Menu

3.21  Control | RTS

3.22  Control | DTR

3.23  Control | Break

3.24  Control | Last

3.25  Control | FIFO

3.26  Control | Clear RX_Buffer

3.27  Control | Clear TX_Buffer

3.3   Options Menu

3.31  Options | Test Mode

3.32  Options | Set Trigger

3.33  Options | Monitor Mode

3.34  Options | Define String

3.35  Options | Log to File

3.36  Options | Transmit File

3.4  Display Menu

3.41  Display | Clear

3.42  Display | Split

3.43  Display | Settings

3.44  Display | Character

3.45  Display | Show/Hide

3.46  Display | 43/50 Lines

3.5  Config Menu

3.51  Config | Load

3.52  Config | Save

3.53  Config | Save As

3.54  Config | About

1.0   General Information

----------------------------------------------------------------------------

1.1  Disclaimer of Warranty

THIS SOFTWARE AND MANUAL ARE SUPPLIED "AS IS".  B&B ELECTRONICS

HEREBY DISCLAIMS ALL WARRANTIES RELATING TO THIS SOFTWARE AND ITS

DOCUMENTATION FILE, EXPRESS OR IMPLIED, INCLUDING BUT NOT LIMITED

TO DAMAGE TO HARDWARE, SOFTWARE AND/OR DATA FROM USE OF THIS

PRODUCT.  IN NO EVENT WILL B&B ELECTRONICS BE LIABLE TO YOU OR

ANY OTHER PARTY FOR ANY DAMAGES.  YOUR USE OF THIS SOFTWARE

INDICATES THAT YOU HAVE READ AND AGREE TO THESE AND OTHER TERMS

INCLUDED IN THIS DOCUMENTATION FILE.

1.2  License

SimpTerm is neither free software nor is it in the public domain.

The software and its documentation, this file, are property of

B&B Electronics and may not be sold without written permission.

SimpTerm is distributed as shareware.  This means that you are

granted a limited license to use it for a period of 30 days.  If

you find it useful and decide to continue using it after the

trial period, registration is required.

Registered users will be granted a just-like-a-book license which

means a registered version of the software can be used by more

than one person and can be moved from one computer to another so

long as there is NO POSSIBILITY of it being used by two different

persons on two different computers at the same time, just like a

book can not be read by two different persons in two different

places at the same time.

If you register for a license you are eligible for assistance

from our tech-support team and will be granted one free upgrade

of SimpTerm.  You can register SimpTerm by phone, fax, E-Mail, or

by mail.  The information is listed below:

PH: 815-434-0846     FAX: 815-434-7094

sales@bb-elec.com    P.O. BOX 1040 - OTTAWA, IL 61350

You can also register SimpTerm by Compuserve:

type: GO SWREG

registration number: 5281

1.3  Introduction

SimpTerm is a shareware software package that assists users in

solving serial communication problems.  Some of the features

included in SimpTerm are the following:

* Up to four ports can be opened at a time

* Any hardware supported baud rate

* RS-232/RS-485 support

* Handshaking control

* Loop back tests

* Triggers

* Monitor Mode which monitors communications between a DTE/DCE

(requires hardware).

2.0   Screen Layout

----------------------------------------------------------------------------

2.1  Main Menu

The Main Menu is located on the top line of the screen.  It allows

you to access all the features of SimpTerm.  It is accessed by

pressing ALT+"highlight char".  Then the right/left arrow keys are

used to move to the next/previous menu while the down/up keys are

used to move to the next/previous menu items.  Some of the menu

items have a key or key sequence to the right of the option, this

is a hot key that can be used at any time to access that menu

function.

2.2   Terminal Screen

This is the area where all the characters that are transmitted

and received are displayed.  On color monitors the transmit

characters are displayed in yellow and the receive in cyan.  On a

black and white monitor the transmit characters are displayed in

white and the receive in gray.  If an Overrun Error, Parity Error,

Framing Error, or Break Interrupt is detected the character will

be blinking (see the Handshake Line section).  When a port is

in split mode (see Display|Split), a horizontal line will be

splitting the terminal screen in which the transmit characters

(DTE side) will be displayed in the upper half, and the receive

characters (DCE side) will be displayed in the lower half.

A 45 line ring buffer is maintained for each of the ports opened.

2.3   Setup Window

This window is located on the right side of the Terminal Screen

when the Settings option is selected (see Display|Settings).  This

is used for a constant reminder of the ports settings.

2.4   Handshake Line

The Handshake Line is located on the bottom of each of the opened

ports and is split up into four sections.

The first is the "DTE:" section.  This section displays the state of

the handshakes: DTR - Data Terminal Ready, RTS - Request to send,

and BRK - Break.

The second section is the "DCE:" section.  This

section displays the state of:  DSR - Data Set Ready, CTS - Clear

to Send, DCD - Data Carrier Detect, and RI - Ring Indicator.

The third is the state of the error flags.  This displays the

state of BRK - Break, OE - Overrun Error, PE - Parity error, and

FE - Framing Error.  If one or more of these errors occur on a

receive character, the state will be set until the next receive or

transmit character is displayed.

The fourth is the current size of the Transmit "TXB:" and Receive

"RXB:" buffers.

For all of these flags, when the state is found to be "high", the

representation characters will be displayed with a gray background

with a black foreground, otherwise they will be displayed with a

black background and a gray foreground.

3.0  Users Guide

3.1  Port Menu

3.11  Port | Open

Allows the user to open a serial communications port.  An

"Address, IRQ, Transmit Buffer Size, and Receive Buffer Size,"

is required to install a port.  The address is the location of

the port, and the IRQ is the interrupt request line the port

will be operating under.  Some of the more common address:

03F8h, 02F8h, 03E8h, and 02E8h are the usual address for

COM1-COM4 respectively, although any address through 3FFh may

be specified.  All IRQ's (2-15) are also supported.

The transmit and receive buffers are used as a temporary

holding areas for characters that are waiting to be transmitted

and characters that are received before they can be placed on

the screen.  The larger the receive buffer the less chance

there is to lose characters.  The minimum size of the each of

the transmit and receive buffers is 2 characters.  The

maximum size has to be less than the following:

(2*Receive Buffer) + (Transmit Buffer)) < 65535.

When a port is opened, it transmits a NULL to determine if

the IRQ is set correctly.

3.12  Port | Close

Allows the user to close an existing port.  Any ports that

are left open are automatically resized to fill the screen.

3.13  Port | Setup

This dialog box allows you to set up the communication

parameters for the port.

3.14  Port | Change

This allows you to change a ports address, irq, receive

buffer size, or transmit buffer size without losing any of

the ports triggers, handshake settings, etc.

3.15  Port | Copy Settings

Copys the baud rate, parity, data bits, stop bits, triggers,

and all the display settings from one port to another.

3.16  Port | Summary

Shows a summary of the active port.  The address, irq,

transmit buffer size, receive buffer size, type of uart, and

FIFO trigger level if the UART is a 16550AFN.  The type of

UART's that are supported are: 8250 or 8250B, 8250A or 16450,

16550, or 16550AFN.

3.17  Port | Exit

Exits SimpTerm.  If a configuration file is open and changes

have been made since it has been open, you will be prompted

to save the changes before exiting.  If no configuration is

open it will just exit.

3.2  Control Menu

3.21  Control | RTS

Toggles the state of Request to Send.

3.22  Control | DTR

Toggles the state of Data Terminal Ready.

3.23  Control | Break

Toggles the state of the Break condition.  This condition is

caused by holding the transmit data line high.

3.24  Control | Last

Toggles continuous transmit.  The character that is repeated

could either be the last character that was typed

(transmitted) or a string that was defined and transmitted.

3.25  Control | FIFO

If the UART is a 16550AFN then the FIFO trigger level can be

set here.  It is a good idea to only change this when the

receive buffer is empty, otherwise characters could be lost.

3.26  Control | Clear RX_Buffer

If there are characters in the Receive Buffer they are

cleared.  Also this option is used to clear the receive

overflow flag which, when set is displayed on the handshake

line as "OF".

3.27  Control | Clear TX_Buffer

If there are characters in the Transmit buffer they are

cleared.

3.3  Options Menu

3.31  Options | Test Mode

Allows you to test a serial port or external device by either

looping back on itself pins 2 and 3 (transmit and receive) or

by connecting one port to another by configuring one as a DCE

device.  If two ports are being used in test mode, then the

active port transmits characters while the second port

receives.

3.32  Options | Set Trigger

A trigger is defined as a string that is looked for while a

port is receiving characters.  If this string is detected, a

second defined string is then transmitted out the port as a

reply to the trigger.

In the Set Trigger dialog box you can enable/disable each

of the triggers separately by the check box preceding each of

the triggers.  Also, there is a "Enable Port ? Triggers"

check box on the bottom of the dialog box that allows you to

enable/disable all of the triggers for that port.  This check

box does not enable triggers that are disabled by the

individual check boxs.  The "Make Global" check box allows

all the ports that have there "Enable Port ? Triggers"

enabled to use that ports triggers.  This allows the user to

set up one set of triggers for all ports open.  The "Trigger"

edit boxes contains the strings that a port is going to look

for before transmitting its reply.  This string is case

sensitive.  The "Reply" check boxes is the string that is

automatically transmitted when the "Trigger" is received.

Hexadecimal values can be specified in either the "Trigger"

or "Reply" edit boxes by placing the value between "<>".  For

example to "Reply" a Null you would include: <0> in the

"Reply" string.

3.33  Options | Monitor Mode

Allows you to monitor communications between a DTE and DCE.

This feature requires an external piece of hardware as well

as two ports being open at a time.  This piece of hardware

can be purchased from B&B Electronics (model 232MMA) for

$34.95, or can be constructed by yourself.  The cable

requires the pinouts in diagram  A.1.

DCE Device                            DTE Device

signal direction

Shield --*-----------------------------*-------------- Shield

TD --|--------------------<--------|--*----------- TD

RD --|--*----------------->--------|--|----------- RD

RTS --|--|-----------------<--------|--|--*-------- RTS

CTS --|--|--*-------------->--------|--|--|-------- CTS

DSR --|--|--|--*----------->--------|--|--|-------- DSR

GND --|--|--|--|--*-----------------|--|--|-----*-- GND

DCD --|--|--|--|--|--*----->--------|--|--|-----|-- DCD

DTR --|--|--|--|--|--|-----<--------|--|--|--*--|-- DTR

RI --|--|--|--|--|--|--*-->--------|--|--|--|--|-- RI

|  |  |  |  |  |  |           |  |  |  |  |

|  |  |  |  |  |  |           |  |  |  |  |

S  R  C  D  G  D  R           S  R  D  D  G

H  D  T  S  N  C  I           H  D  S  C  N

I     S  R  D  D              I     R  D  D

E                             E

L                             L

D                             D

COM A                      COM B

Diagram A.1

Pinouts for the Monitor Mode Adapter.

COM A and COM B connect to the SimpTerm ports that you

have opened.  COM A monitors the DCE side and COM B monitors

the DTE side.

When this mode is started, if two ports are not currently

open, it prompts you to open a second port.  Then a dialog

box comes up that looks similar to the following:

COM B (DTE)    COM A (DCE)

[X] Port 1     [ ] Port 1

[ ] Port 2     [X] Port 2

Here you tell SimpTerm which port will be monitoring the DTE

side (which is connected to COM B) and which port is going to

be monitoring the DCE side (which is connected to COM A).

The number of ports listed in the dialog box will correspond

to the number of ports you have open.  After you select

both of the ports you wish to do the monitoring, any leftover

ports will be closed automatically.

If either of the two ports selected have a 16550AFN type

UART you will be prompted to disable the triggers.

RS-422 communications can also be supported by purchasing

a 422BOC cable from B&B Electronics for $74.95.

3.34  Options | Define String

There are nine hot keys that are available to assign to user

defined strings.  Here strings can be set up that will be

transmitted by pressing the hot key assigned to it.

Hexadecimal numbers can be specified by placing them between

"<>".  For example to transmit a NULL you could set up one

string like the following, <0>.

3.35  Options | Log to File

This is a toggle switch which allows you to log all data

transmitted and received to a file.  If the "Character

Display" (see Display | Character) is defined to be anything

but character, a "{T}" will be placed in front of the first

character transmitted after a receive and a "{R}" will be

placed after the first character received after a transmit.

If the character mode is selected only the characters

received and transmitted be be placed in the file.  This

option can be turned off by selecting it for a second time.

3.36  Options | Transmit File

This is a toggle switch that once selected will transmit a

complete file or will transmit until it is selected again

which will cancel transmitting.

3.4  Display Menu

3.41  Display | Clear

Clears the current ports screen as well as the display buffer.

3.42  Display | Split

Is a toggle switch that splits the current ports screen.

When the screen is split, all the transmit characters will be

displayed on the top half of the screen and all the receive

characters will be displayed on the bottom half.  Otherwise it

is in full screen mode, where all the transmit and receive

characters will be combined.

3.43  Display | Settings

Turns the display of the port settings on and off.  If

display port settings is off the terminal screen will expand

to the full width of the screen.

3.44  Display | Character

Allows you to select the character display mode.  There are

four options, Character, Hex, Decimal, and Mix.  In character

mode the actual character will be placed on the screen.  In

Hex, a hexidecimal representation of the character will be

displayed inbetween "<>".  In decimal a decimal representation

of the character will be displayed between "<<>>".  In mix mode

all characters between 32 decimal and 126 decimal will be

displayed as its character and the rest will be displayed

between "<>" as a hexidecimal representation.  The check box

"Change All Ports" located on the bottom of the dialog will

change the display character for all the ports open.  There

is also a Display Transmit Characters check box.  When this

check box is checked, all transmit characters will be

displayed to the screen, otherwise they will not be displayed.

3.45  Display | Show/Hide

This option allows you to temporarily hide a port.  This

feature would be used if you have more than one port open and

you require more display for a particular port.  When a port

is hidden, all receive characters will be stored and will be

displayed when the port is brought back to its visible state.

3.46  Display | 43/50 Lines

Toggles between 25 and 43/50 line displays.

3.5  Config Menu

3.51  Config | Load

Loads a configuration file.

3.52  Config | Save

Saves a configuration file.  If a configuration is not

currently open, a dialog box will come up for you to specify

a name for the file.  The default directory is the directory

where SimpTerm is located.  If a configuration file is

currently open it saves the current settings to the file.

3.53  Config | Save As

Saves the current configuration file to a user suplied name.

The default directory is the directory SimpTerm is located in.

3.54  Config | About

Displays copyright notice and version number.