README.1ST

Latest Documentation and Instructions for MONITORS

---------------------------------------------------------------------

THANK YOU FOR YOUR INTEREST IN MONITORS!

Dynamic Learning Systems

P.O. Box 805

Marlboro, MA  01752

Tel: 508-366-9487

Fax: 508-898-9995

BBS: 508-366-7683

CompuServe: 73652,3205

Internet: sbigelow@cerfnet.com

SHAREWARE AGREEMENT

--------------------------------------

This is a shareware version of MONITORS.  You may have acquired it from a BBS or other on-line resource, but you probably got it bundled with "Troubleshooting, Maintaining, and Repairing Personal Computers".

As a shareware product, you are free to use MONITORS for 30 days.  If you choose to keep MONITORS, you must register the software with Dynamic Learning Systems.  The registration fee is only $20 - a very reasonable fee considering the hundreds you can save by testing and aligning a monitor yourself (even more if you pursue monitor repair as a sideline or full-time job).  Just complete the shareware registration form REG.FRM, and remit the form and payment (M/C and Visa accepted).  As part of your registration benefits, you will receive:

A complete, up-to-the-minute, registered copy of MONITORS

The latest copy of BURNIN for long-term monitor testing

A free update of MONITORS

Dynamic Learning Systems also publishes The PC Toolbox - a premier PC/peripheral troubleshooting newsletter.  Normally, a 1 year subscription is $39, but when registering MONITORS, you can pick up a 1 year subscription as well for a total of only $49 (a $10 savings).  If you want to stay on top of PC and peripheral repair, we encourage you to consider The PC Toolbox.

DISCLAIMER and WARRANTY

------------------------------------------

This computer software is intended to assist the user in the testing and alignment of computer monitors.  DYNAMIC LEARNING SYSTEMS MAKES NO WARRANTY, EITHER EXPRESS OR IMPLIED, OF THIS COMPUTER SOFTWARE OR ITS ASSOCIATED DOCUMENTATION, INCLUDING, BUT NOT LIMITED TO, THE IMPLIED WARRANTIES OF QUALITY, MERCHANTABILITY, AND FITNESS FOR ANY PARTICULAR PURPOSE.  Neither Dynamic Learning Systems, nor anyone directly or indirectly connected with the development of this software and associated documentation shall be liable for errors or omissions contained herein, or for incidental or consequential damages, injuries, or financial or material losses resulting from the use, or inability to use, this software or associated documentation.  You understand and acknowledge that working with computer monitors carries substantial personal risk.  If you do not understand the implications of monitor alignment or are uncomfortable performing these alignments, refer the alignment to trained technicians.  This product is provided AS-IS, and the user bears all responsibilities connected with its use.

This diskette is warrantied against material defects for a period of 90 days from the date of purchase.  Return the defective diskette, with your name, address, telephone number(s), and the reason for your return, to:  Dynamic Learning Systems, P.O. Box 805, Marlboro, MA  01752

A replacement diskette will be mailed promptly.

INTRODUCTION

To successfully test and align a monitor, it is necessary to display a comprehensive set of test patterns, and ensure that each image follows a predictable set of criteria or behaviors.  Traditionally, specialized hardware tools have been employed as test pattern generators.  While signal generators produce clean, precise patterns, the outright cost for such a device is often more than the casual user or electronics enthusiast can justify - even professional service shops are looking for cost-saving options.  Since homes and businesses with monitors also have PCs, it makes good sense to use the existing PC as a test instrument.  This document shows you how to install MONITORS on your PC, and use it productively in a matter of minutes.

All About MONITORS

The companion disk actually contains two computer programs.  The feature program is MONITORS - a self-contained video board diagnostic and test pattern generator.  MONITORS is designed to work with MDA, CGA, EGA, VGA, and SVGA video systems, so it is compatible across a wide selection of hardware (both video adapters and monitors).  MONITORS allows you to check a PC's video adapter capabilities, and perform a suite of standard test/alignment procedures after a repair is complete.  The system requirements for MONITORS are listed below.

System requirements for MONITORS

CPU:				Intel i286, i386, i486, or Pentium

DOS:				ver 5.0 or later

RAM:				640KB (conventional memory only)

Mouse or Trackball:		not required

Video System:			MDA, CGA, EGA, VGA, SVGA

Hard Drive Space:		about 1.5MB for hard drive installation (optional)

Installing and Starting From the Floppy Drive

Your first task should be to make a backup copy on a blank floppy disk.  You can use the DOS diskcopy function to make your backup.  For example, the command line:

C:\> diskcopy a: a:   <ENTER>

will copy the original disk.  Keep in mind that you will have to do a bit of disk swapping with this command.  If you wish to use a floppy drive besides a:, you should substitute the corresponding letter for that drive.  If you are uncomfortable with the diskcopy command, refer to your DOS manual for additional information.  MONITORS IS designed to be run directly from the original floppy disk, so you can keep the original disk locked away while you run from the copy.  This allows you to take the disk from machine to machine so that you will not clutter your hard drive, or violate the liscense agreement by loading the software onto more than one machine simultaneously.

1.	To start MONITORS from the floppy drive, insert the floppy into the drive and type the letter of that drive at the command prompt and press enter.  The new drive letter should now be visible.  For example you can switch to the a: drive by typing:

C:\> a:     <ENTER>

the system will respond with the new drive letter:

A:\>_

2.	Then, type the name of the utility you wish to use:

A:\> monitors     <ENTER>

3.	The appropriate utility will start in a few moments and you will see the respective main menu.  If you are using a floppy drive other than a:, you should substitute that drive letter (such as b:) in place of the a:.

Installing and Starting From the Hard Drive

If you will only be using one PC and you have an extra 1.5MB or so, you should still go ahead and make a backup copy of the companion disk as described in the previous section, but it would probably be more convenient to install the utilities to your hard drive.  There is no automated installation procedure to do this, but the steps are very straightforward.

1.	Boot your PC from the hard drive and when you see the command prompt, switch to the root directory by typing the cd\ command:

C:\> cd\     <ENTER>

the system should respond with the root command prompt:

C:\>_

2.	Use the DOS md command to create a new subdirectory that will contain the companion disk's files.  One suggestion is to use the name MONITORS such as:

C:\> md monitors     <ENTER>

then switch to the new subdirectory using the cd\ command:

C:\> cd\monitors     <ENTER>

The system should respond with the new subdirectory label such as:

C:\MONITORS>_

You may of course use any DOS-valid name for the subdirectory, or nest the directory under other directories if you wish.  If you are working with a hard drive other than c:, substitute that drive label for c:.

3.	Insert the backup floppy disk into the floppy drive.  Use the DOS copy command to copy all of the floppy disk files to the hard drive such as:

C:\MONITORS> copy a:*.* c:     <ENTER>

This instructs the system to copy all files from the a: drive to the current directory of the c: drive.  If you have received MONITORS in a compressed format, you will need to unzip the file.

4.	After all files have been copied, remove the floppy disk and store it in a safe place.  Then, type the name of the utility you wish to use:

C:\> monitors     <ENTER>

The main menu for the desired utility should appear almost immediately.

Using MONITORS

When MONITORS is started, you will see a brief initialization text, followed by a disclaimer and media warranty (press any key to clear the disclaimer).  You will then see the main menu.  The top portion of the main menu lists the utility name and version.  Fourteen selections are available from the main menu (the main menu for version 1.00 is shown, but subsequent revisions may appear slightly different).  The first two menu selections allow you to test the video adapter and set the appropriate video mode for the monitor in use.  If you are done with the program, press <ESC> at the main menu to return to DOS.

Testing Your Video Adapter

MONITORS allows you to test the capabilities of your video adapter by selecting option "1" from the main menu.  The General Adapter Specifications illustrate an overview of the adapter's capabilities.  Six key pieces of information are provided.  The Chipset in use identifies the particular video chipset employed by the adapter.  MONITORS is designed to identify up to 16 types of video chipsets (a listing is provided below).  If the chipset can not be identified, MONITORS will indicate an "Unknown Chipset".  In addition to the type of chipset in use, the Chipset revision is also identified wherever possible.  If the chipset type is not known or the revision can not be determined, MONITORS will indicate an "Unknown Revision".

MONITORS compatibility in identifying video chipsets

1.	Ahead Systems

2.	ATI

3.	Avance Logic

4.	Cirrus Logic

5.	Compaq QVision

6.	IBM XGA

7.	NCR

8.	Oak Technologies

9.	Paradise

10.	S3

11.	Trident

12.	Tseng ET3000

13.	Tseng ET4000

14.	VESA (compatible)

15.	Video 7

16.	Weitek

The total amount of Video Memory available on the video board is listed next.  The specification is given in KB.  For VESA-compatible video boards, the VESA Version will be shown.  Otherwise, a "No VESA Driver Installed" message is shown.  If I/O registers are used with the video board, the I/O Register Base address will be listed.  Otherwise, an "I/O Registers Not Installed" message is displayed.  When a video board with a built-in VESA BIOS is used, the VESA BIOS Status is listed last.  When VESA BIOS support is not included, a "Not Installed" message is shown.  The last two entries indicate the General Video Adapter (the generic video system in use) and the recommendation for Set Test Display Mode (video mode that should be used for testing and alignment).

The next step in adapter analysis identifies each major video mode that the adapter can successfully emulate.  Available modes are highlighted in green, while incompatible modes are highlighted in gray.  A large number of VESA modes are tested.  One important thing to keep in mind when reviewing Video Mode Compatibility is that the available modes do not necessarily mean that the attached monitor is capable of working in every mode.  Once you have finished reviewing the available modes, press the <ENTER> key to return to the main menu.

Choosing Video Test Modes

By default, test patterns are generated in VGA at a resolution of 640x480x16.  This automatically accommodates the broad base of VGA and SVGA video systems in service.  However, MONITORS allows you to override this default and select a different video mode for test pattern displays.  There are six selections; MDA (for text-only displays), CGA, EGA, VGA, SVGA, and an Auto-Detect mode.  If you are not certain what mode is appropriate for your system, the Auto-Detect selection will generally make a reasonable choice for you.  You can also switch back and forth between different screen modes at your discretion.  Once you make a selection, the main menu will reappear.  If you change your mind and decide not to change the working mode, simply press <ESC> to return to the main menu.  You can see the availability of various test patterns under each screen mode shown below.

Availability of test patterns in every screen mode.

Pattern			SVGA		VGA		EGA		CGA		MDA

---------------------------------------------------------------------------

Color Bars		256 col		16 col		16 col		4 col		n/a

(Grey Scale)	10 gry		10 gry		n/a		n/a		n/a

Convergence (Cross)	graphic		graphic		graphic		graphic		n/a

Convergence (Dots)	graphic		graphic		graphic		graphic		n/a

Linearity		graphic		graphic		graphic		graphic		ASCII

Phase			graphic		graphic		graphic		graphic		ASCII

Focus			ASCII		ASCII		ASCII		ASCII		ASCII

White Purity		graphic		graphic		graphic		graphic		n/a

Red Purity		graphic		graphic		graphic		n/a		n/a

Green Purity		graphic		graphic		graphic		n/a		n/a

Blue Purity		graphic		graphic		graphic		n/a		n/a

Blank Raster		graphic		graphic		graphic		graphic		ASCII

High-Voltage		graphic		graphic		graphic		graphic		ASCII

n/a = not available

col = colors

gry = gray

Color Bars

The color bar display is available by pressing option "3" from the main menu.  In the SVGA mode, a palette of 256 colors is displayed.  In VGA and EGA, 16 colors are shown in the VGA display.  The CGA mode shows 4 colors.  Color bars are unavailable in the MDA mode.  Color bars are not used for specific alignment tests, but rather for general quality testing and settings of brightness and contrast.  Color bars provide a good overview of monitor quality.

When you are through with the color bars, press any key on the keyboard.  Displays in the SVGA and VGA modes will switch from color bars to a set of 10 gray bars.  Displays in the EGA and CGA modes will return to the main menu.  This is a gray scale linearity test which should produce 10 even graduations of gray (including black and white).  As with color bars, gray scale linearity is used to judge the quality of a monitor's gray scale generation.  Press any key on the keyboard to return to the main menu.

Convergence Test (Crosshatch)

Static convergence is an important test for color monitors of all resolutions.  When option "4" is selected from the main menu, a white grid will appear.  The use of white is important since all three electron guns are running at equal amplitude.  Other colors may be more visually appealing, but only pure white provides useful information.  When you observe the convergence grid, there should be no other colors (blue, green, or red) bleeding out from around the grid edges.  If there are, you will need to perform a static convergence alignment.  By pressing the "M" key, the grid color turns magenta (red and blue electron guns only).  This allows you to adjust the magenta convergence magnets.  By pressing the "W" key again, the grid returns to white, and you can adjust the white convergence magnets.  You can switch back and forth between magenta and white as needed by alternately pressing the "M" and "W" keys.  Pressing any other key will return to the main menu.  A convergence crosshatch pattern is available in all screen modes except for MDA.

Convergence Test (Dots)

This is another test for static convergence.  When option "5" is selected from the main menu, a pattern of white dots will appear.  The use of white dots remains important since all three electron guns must be running at equal amplitude.  As with the crosshatch pattern, other colors may be more visually appealing, but only pure white provides useful information.  When you observe the convergence dots, there should be no other colors (blue, green, or red) bleeding out from around the individual dots.  If there are, you will need to perform a static convergence alignment.  By pressing the "M" key, the dot color turns magenta (red and blue electron guns only).  This allows you to adjust the magenta convergence magnets.  By pressing the "W" key again, the dots return to white, and you can adjust the white convergence magnets.  You can switch back and forth between magenta and white as needed by alternately pressing the "M" and "W" keys.  Pressing any other key will return to the main menu.  The convergence dot pattern is available in all screen modes except for MDA.

Linearity Test

Option "6" from the main menu selects the linearity test pattern.  By using a series of geometric shapes (i.e. circles and squares), linearity tests show the "even-ness" of both horizontal and vertical raster.  If any portion of the screen image appears compressed or expanded, linearity should be adjusted.  When distortion occurs in the horizontal orientation, horizontal linearity needs to be adjusted.  When distortion occurs in the vertical direction, vertical linearity needs to be adjusted.  Graphic linearity patterns are available in all screen modes except MDA.  An ASCII pattern is used to check for MDA display linearity.  Press any key on the keyboard to return to the main menu.

Phase Test

Option "7" from the main menu selects the phase test pattern.  Ideally, an image should be horizontally centered in the raster.  You can see the phase of an image by turning up brightness until the raster is visible.  The thin white box around the perimeter of the image allows you to adjust horizontal phase until the image is reasonably centered.  This allows horizontal width to be adjusted evenly.  If phase is incorrect, widening the image will cause the edge closest to the edge of the raster to "run off" the display.  Graphic phase test patterns are available in all screen modes except MDA.  An ASCII pattern is used for the MDA mode.  Press any key on the keyboard to return to the main menu.

Focus Test

The focus test pattern is displayed by selecting option "8" from the main menu.  One of the best tests of focus is in the display of ASCII text.  When the text image appears out of focus, high-voltage levels to the CRT's focus grid(s) can be adjusted to compensate.  Older monitors provide a panel-mounted focus control, but newer monitor designs place the focus control on the main PC board.  When testing and adjustment are complete, press any key on the keyboard to return to the main menu.

The Purity Tests

Options "9", "A", "B", and "C" are color purity selections used to check color gun consistency across the display.  The white purity selection ("9") is by far the most common and widely used mode.  By filling the screen with a pure white image, it is possible to check for discoloration or uneven coloring that may indicate the need for manual degaussing.  The other color selections may also be selected at your discretion.  When you are done reviewing a purity test pattern, press any key on the keyboard to return to the main menu.  Note that the white purity test is available in all screen modes but MDA.  However, the red, green, and blue purity tests are not available in the CGA mode.

Blank Raster Test

Option "D" from the main menu starts the blank raster test.  Essentially, the blank raster is just a black image.  This allows you to increase screen brightness and check the presence of raster without the distraction of a screen image.  Blank raster is available in all screen modes.  When you are through, press any key on the keyboard to return to the main menu.

High-Voltage Test

Option "E" from the main menu selects the high-voltage test.  This pattern us used to check high-voltage regulation by blinking the middle white box on and off.  Ideally, the boarder should not flinch significantly.  If it does, you may need to make repairs to the high-voltage system.  Once the test is complete, press any key on the keyboard to return to the main menu.  A graphic image is used to test high-voltage regulation in all screen modes but MDA.  An ASCII image is used in the MDA mode.

When Trouble Occurs

The system requirements for MONITORS are very mild compared to other commercial software in the marketplace today.  As a result, the probability of problems with the software should be extremely rare.  Make sure that the monitor is connected properly, and that it is appropriate for the video board in the PC.

Check the Screen Mode

The most common error is choosing a screen mode that is incompatible with the monitor.  When choosing a screen mode, make sure that the monitor is capable of supporting it.  For example, choosing an SVGA screen mode for an EGA monitor will result in display problems.  However, this should not cause an error in the software.  Follow the proper key sequence to return to the main menu and select a more appropriate screen mode.

Not all video boards support the same video mode in the same way, so the SVGA mode used for MONITORS may not work properly for all systems.  If the displays in the SVGA mode appear distorted, return to the main menu and select the VGA mode.  Virtually all SVGA video boards and SVGA monitors support VGA.  Other than the initial color bar display, the VGA and SVGA test patterns are identical.

If problems persist, write your symptoms and system setup on a sheet or paper along with your name, address, telephone, and fax numbers (if possible), then mail or fax it to:

Customer Service

Dynamic Learning Systems

P.O. Box 805

Marlboro, MA  01752  USA

Fax: 508-898-9995

BBS: 508-366-7683

CompuServe: 73652,3205

Internet: sbigelow@cerfnet.com

Making MONITORS Better

Dynamic Learning Systems is committed to providing useful and reliable software tools, so we welcome your comments and suggestions on how we can improve MONITORS.  You can also reach us "on-line" through our BBS or on CompuServe.  Feel free to send your feedback to:

Customer Service

Dynamic Learning Systems

P.O. Box 805

Marlboro, MA  01752  USA

Fax: 508-898-9995

BBS: 508-366-7683

CompuServe: 73652,3205