Daniela's generic ATAPI Filter - Release 0.4.0

-----------------------------------------------

NAME

DaniATAPI.FLT  -  replacement for IBMIDECD.FLT, IBMATAPI.FLT, DaniADsk.FLT

and all other ATAPI related filters.

SYNOPSIS

This filter driver supports all types of ATAPI devices. It is a superset

of IBMIDECD.FLT, IBMATAPI.FLT and DaniADsk.FLT, and supports all devices

that the mentioned drivers do.

Additionally, all ATAPI devices can be accessed through the OS/2 ASPI

software interface. Each EIDE channel shows up as a SCSI channel, and

each ATAPI device as a SCSI device (master = SCSI ID 0, slave = SCSI ID 1).

DESCRIPTION

This release of my ATAPI filter driver is based on the IBMATAPI.FLT and

IBMIDECD.FLT sources.

My enhancements are:

- can deal with the ATAPI ZIP-250 and ZIP-750

- improved ZIP performance (up to factor 4 on ZIP-250)

- supports arbitrary swapping of 100MB, 250MB and 750MB media

- reflects the current media capacity up to FDISK, DISKIO or the like

- OS/2 boot from ZIP is possible

- faster 32bit PIO in conjunction with DANIS506.ADD (rel. 1.1.1 or later)

- Busmaster DMA mode works and is tested with Intel, VIA, AMD, HPT, SiS,

CMD, SiliconImage, ServerWorks, Promise, Nvidia and ACARD chipsets until

now.

Busmaster DMA is available now for ALi as well (CD/DVD-ROM is safe, but

some chip revisions fail on DMA write operations!). Due to hardware

limitations there is no 32bit PIO and no busmaster DMA possible for ATAPI

devices attached to a Promise Ultra (non-TX2) controller.

- ATAPI devices appear to the rest of the system as regular SCSI-2 devices

if used with DANIS506.ADD (rel. 1.1.5 or later). This may be disabled

globally or individually.

- advanced "warm-swap" support if used with DANIS506.ADD (rel. 1.1.9 or

later)

- support for PCCard (formerly: PCMCIA) ATAPI devices (like the Iomega

PCMCIA ZIP-250 or the Iomega Click!40)

- support for PCCard CD writers (CDRecord, RSJ)

- the translation of old-style 6-byte SCSI commands into their more

modern 10-byte counterparts is no longer hardcoded. The device is

sensed now to check if it requires this translation.

- like DaniS506.ADD, this driver stores boot messages internally for

later readout. To view them, issue "copy ATAPI$ con:", to dump them

into a file, issue "copy ATAPI$ anyfile".

- automatic media unload on shutdown if desired

- supports up to eight IDE channels

- supports ATAPI MO drives (tested with Fujitsu 1.3GB and 2.3GB drives)

- supports ATAPI units with more than one logical unit (LUN) built in.

Examples are the Panasonic/IBM PD drive (tested) or some CD-ROM changers.

- fix compatibility issues with the increasing number of devices that

bluntly violate the ATAPI specs.

- fix some hangs at driver initialization due to SCSI MODE_SENSE size

problems with particular ATAPI units.

- fix a hang at driver initialization due to slow recover from IDENTIFY

commands seen with some ATAPI units.

***  - support for high interrupts (IRQ16 and higher)

###  Caution: this release doesn't work with DaniS506 v1.7.x or older!

it is meant to be used with DaniS506 v1.8.0 only.

This means, all freeware utilites designed to work with SCSI devices

are supposed to work with ATAPI devices as well:

CDRecord, CDDA2WAV, TAR, ...

COMMAND LINE OPTIONS

This driver supports all switches and options that IBMATAPI.FLT supports.

Syntax Diagram

--------------

+------- /I --------+

+------------------+       | +---------------+ |

V		       |       | V		 | V

BASEDEV=DaniATAPI.FLT --+----------------++- /A:n +--+-------------+--+->

+--- /!V --------+ 	  +- /EJ -------+

+--- /EJ --------+ 	  +- /!SCSI ----+

+--- /ZA --------+ 	  +- /RSJ ------+

+--- /!SCSI -----+

+--- /RSJ -------+

>----------+-------------------------------------+------------------|

|	  +----------- /I ----------+	|

|	  | +---------------------+ |	|

|	  | V			  | V	|

+- /U:n +--+-------------------+--+---+

+- /EJ ------------+

+- /!SCSI ---------+

+- /TYPE:x --------+

+- /RSJ -----------+

In most cases, options prefaced with ! can be used with the ! modifier to

explicitly disable the option.  Options are valid both with and without

the ! modifer.

The values in parentheses are the default values if the option is not

present.

Options may be prefixed by "-" instead of "/" as well.

Generic options

---------------

/!V						    (quiet)

(not) Verbose mode

/EJ						    (don't eject)

eject medium at shutdown

/ZA						    (default mapping)

map ZIP drive to A:

/!SCSI						    (export all)

don't export any ATAPI devices as SCSI devices

/RSJ						    (off)

RSJ CDROM backdoor

Adapter options

---------------

/A:n						    (n/a)

Adapter number (0, 1, .. 7)

/I 						    (off)

Ignore adapter

/EJ						    (don't eject)

eject medium at shutdown

/!SCSI						    (export all)

(don't) export ATAPI devices as SCSI devices

/RSJ						    (off)

RSJ CDROM backdoor

Unit options

------------

/U:n						    (n/a)

Unit number (0, 1)

/UNIT:n

same as /U

/I 						    (off)

Ignore unit

/EJ						    (don't eject)

eject medium at shutdown

/!SCSI						    (export)

(don't) export ATAPI devices as SCSI devices

/TYPE:x						    (none)

type definition of swappable units

/RSJ						    (off)

RSJ CDROM backdoor

The export of ATAPI devices as SCSI-2 device can be enabled and/or disabled

globally, per adapter channel, or per individual unit by adding the options

/SCSI or /!SCSI. If these options are in front of any /A: option, they

apply globally. If they come after an /A:x option but before any /U:

option, they apply to adapter channel x and override the global setting. If

they come after any /U:y option, then they apply to the single unit y only

and override both the global and the adapter setting. Default is a global

visibility of all ATAPI devices as SCSI devices.

The export is globally disabled also and cannot be overridden if the driver

is renamed to any name beginning with IBM (case insensitive). This avoids

confusion of the OS/2 installer if you replace IBMIDECD.FLT on installation

disk #1 by a renamed version of DaniATAPI.FLT.

To enable booting OS/2 from an ATAPI ZIP drive, I introduced the new

/ZA switch ('map ZIP to A:'). It takes effect only if the BIOS has mapped

the ZIP to drive letter A: in order to boot from it; in all other cases

the /ZA option is a no-op.

If there are floppy disk drives in your system, then - as a side effect -

/ZA remaps floppy drive A: to B:, and floppy drive B: (if present) is

no longer accessable. Due to limitations in OS2DASD.DMD (?) floppy

reads and writes are a little 'rocky' in this case.

A boot from ZIP automatically locks the boot medium in the drive, media

swapping is no longer possible! This lock is imposed automatically by

OS2DASD. If you insist to eject the locked medium by the 'eject' command,

this command obviously is routed into nirvana and causes a kernel trap.

Some people (like me) don't like media left in removable media devices

(especially ZIP drives) on system shutdown, but tend to forget to eject

them before powering down the system. The new /EJ option fills the void. If

/EJ appears in front of all /A:n options, it acts globally on all devices

controlled by DANIATAPI.FLT; if /EJ comes after an /A:n but before

accompanied /U:m options, it acts on all units of that adapter channel; if

it comes after an /U:m option, /EJ acts only on that particular unit. In

any case, all units marked by /EJ receive a command to eject inserted media

on shutdown right after file systems are closed.

Units marked as "warm-swappable" on DANIS506.ADD's command line (by

/A:x /BAY) need the new option /TYPE: to advice DaniATAPI.FLT to report

appropriate device types to the device managers.

The same applies to PCCard adapter channels (found by /PCS on

DANIS506.ADD's command line). This is done by additional characters

designating the supported device types:

'z' or 'Z': all ZIP drives

'l' or 'L': the LS-120 drive

'c' or 'C': all CD-ROM like devices (CD-ROM, CD-R/W, DVD-ROM, DVD-RAM)

'w' or 'W': all CD writer like devices (CD-R/W, DVD-RAM)

'W' includes 'C' but additionally enables the features required for proper

operation of the RSJ CDWriter software even if there is no CD writer unit

attached at boot time.

There may be more in the future. Some examples on how to use this option:

1) You have an IBM ThinkPad 600 and intend to plug a floppy drive, a

CD-ROM, and a ZIP-100 into its UltraBay. The TPs have got the UltraBay

connected to the secondary EIDE port. Then you need

BASEDEV=DaniS506.ADD [other options] /A:1 /BAY

BASEDEV=IBM1FLPY.ADD /A:0 /FORCE:1 /U:0 /F:1.44MB

BASEDEV=DaniATAPI.FLT /A:1 /U:0 /TYPE:ZC

(the adapter numbers above are correct!)

This is tested on Dell Inspiron 4150 and should work on other notebooks

as well.

2) You have notebook which can be equipped with either a LS-120 drive,

a DVD-ROM, or a CD writer. Then you need

BASEDEV=DaniATAPI.FLT /A:x /U:y /TYPE:WL

To pick the correct values for /A and /U have a look at the boot messages.

INSTALLATION

Just copy DaniATAPI.FLT to bootdrive:\OS2\BOOT and add the line

BASEDEV=DaniATAPI.FLT

Make sure that all other ATAPI related drivers are inactive or come

after DaniATAPI.FLT in your CONFIG.SYS.

If you prefer chasing icons, you can do the copy using DDINSTAL; you

still have to tidy up CONFIG.SYS!

If you intend to run the BootOS utility or to install this filter driver

on a 8.3-only filesystem, I recommend to rename the driver to DaniATAP.FLT.

INTEROPERABILITY TIPS

This version of DaniATAPI.FLT works with DaniS506 v1.8.0 only. Due to

changes in the inter device driver interface, this version of DaniS506

no longer works with other ATAPI-related filter drivers. Thus there are

no interoperability issues with them.

Unfortunately, many people don't bother to read docs carefully enough or

at all. So I state it here explicitely once again:

To make SCSI or ATAPI devices visible to programs which use the ASPI API,

you'll need the following two lines to be present in CONFIG.SYS!

BASEDEV=OS2ASPI.DMD /ALL	    (/ALL makes all devices accessible)

DEVICE=[path]\ASPIROUT.SYS or [path]\ASPINKK.SYS

CDRecord:

There are no further options required if you like to run CDRecord/2, but

not RSJ with your CD writer. The writer is visible to CDRecord as a

regular SCSI CD writer by default. Just run "CDRecord -scanbus" to locate

the CD writer on one of the SCSI buses and memorize its address

(bus,unit,lun).

RSJ:

DaniATAPI.FLT is fully compatible to the RSJ cdwriter filesystem and its

accompanied utilities. In fact, if you run DaniATAPI.FLT instead of

RSJIDECD.FLT you can take advantage of the improved data transfer modes

(32bit PIO and busmaster DMA) on all supported chipsets (RSJIDECD.FLT is

derived from IBMIDECD.FLT and thus supports busmaster DMA on Intel chipsets

only). If your CD writer model is listed cddrv.inf you don't even need

LOCKCDR.FLT; DaniATAPI.FLT reorders its SCSI unit mapping if it finds

a CD writer unit so that RSJ is happy.

So there are several choices available (listed in order of preference)

1) BASEDEV=DaniATAPI.FLT <other options> /A:x /U:y /RSJ   or

/A:x /RSJ        or

/RSJ

BASEDEV=LOCKCDR.FLT

Both RSJ and CDRecord may use the writer. If your CD writer model isn't

listed in cddrv.inf, LOCKCDR.FLT from RSJ 3.x will autodetect and

autoconfigure it. DaniATAPI will install an additional backdoor to

re-enable regular CD ROM operation via OS2CDROM.DMD (or replacements)

even if it is hidden by LOCKCDR.FLT. You better prevent DaniATAPI.FLT

from exporting the CD writer as SCSI device by applying one of the

/!SCSI options, because it will be exported by LOCKCDR.FLT now (too).

RSJ and UDF peacefully coexist now.

2) BASEDEV=DaniATAPI.FLT

This enables full use of the CD writer unit as regular CD ROM device,

RSJ writer device, and CDRecord writer device. If your particular model

isn't listed in RSJ's cddrv.inf, then you need to paste the LOCKCDR.FLT

autodetection info from the RSJ control panel.

3) BASEDEV=DaniATAPI.FLT </!SCSI> or </A:x /!SCSI> or </A:x /U:y /!SCSI>

BASEDEV=LOCKCDR.FLT

This disables using the CD writer unit as a regular CD ROM drive. You

better prevent DaniATAPI.FLT from exporting the CD writer as SCSI device

by applying one of the /!SCSI options, because it will be exported by

LOCKCDR.FLT now (too). Both RSJ and CDRecord may use the writer. If your

CD writer model isn't listed in cddrv.inf, LOCKCDR.FLT from RSJ 3.x and

above will autodetect and autoconfigure it.

4) BASEDEV=DaniATAPI.FLT

BASEDEV=LOCKCDR.FLT

DEVICE=JJSCDROM.DMD /W

This is similar to the section above, but DaniATAPI will not install

the additional backdoor; with the /W option JJSCDROM will grab the

WORM unit exported by LOCKCDR.FLT.

BackAgain/2K:

The vendor supplied ATAPI tape driver (f.e. CDSEIDE.FLT,BTSEIDE.FLT)

doesn't work with DaniS506 v1.8.0.

SANE:

SANE users may find that the SCSI bus number, for their scanner, will

change when DaniATAPI.FLT is used. You may need to run FIND-SCANNER.EXE,

and edit the scanner address in the *.CONF file, to correct the address.

This should be a one time change, unless you switch back to the IBM

supplied drivers.

UDF:

Lots of UDF failures can be attributed to flaws with particular versions of

OS2CDROM.DMD or JJSCDROM.DMD. Best results with UDF 2.1.2 were seen with

OS2CDROM.DMD build level 10.127.

If you see UDF formatting failing like this

[d:\]format w: /FS:UDF /F

The type of file system for the disk is CDFS.

WARNING: Formatting will erase ALL data on this disk.

Continue (Yes or No)? Y

Formatting is in progress...Format failed.

Internal error

then you probably have to blank the CD-RW before. CDRecord is recommended:

[d:\]cdrecord blank=fast -eject -speed 10

FUJITSU MO drivers

these are obsolete now, everything is handled by DaniATAPI.FLT and the

DASD manager (both large floppy and partitioned media).

FILES

DaniATAPI.FLT  The driver itself.

DaniATAPI.DDP  The device driver profile for DDINSTAL

DaniATAPI.DOC  The file you are reading right now :-)

cmprssd\DaniATAPI.FLT

The same as above, but LXlite compressed. This one is *not*

guaranteed to work on every system.

BUGS

Hopefully none, but who knows ...

TO DO

- merge with the base S506 driver

COPYRIGHT

DaniATAPI - Copyright (c) 2001-2007 Daniela Engert. All rights reserved.

STATUS OF PROGRAM

DaniAATAPI is freeware.

DISCLAIMER

YOU ARE USING THIS PROGRAM AT YOUR OWN RISK! I don't take any

responsibillity for damages, problems, custodies, marital dis-

putes, etc. resulting from use, inability to use, misuse, pos-

session or non-possession of this program directly or indirectly.

I also don't give any warranty for bug-free operation, fitness for

a particular purpose or the appropriate behaviour of the program

concerning animals, programers and little children.

THE SOFTWARE IS PROVIDED ``AS IS'' AND WITHOUT ANY EXPRESS OR

IMPLIED WARRANTIES, INCLUDING, WITHOUT LIMITATION, THE IMPLIED

WARRANTIES OF MERCHANTABILITY AND FITNESS FOR A PARTICULAR

PURPOSE. THE ENTIRE RISK AS TO THE QUALITY AND PERFORMANCE OF THE

PROGRAM IS WITH YOU.  SHOULD THE PROGRAM PROVE DEFECTIVE, YOU

ASSUME THE COST OF ALL NECESSARY SERVICING, REPAIR OR CORRECTION.

Or, in a few words:

If its good, I am responsible.

If its bad, its all your fault. ;)

Permission is granted to redistribute this program free of charge,

provided it is distributed in the full archive with unmodified

contents and no profit beyond the price of the media on which it

is distributed is made. Exception to the last rule: It may be

included on freeware/shareware collections on CD-ROM, as well as

on magazine cover CD-ROMs.

All trademarks mentioned anywhere around here are property of their

owners and the like ...

AUTHOR

Daniela Engert

Internet: "Daniela Engert" <dani@ngrt.de>

EPILOGUE

Suggestions and bug-reports are always welcome. Well ... bug-reports

are perhaps not *that* welcome ... ;)