CardAccess Lite Revision 1.8 Documentation

(c) 1994-1998 altec ComputerSysteme

Contents :

___________

1. Introduction

���������������������������������������������������������������������������ͻ

�    1. Introduction                                                        �

���������������������������������������������������������������������������ͼ

CardAccess Lite is a software Package designed to help the Memory Card User

to get access to all the data on a PCMCIA Memory Card without knowing

anything of the hardware. Based on the lowest level of the PCMCIA software

structure, the Socket Services Interface, CardAccess hides the hardware from

the user without loosing any functionality of the card.

Socket Services acts with Adapters and Sockets. An Adapter is a kind of

Controller which Controls a number of Sockets. A Socket is the Slot in

which you insert a PCMCIA Memory Card.

Although Socket Services hides the hardware from the user there are two

different types of Adapters: Window based and register Based.

Window Based Adapters are able to map a Memory Card through a Window into

the Adress Space of the Processor, therefore reading Memory Card on this kind

of Adapter is very easy. Two Examples of Window Based Adapters and Socket

Service Implementations are the HP100LX /HP200LX and the HP OMNIBOOK.

Register Based Adapters Access Cards through a set of Control registers.

Special Support routines for reading and writing to the card must be

implemented. All altec desinged Memory Card Drives

are register Based Adapters therefore the full support for Window Based

Adapters will follow in the next release.

CardAccess Lite consists of two parts :

-  DOS Device Driver which optionally installs a Socket

Services Interface

-  CardXS.EXE Utility Program for direct browsing of Memory Card

Data

Installation :

Copy all the files on the Diskette to your harddisk, there is no special

Installation routine needed.

From the Command Line of DOS run MCD3.COM /S or EASY.com /S to install the

Socket Service Interface. Now you are ready to work with the

Utility Program or to write your own application.

Usage :

The Program is fully menu/Dialog oriented, so using this program

requires no additional information.