Flash Upgrade Applet for External Active Modem

# Installation

No special installation step required. Just simply unzip the zip file into a folder you preferred (say c:\temp).

# System Requirement

- Windows 95 / NT 4.0

- Connect the external RS232 modem to a COM: port

(COM1: - COM8:)

# Starting the Program

- Power on the modem, and execute the main program lashup.exe’ to start.

- Pls. close any other application program before upgrading the firmware.

- The applet will automatically selected the COM: port with modem connected (COM1: to COM8:)

- Once a valid COM: detected, it will collect the current default country code from the modem and also validating the T I3’ and T I119’ response for modem firmware verification

- lashUp’ only recognize the modem with the T I3’ and T I119’ response matched with the expected values as specified in the lashUp.dat’ data file. If matched, you will see the firmware upgrade menu. Otherwise, the program will terminate.

- Pls. provide the filename for the new firmware to be upgraded. You can select the desired firmware from a file dialog by clicking the ‘…’ button to browse directory and locate the latest firmware file (.dlf)

- Once the firmware filename is provided, you can click the pgrade’ button for flash memory upgrade

# lashUp.dat’ data file

In general, user will not need to modify the lashup.dat’ file as it is dedicated for the  Acer modem.

The lashup.dat’ should only be modified by the modem manufacturer if they have tailor-made a new set of T I3’ and/or T I119’ response for their 50RDK-based modem in the new firmware.

A general lashup.dat’ file is shown below:

;

; FlashUp Data File

;

Check length for ATI3 response = 4

Expected ATI3 response = "ACER56 Modem"

;

Check length for ATI119 response = 6

Expected ATI119 response = "Netxus D/F/V/SP"

It contains 4 lines (any line starting with semi-colon ‘;’ treats as a remark and will be ignored) and is self-explained.

heck length’ means how many characters should the program need to check from left to right between the received response from modem and the expected response.

For example, heck length for ATI119 response = 3’ means only 3 characters need to be compared (i.e. only ot” part will be checked)

Note that if heck length’ = 0 means o need to compare’. For example, if the firmware does not provide T I119’ response, you may set heck length for ATI119 response = 0’ to bypass comparison.

# Notes when using Windows 98

When running lashUp’ under Windows 98, you may encounter the following warning message:

Pls. ignore this warning and click “OK” to continue whenever you get this system message. It is only a warning and should be okay.

In fact, this message is referred to  serial I/O operation completed because the time-out period expired.’ when running lashup’ in Windows 98.

It would not have such message when running lashUp’ under Windows 95 / NT 4.0.