FlashCom

Document Control: FC.0101.081397A

Modem Flash Memory Loader

OEM Usage Guide

# Welcome

FlashCom was written by Tradewind Software for Rockwell Semiconductor Systems.  It is meant to be used by Rockwell OEMs for distribution to modem end users.

FlashCom reloads a modem flash-memory firmware.  Its purpose is to upgrade a modem to support new capabilities and to meet emerging industry standards.

This Guide describes how OEMs may configure FlashCom to meet their specific needs and how to prepare a distribution diskette (or Internet download file) for distribution to customers who have flash-loadable modems.

# Table of Contents

Welcome

Table of Contents

Overview

Supported Platforms

OEM Options (Summary)

FlashCom Distribution File Set

FlashCom Distribution Vehicle

User Interface

Built-in Tech Support Tools

Modem Trace Window

Overriding Automatic Modem Detection

COM Port Access Override

COM Port Speed Override

INI File Settings Summary

Example FLASHCOM.INI file

[General]

FileCnt= number

ValidatorCnt= number

[Options]

BlueScreen= 0 or 1

OEMAbout= string

OEMWarning= string

SplashBmp= string

SplashSecs= number

Debug and Tech Support Options

AutoDetect= 0 or 1

LoadSlowly= 0 or 1

OnePortOnly= number

ShowTrace= 0 or 1

[Files]

FileNN= filename.s37, loadOrder, friendlyName, description

[Validators]

ValidatorNN= mdmCmd, compareOp, text, comment

Technical Support Issues

The program crashes at startup

The program says oad Now (?)” with a warning

The program says issing files”

Program says o Modem Detected”.

Windows won recognize the PCMCIA card.

Index

ShowRokBmp= none

# Overview

FlashCom is intended for general distribution to users of Windows-based computers who already own a flash-loadable modem based on Rockwell chipsets.

## Supported Platforms

FlashCom has been tested with and works nominally on the following system configurations:

- 80386 or better CPU with at least 4 Megabytes of memory (just the minimum required for the Operating System; FlashCom memory usage is trivial)

- Operating systems:

- Microsoft Windows 3.1 and Windows 3.11

- Windows for Workgroups 3.11

- Windows 95 (including )SR2)

- Windows NT 3.51 and Windows NT 4.0

- Hard disk with at least 1 Megabyte of available storage

- A flash-loadable modem based on Rockwell chipsets (those supporting the AT** flash-load protocol) attached to COM1 - COM9.  The modem may be any of:

- External modem

- Internal modem

- Modem in a PCMCIA slot

## OEM Options (Summary)

OEMs can control the following facets of FlashCom operation:

- The image that is displayed during program load (the plash screen”) and the length of time that that image is displayed, and an option to cover the entire display area with a blue background.

- Information that is displayed in the Help>About FlashCom dialog box.

- Text of a warning message that is displayed just before loading begins.

- The S37 files (flash-load modules) that are to be loaded to the modem

- riendly names” and descriptions of the flash-load modules

- Modem validation sequences:  Modems are recognized by issuing AT commands and comparing the results to specific strings and wildcard templates.  OEMs can control what modems will be recognized as lash-loadable modems”.

These options are described in detail in the Error: Reference source not found section of this document, starting on page Error: Reference source not found.

## FlashCom Distribution File Set

A complete FlashCom distribution file set consists of the following files:

| File | Required? | Description |
| --- | --- | --- |
| FLASHCOM.EXE | Ö | The program that interacts with the user and detects modems and loads firmware to the modem flash RAM. |
| FLASHCOM.INI | Ö | Configuration file. Describes files and modem ID strings and sets various OEM options. |
| FLASHCOM.HLP | Ö | The context-sensitive help file seen by the user upon clicking the [Help] button. |
| *.S37 | Ö | The flash-load modules that reprogram the modem flash RAM firmware. This will normally consist of two files, the Flash Loader module and the Firmware module. |
| README.TXT | Ö | The OEM must provide a text file containing specific information applicable to the user modem. It is recommended that the OEM provide a technical support phone number and email address. Note: A file by this name is expected. The online help will attempt to open this file with Notepad when the user clicks certain hyperlinks. |
| OEMLOGO.BMP |  | A .BMP file that may optionally be displayed upon program startup. If missing or invalid, a small generic lashCom” logo image is displayed. A different name may be used |
| *.INF |  | Modem setup information files used by Windows95 and Windows NT for recognizing and configuring modems. Note: FlashCom does NOT use these files, but after the modem has been upgraded, the operating system may prompt the user for a disk containing modem driver files. It is recommended that these drivers me made available as part of the FlashCom file set. |

## FlashCom Distribution Vehicle

OEMs may distribute FlashCom in any of several ways:

- Diskette distribution: A set of files on a diskette, with or without a Setup program that copies those files to the user hard disk and optionally executes FLASHCOM.EXE upon completion of the copy operation.

- Internet distribution as a ZIP file: As a single .ZIP file with instructions to the user to use PKUNZIP to unpack the files into an empty directory and to then execute FLASHCOM.EXE.

- Internet distribution as a self-extracting EXE file: As a single .EXE file with instructions to the user to run the program (it would unpack itself and optionally launch FLASHCOM.EXE)

- Other: The OEM may use any desired distribution technique, as long as all required files end up in a single directory on the user hard disk.  Since FlashCom is meant to be used just once, it is recommended that your setup procedure NOT create a group box (Win3x) or add items to the Start Menu (Win95/NT).

|  | Note that FlashCom itself does NOT provide a single-file distribution vehicle. This is left to OEMs to handle in their preferred manner. |
| --- | --- |

## User Interface

Port Info List shows results of modem detection

Load Modules from the [Files] section of the INI file

Firmware rev read via ATI3 command to modem

Validated against the [Validators] section of the INI file

FlashCom is designed to be easy to use for even the most novice of computer users.  Basically, there is only one decision to make.

FlashCom Main Screen.

Immediately upon starting, FlashCom begins its modem detection sequence.  This is automatic and does not require user intervention.  The first choice for the user is at the Main screen, shown above.  In most cases, only a single item will be listed as a modem; that item will be pre-selected in the Port list.  If the user moves the highlight in the listbox, over a port that does not have a modem attached, the [Load Now] button becomes disabled.

The user may click the [Info] button to learn more about the load modules and/or click the [Modem Detect] button to rebuild the Port list.  The [Help] button displays context-sensitive help using the Windows help system.

The [Load Now] button begins the load operation.  Again, there is no user interaction; that is there are no decisions to make.  The Load Now button verifies that all load modules are present and that the modem is still available and answering to AT commands.  It then automatically loads the modules to the modem.

When the load operation is finished, FlashCom displays the previous firmware revision ID and the new one (as detected via ATI3):

After a  successful load.

# Built-in Tech Support Tools

Although FlashCom will require no technical support for the vast majority of users, experience with modem software has shown that there will always be a need for technical support staff to handle unusual situations and configurations.

Part of FlashCom design criteria was to simplify technical support for the OEM.  FlashCom provides these functions that may be useful when helping a customer through the upgrade process:

- Modem Trace: View all communication with the modem.

- Override automatic modem detection: In the event FlashCom crashes during modem detection, we provide a way to avoid having it crash every time it starts up.

- COM Port Detection Override: Avoids some potential IRQ conflicts by accessing just a single specific COM port.

- COM Port Data Rate Override:  Overrides the normal COM Port speed of 56K baud so that loading is performed at 9600 baud.

## Modem Trace Window

The (normally hidden) Modem Trace window

Trace box controls;

TX, RX, XX:	Click to turn on or off specific classes of trace message.

ToFile:	Click to start or stop duplicating all Trace messages to a file named TRACECOM.TXT
Warning: Logging the trace to a file can affect performance on some slower systems and therefore could possibly cause problems during a flash-load sequence.

Clear:	Click to clear the trace window.

Close:	Click to close the trace window.

OEM tech support personnel can have the user display a modem trace window by either of two methods:

- Click the [About] button in the Main window, and then press and hold the Ctrley while clicking the [OK] button to dismiss the dialog.

- Use Notepad (or other text editor) to modify the file FLASHCOM.INI.  In the [Options] section, edit (or add) the ShowTrace line:
	[Options]
	ShowTrace=1

OEMs may also find this trace window useful for testing before shipping.

|  | Be sure to set ShowTrace=0 in the FLASHCOM.INI file (or remove the line altogether) when you prepare the file set for final distribution. |
| --- | --- |

## Overriding Automatic Modem Detection

When FlashCom starts up, it normally proceeds immediately into its modem-detection sequence.  Experience has shown that on some systems (in particular, Win3x systems in which there are IRQ conflicts), it is possible for modem detection (actually just the simple act of attempting an OpenComm call) to cause a system lockup.

For this reason, FlashCom provides two ways to override Automatic Modem Detection.

- Press Ctrl while Launching: Instruct the user to press and hold the Ctrl key when double-clicking the FlashCom icon.  Keep Ctrl depressed for several seconds until the splash screen disappears and the Main Window appears.

- Use Notepad (or other text editor) to modify the file FLASHCOM.INI.  In the [Options] section, edit (or add) the AutoDetect=0;
	[Options]
	AutoDetect=0

## COM Port Access Override

When FlashCom starts up, it normally proceeds into its modem-detection sequence, which attempts to open each COM port on the system.  Experience has shown that on some systems (in particular, systems in which there are IRQ conflicts), it is possible just the simple act of attempting an OpenComm call to cause a system lockup.

For this reason, FlashCom provides a way perform Modem detection on just a single COM port.  In general, the user should resolve the IRQ conflict that is causing the problem.  However, this feature lets FlashCom proceed with the flash-loading operation -- assuming that accessing just the one specified COM port can be done safely and reliably.

To force FlashCom to open and access just a single COM port, choose either of these methods:

- Add a parameter to the FLASHCOM command line:  Instruct the user to use the File>Run (or [Start]>Run) command rather than double-clicking the FlashCom program icon.  Browse to locate the file FLASHCOM.EXE and add /1 (or /2 or /3 … /9)  to the end of the Command Line.

- Use Notepad (or other text editor) to modify the file FLASHCOM.INI.  In the [Options] section, edit (or add) a OnePortOnly=n line; for instance:
	[Options]
	OnePortOnly=2
will cause FlashCom to access only COM2.

## COM Port Speed Override

FlashCom normally communicates to the modem at a data rate of 56K, but on a very few slow computers running Windows 3.x,  a slower speed may be advisable.

For this reason, FlashCom provides a way to override the normal data rate.

- Use Notepad (or other text editor) to modify the file FLASHCOM.INI.  In the [Options] section, edit (or add) the AutoDetect=0;
	[Options]
	LoadSlowly=1

# INI File Settings Summary

This section of the OEM User Guide describes settings that may be specified in the FLASHCOM.INI file to control the behavior of the FlashCom.  OEMs should carefully check each of these settings in the FLASHCOM.INI that they will ship with the FlashCom file set.

To set and change the settings, use Notepad or any text editor.

## Example FLASHCOM.INI file

Here is an example FLASHI.INI file in its entirety:

[General]

FileCnt=2

ValidatorCnt=3

[Options]

SplashBmp=JoeLogo.Bmp

SplashSecs=3

OemAbout=For use with Joe's Modem Co. modems\nTech Support 818-555-1234

OemWarning=Click OK to begin the load process.\n\nYou cannot back out later.

[Files]

File01=BPFL40.s37,  1,Module Loader Code, This MUST be sent at the start

File02=100L300D.s37,2,Driver Code,        This is the actual 65K driver

[Validators]

Validator01= ATI3,x,TelePath,       NEVER attempt to flash-load these

Validator02= ATI3,=,V1.000-K56_DLL, Targeted upgradable modem

Validator03= ATI3,*,K56_D,          Any thing containing this is good

## [General]

The [General] section of the FLASHCOM.INI file specifies certain settings needed to process the rest of the INI file.

### FileCnt= number

This specifies how many items are in the [Files] section of the INI file (see below).  In most cases this must be set to 2.

Default setting:	(None)

Optional/Required:	Required

Example:

[General]

FileCnt=2

### ValidatorCnt= number

This specifies how many items are in the [Validators] section of the INI file  (see below).  This will vary, depending upon the target modems that you expect to find on users’ systems.

Default setting:	(None)

Optional/Required:	Required

Example:

[General]

ValidatorCnt=4

## [Options]

The [Options] section of the FLASHCOM.INI file specifies certain OEM display options and some debugging options for use by tech support personnel.

### BlueScreen= 0 or 1

This specifies whether or not to fill the window with a blue background (a familiar visual indication that a setup process is in progress)..

Default setting:	0 (standard window; no blue background)

Optional/Required:	Optional

Example:

[Options]

BlueScreen=1

### OEMAbout= string

This specifies some optional text that will be displayed in the About FlashCom dialog box.  This is a good place to display your company name, details about which modems this is meant to upgrade, a tech support hotline number, an email address, a web site URL, etc.

There is room enough for two lines of text and you may embed n” in the string to force a line break.

Default setting:	“” (none)

Optional/Required:	Optional

Example:

[Options]

OEMAbout=For use with Joe's Modem Co. modems\nTech Support 818-555-1234

### OEMWarning= string

This specifies some optional text that will be displayed at the point just before the loading begins.  The text is displayed in a message box with the standard [OK] and [Cancel] buttons.

This string may be up to 512 bytes ong. You may embed n” in the string to force line breaks.

Default setting:	“” (none)

Optional/Required:	Optional

Example:

[Options]

OEMWarning=Importatnt:\nDon reset the modem.\n  Click OK to start.

### SplashBmp= string

This specifies the name of the file that will be displayed at startup.  Do NOT include a drive or path in this name.  To find this file FlashCom looks into the current default directory and the directory in which FLASHCOM.EXE resides.

If this is missing or the BMP file is invalid, FlashCom will use a small generic bitmaps for the splash screen.

Default setting:	OemLogo.Bmp

Optional/Required:	Optional

Example:

[Options]

SplashBmp=MyLogo.Bmp

### SplashSecs= number

This specifies how long, in seconds, that the SplashBmp should remain on the screen at program startup.

Default setting:	5

Optional/Required:	Optional

Example:

[Options]

SplashSecs=3

## Debug and Tech Support Options

The following FLASHCOM.INI settings are NOT meant to be included in the shipping version of the FlashCom file set.  They may be used only by tech support personnel while helping a customer on the phone or via email, etc.

### AutoDetect= 0 or 1

This specifies whether FlashCom should immediately begin modem detection upon startup.  Set to 0 (zero) to avoid automatic modem detection.

Default setting:	1 (one)

Optional/Required:	Optional

Example:

[Options]

AutoDetect=0

### LoadSlowly= 0 or 1

This specifies whether FlashCom should set the COM port data rate to a slow value or send data at 56000 baud.  You might have a user change this if his (slow 386, win3x) computer is having trouble completing a load.

Set to 0 to load at full speed (56K baud) or 1 to load a 9600 baud

Default setting:	0 (load at full speed)

Optional/Required:	Optional

Example:

[Options]

LoadSlowly=1

### OnePortOnly= number

This specifies whether FlashCom should check just a single COM port when it performs modem detection.  You might use this if the standard (all ports) detection causes problems on a user system.

For the number, use: 0 to detect all ports; or 1 through 9 to detect ONLY COM1 through COM9.

Default setting:	0 (all ports)

Optional/Required:	Optional

Example:

[Options]

OnePortOnly= 2

### ShowTrace= 0 or 1

This specifies whether FlashCom should display the COM Port Activity Log window (a debugging aide, see page Error: Reference source not found).  This window may also be raised interactively by pressing the Ctrl key wile closing the About dialog box.

Default setting:	0 (remain hidden)

Optional/Required:	Optional

Example:

[Options]

ShowTrace= 1

## [Files]

The [Files] section of the FLASHCOM.INI file identifies the flash-load module that will be sent to the modem.  It is critical that this section is complete and that the files described are included in the FlashCom file set that is distributed to the end user.

### FileNN= filename.s37, loadOrder, friendlyName, description

There must be as many of these as are specified in the [General] FileCnt= setting.  In most case, there should be two entries, File01= and File02=

NN:	Two digits (i.e., 01, 02,…99) These should be in sequence from lowest to highest.

filename.s37:	The filename and extension of the S37-format flash-load module file.  Do not specify drive:\path information; these files must be in the same directory in which the application program resides.

loadOrder:	The order in which this file should be loaded.  The Flash-load loader file must have a 1 in this position.

friendlyName:	The module name for display to users; e.g., Flash Load Loader.

description:	Additional information about this file.  This text is displayed when the user clicks the [Info] button in the main window.  This text may be up to 256 characters.  You may embed \n in the string to break lines for screen formatting purposes.

Default setting:	(none)

Optional/Required:	Required

Example:

[General]

FileCnt=2

[Files]

File01=BPFL40.s37,  1,Module Loader Code, This must be sent first

File02=100L300D.s37,2,Driver Code,        The firmware that supports 56K

## [Validators]

The [Validators] section of the FLASHCOM.INI file contains information used by FlashCom to determine whether or not a modem should be considered lash loadable”.  If a modem fails the test, the user will still be able to attempt the load, but will see a warning message and have a chance to bail out.

### ValidatorNN= mdmCmd, compareOp, text, comment

There must be as many of these as are specified in the [General] ValidatorCnt= setting.  This will vary from OEM to OEM.  You will want to make sure that the Validators specifically exclude modems for which the load may be damaging and specifically include the target modems for which the load has been programmed.

The order can be important.  Once a modem has been excluded via any ! or x operator, or once it has passed any of the inclusive operator tests (=, ~, or *), no further checks are made.

NN:	Two digits (i.e., 01, 02,…99) These should be in sequence from lowest to highest.

mdmCmd:	This must be ATI3 (future version may support additional modem commands.

compareOp:	This is a one-character comparison operator.  It may be one of:

Inclusive Operators:

=	(equal sign) The return from  mdmCmd  must match text  exactly, including character case.

~	(tilde) The return from  mdmCmd  must match text, but character case is ignored.

*	(asterisk) The text must be contained in the return from mdmCmd (but character case is ignored).

Exclusive Operators:

!	(asterisk) If the text matches the return from mdmCmd (case insensitive) then the modem is specifically NOT considered to be a flash-load target.

x	(ex) If the text is contained anywhere in the return from mdmCmd (case insensitive) then the modem is specifically NOT considered to be a flash-load target.

text:	Text to use in the comparison (this string is compared to the text returned by the modem when it is issued the ATI3 command).  Leading and trailing spaces are removed.

comment:	This is not used or displayed, but should be entered to help document the file.

Default setting:	(none)

Optional/Required:	Required

Example:

[General]

ValidatorCnt=4

[Validators]

Validator01= ATI3,x,TELEPATH,          NEVER validate any of these

Validator02= ATI3,=,V1.000-K56_DLL,    An upgradable modem

Validator03= ATI3,=,V1.000HY-K56_DLL,  Another upgradable modem

Validator04= ATI3,*,K56_D,             Any thing with this is good

In the above example Validator 01 specifically excludes any modem in which the word ELEPATH” is returned anywhere in the ATI3 value.  Validators 02 and 03 identify specific strings that are expected to be seen on the target modems.  Validator 04 indicates that any modem that returns  K56_D anywhere in the ATI3 text will be considered to be flash-loadable (unless the modem has been excluded by a previous Validator).

|  | Sending the wrong load module to a modem can render the modem permanently inoperable. Try to be specific about which modems are targeted and excluded by the Validators section. |
| --- | --- |

# Technical Support Issues

### The program crashes at startup

This may occur on systems that have an IRQ conflict -- in particular on Win3.x.  Just opening a COM port (an unavoidable operation!) can cause this.  You may be able to proceed with the flash-load by having the user append /n (where n is the single COM Port that should be accessed) to the command line.  For instance, launch the program from the Run... dialog box via c:\upgrade\FLASHCOM.exe /3.  Alternately, you can have the user add a line OnePortOnly=n to the Options section of the INI file.

It is also possible to force FlashCom to bypass modem auto-detection altogether, as follows:

- Press Ctrl while Launching: Instruct the user to press and hold the Ctrl key when double-clicking the FlashCom icon.  Keep Ctrl depressed for several seconds until the splash screen disappears and the Main Window appears.

- Use Notepad (or other text editor) to modify the file FLASHCOM.INI.  In the [Options] section, edit (or add) the AutoDetect=0;
	[Options]
	AutoDetect=0

See Error: Reference source not found and Error: Reference source not found starting on page Error: Reference source not found.

### The program says “oad Now (?)” with a warning

FlashCom recognized that a modem is on the COM port, but it did not recognize the modem as a flash-loadable modem (it did not match anything in the [Validators] section of the FLASHCOM.INI file).  If you can verify that it IS a flash loadable modem, it is OK to proceed with the load attempt.

To see exactly what FlashCom is seeing, open the Modem Trace window (see page Error: Reference source not found) and run the modem detection.  The ATI3 string will be displayed in the Action Log in the Modem Detect dialog box and the raw COM port I/O  will be displayed in the COM Activity Log window.

If the ATI3 command returns only OK, ask the user if he attempted to load previously and did the previous attempt fail (this could be symptomatic of what happens when part one (the loader) made it into flash RAM, but the firmware did not).  If that is the case, try to determine why the previous attempt failed (to avoid a reoccurrence) and then go ahead and try loading again (click through the warning).

### The program says “issing files”

FlashCom will not start the load process if it cannot find ALL of the required load modules.  Have the user unzip (or otherwise refresh) the FlashCom file set.

### Program says “o Modem Detected”.

Specifically, this means that the COM Port opened without error, but the device attached did no respond to AT commands.  If the user has previously attempted a load and that load failed, and you see this message, you could have a serious problem (see below).  Otherwise, it just might mean that the external modem has not been turned on or the cables are not attached snugly.  Wiggle the cables and click [Detect] again.

### Windows won’ recognize the PCMCIA card.

If the user has previously attempted a load and that load failed, and you see this message, you could have a serious problem.  Although it takes a very unlikely sequence of events, it is possible for the flash RAM to be totally erased.  That is, it does not even contain the loader or any code that will respond to AT commands.

The first thing to try is [Modem Detect] several times.  The modem may eventually start responding.  If it responds as an nrecognized modem” you should try the load again.

If that fails, and the modem is a PCMCIA card, eject the card and reinsert it.  Windows should make the distinctive ard inserted” sound if the modem is working at all (if it IS working, try [Modem Detect] again and then try [Load Now]).

If Windows refuses to recognize the insertion, you could try cycling the computer off and back on -- just in case it is a problem with Windows.  Finally, if the PCMCIA card continues to fail to respond, the only apparent course of action is to have the user return the device for repair or replacement (OEMs should formulate a policy to handle this possibility).

# Index

[

[Files] Section, 16

[General] Section, 11

[Options] Section, 12

[Validators] Section, 17

A

AutoDetect=, 14

B

Bitmap for OEMs, 13

BlueScreen=, 12

BMP file, 2

Built-in Tech Support Tools, 6

C

COM Activity Log, 7

COM Port Access Override, 8

COM Port Speed Override, 9

D

dead modem, 20

F

FileCnt=, 11

FileNN=, 16

FlashCom Distribution File Set, 2

FlashCom Distribution Vehicle, 3

FlashCom.Ini, 2

flash-loadable, 17

H

Help file, 2

I

INF Files, 2

INI file, 2

IRQ Conflict, 19

L

Load Now (?), 19

LoadSlowly=, 14

Logo Bitmap, 13

M

Missing files, 20

Modem Trace Window, 7

Modem validation, 17

N

No Modem Detected, 20

O

OEM Logo Bitmap, 13

OEM Options, 1

OEMAbout=, 12

OEMWarning=, 12

OnePortOnly=, 14

Overriding Automatic Modem Detection, 8

Overview, 1

P

PC Card not recognized, 20

PCMCIA card, 20

Permanent Damage, 19

program crash, 19

R

ReadMe.Txt, 2

S

S37 files, 2, 16

ShowTrace=, 15

SplashBmp=, 13

SplashSecs=, 13

Supported Platforms, 1

T

Tech Support Tools, 6

Technical Support Issues, 19

U

User Interface, 4

V

ValidatorCnt=, 11

ValidatorNN=, 17

Rockwell Internal Addendum

The following FLASHCOM.INI setting is to be supplied only to selected OEMs.

### ShowRokBmp= none

This specifies whether the small 56 FLEX” bitmap image in the lower left corner of the Main Window should be displayed.  By default it IS displayed and it can only be hidden by an OEM who knows the setting exists and the correct value to be used.

Leave this setting out of the INI file to have the bitmap display normally.

Set it to none to hide the image.

Default setting:	“” (nothing, indicating to show the image)

Optional/Required:	Optional

Example:

[Options]

ShowRokBmp=none