WESTERN DIGITAL ETHERCARD PLUS - FAMILY LAN ADAPTER DRIVERS

FOR USE WITH NOVELL ADVANCED NETWARE 286 VERSION 2.1X AND

2.2.  AND FOR USE IN WORKSTATIONS WITH NETWARE 386 VERSION

3.0 AND LATER

Installation Guide

------------------

OVERVIEW

--------

To successfully complete the installation of the adapter driver

you must perform the following steps:

1.0 Hardware:

1.1    Determine the LAN adapter hardware type.

1.2    Determine the LAN adapter configuration.

1.3    Install LAN adapter hardware.

2.0 Software:

2.1    Generate Software.

2.2    Select Software Configuration.

* NetWare Generation Utility.

* Jumpers Configuration Utility.

2.3    Install LAN software.

Specific details are provided in the following sections.

INSTALLATION PROCEDURE

----------------------

1.0 HARDWARE

1.1   DETERMINING THE LAN ADAPTER HARDWARE TYPE

1.1.1 Jumper Configured Adapters

WD8003E        8K RAM

WD8003EBT     32K RAM

WD8003S        8K RAM

WD8003SH       8K RAM

WD8003WT       8K RAM

WD8013EBT     16K RAM

Note: Refer to JUMPERS.DOC in the root directory

for more information on jumpers.

1.1.2 Micro Channel Adapters

WD8003ET/A    16K RAM

WD8003E/A     16K RAM

WD8003ST/A    16K RAM

WD8003W/A     16K RAM

1.1.3 Software Configured Adapters

WD8003EB       8K RAM

WD8003W        8K RAM

WD8003EP       8K RAM

WD8013EP      16K RAM

WD8013W       16K RAM

WD8013EW      16K RAM

1.2   DETERMINING THE LAN ADAPTER CONFIGURATION

For information on RAM, Base I/O and IRQ, see the

SYSMAP.DOC file furnished.

Also refer to User Installation guide available with the

retail packaged adapter.

1.3   INSTALLING LAN ADAPTER HARDWARE

Refer to User Installation guide available with the retail

packaged adapter.

2.0 SOFTWARE

2.1   Generate Software

2.1.1 From Floppy

When generating NetWare 286 Software from a floppy drive,

the generation software will prompt the user to insert

the driver diskette at the appropriate time.

Note: NetWare installation procedures require the driver

files to be located in the root directory of a

floppy disk whose volume label is LAN_DRV_200.

If you copy these files to another floppy disk,

be sure that you set the volume label of the new

disk to LAN_DRV_200.

2.1.2 From Hard Disk

After copying the Netware 286 floppies to the hard disk as

described in the Netware 286 manual, a directory with the

name 'LAN_DRV_.200' must be created at the same level as

the 'LAN_DRV_.001' directory. Copy all the .OBJ files to

this directory.

- For Netware 2.1x -

Copy the WDPLUSSH.LAN file to the directory 'SHGEN-1'.

Copy the WDPLUS.LAN file to the directories 'AUXGEN'

and 'BRGEN'.  Then run either 'SHGEN' for a workstation,

'NETGEN' for a server or 'BRGEN' for a bridge.

- For Netware 2.2 -

Copy the WDPLUSSH.LAN file to the directory 'WSGEN'.

Copy the WDPLUS.LAN file to the directories 'SYSTEM-1'

and 'ROUTEGEN'.  Then run either 'WSGEN' for a workstation,

'INSTALL' for a server or 'ROUTEGEN' for a router.

For more information on using this software, refer

to NetWare 286 Installation Manual.

2.1.3 Running Appletalk On Your Server

First determine whether the network is using AppleTalk

Phase I or Phase II.  If you are not sure, read the

APPLE.DOC file located in this directory.

AppleTalk Phase I software is incorporated in the regular

286 server driver.

AppleTalk Phase II requires a special server driver called

WDPLS2 which is located in the Network21 directory.  These

driver files must be copied into the root of the SuperDisk

or the LAN_DRV_200 directory.  Either the standard driver

or AppleTalk Phase II driver, but not both, may be in the

root directory of LAN_DRV_200 when the INSTALL or NETGEN

programs are run.  If you are using the SuperDisk, the

APPLE2.BAT file can be used to perform the file copying.

APPLE2.BAT copies the following AppleTalk phase 2 files into

the root directory on the SuperDisk:

WDPLS2AD.OBJ

WDPLS2BD.OBJ

WDPLS2CD.OBJ

WDPLS2DD.OBJ

WDPLS2.LAN

The following AppleTalk phase 1 files are copied to

NETWARE21 directory for safe keeping.

WDPLUSAD.OBJ

WDPLUSBD.OBJ

WDPLUSCD.OBJ

WDPLUSDD.OBJ

WDPLUS.LAN

2.2   Select Software Configuration

Once the proper LAN driver has been selected, the desired

configuration parameters must be chosen from a menu.  This

Western Digital EtherCard Plus family workstation driver has

been designed to limit the need to reconfigure or regenerate

the IPX.COM file, even though a variety of Western Digital

EtherCard Plus family adapters and adapter configurations

may be present in your workstations.  In most cases

configuring the workstation's IPX.COM file to the default

values will function with all adapter types.  See the

specific adapter type for details.

2.2.1 Jumper Configured Adapters

When configuring a server or workstation employing a jumper

configured adapter, one of the first four configuration

entries must be used, the default being the first

configuration entry. If none of these choices match the

hardware configuration desired, then select the first/default

configuration entry and run Novell's JUMPERS.EXE version v2.00

configuration program. This utility allows the user to

configure the LAN software to use any supported configuration

parameters.  For more information on JUMPERS.EXE, refer to

the documentation furnished with the software.  The JUMPERS

utility is available from Novell or CompuServ.

The RAM location for jumper configured adapters is set by

the driver to the location specified in the configuration

selected.

- Selecting RAM Size -

There are configuration selections designating RAM sizes of

8, 16 and 32 Kbytes of RAM.

Adapters with 8 Kbytes of RAM can be used with configuration

selections which specify 8, 16 or 32 Kbytes of RAM.

Adapters with 32 Kbytes of RAM must be used with configura-

tion selections which specify 32 Kbytes of RAM.

The WD8003/WD8013 family AT bus 16-bit adapters can function

in two modes,  16-bit mode and 8-bit mode.

In 16-bit mode, the adapter performs 16-bit data transfers

and uses the full 16 Kbytes of RAM. In 8-bit mode, the adapter

performs 8-bit data transfers and uses only 8 Kbytes of RAM.

To operate the 16-bit adapter in 16-bit mode, place the

adapter in a 16-bit bus slot and configure the driver to a

configuration selection which specifies 16 or 32 Kbytes of

RAM.

To operate the 16-bit adapter in 8-bit mode, configure the

driver to a configuration selection which specifies 8 Kbytes

of RAM, or place the adapter in an 8-bit bus slot and specify

any driver configuration.

NOTE:  The Default StarLAN packet type is IEEE 802.3 as of

release V3.07 of the 286 Server and Workstation

driver.  If the Novell StarLAN (AT&T StarLAN

compatible) packet type is required, it can be

changed via the JUMPERS Configuration Utility.

2.2.2 Software Configured Adapters

- Workstation Software -

If one LAN adapter card is used, any of the first five

configurations can be selected when linking a workstation.

The shell driver automatically detects software configured

adapters and reads the configuration information directly

from the adapter, even if the configuration entry selected

specified a different I/O base, IRQ and RAM base.

If there is more than one LAN adapter, you need one of the

'Software Configured Adapter' selections for each adapter.

(See - Server Software - description.)

- Server Software -

Do NOT select the default configuration.  Select 'First

Software Configured Adapter' for the first LAN adapter.

Select 'Second Software Configured Adapter' for the second

LAN adapter, etc.

First, Second, etc., are determined by the lowest Base I/O

address of the adapter, i.e., if there are two adapters one

with Base I/O address 240 and the other has an address of

280, then 240 will be the 'First Software Configured Adapter'.

2.2.3 Micro Channel Adapters

- Workstation Software -

If one LAN adapter card is used, any of the first five

configurations can be selected when linking a workstation.

The shell driver automatically detects Micro Channel adapters

and reads the configuration information directly from the

adapter, even if the configuration entry selected specifies

a different I/O base, IRQ and RAM base.

If there is more than one LAN adapter card, one of the 'Soft

Configured Adapter' selections is needed for each adapter.

(See - Server Software - description.)

- Server Software -

Select 'First Micro Channel Adapter' for the first LAN

adapter.

Select 'Second Micro Channel Adapter' for the second LAN

adapter, etc.

First, Second, etc., are determined by the lowest slot number

of the adapter, i.e., if two adapters are in the machine in

lots 2 and 4, then the adapter in slot 2 is the 'First Micro

Channel Adapter'.

NOTE:  When there is more than one adapter in a machine,

each adapter needs to be configured so that

interrupts are not shared.  Do NOT use the automatic

configuration option when initializing the adapter

from the Reference Diskette.  Automatic

configuration assumes that interrupts are shared.

Instead, initialize the adapter by selecting the Set

Configuration option from the main menu.  Set the

interrupt vector to an interrupt not already in use

by another adapter.

2.3   Install LAN software

If installing an AppleTalk driver in a NetWare 286 server,

see NetWare for Macintosh Installation Supplement.

NOTE:  When installing a NetWare External Bridge to be used

with AppleTalk phase 2, the minimum requirements for

system memory are 5 Mbytes.

Licensed by Western Digital for use by its customers only.

Western Digital is a registered trademark of Western Digital

Corporation. Other trademarks mentioned herein belong to

their respective companies.

Copyright (c) 1991 Western Digital Corporation.

All Rights Reserved.