QUICK INSTRUCTIONS FOR USING THE 16 PORT VERSION OF GCOM...

Rule number 1 --- this driver won't work if you only have one

board.  You MUST have two cards installed.  If you only have one,

then use GCOM.SYS for 8 ports.

The installation procedure for the driver is the same except for

the CONFIG.SYS command line.  You must add the second board's

base address as follows in this example:

DEVICE=GCOM16.SYS (2E0,218,10,D)

^   ^   ^ ^

board #1 base address.....|   |   | |... optional DTR switch

|   |

|   |..... IRQ setting

|

|......... board #2 base address

Here the base addresses and IRQ will change for your particular installation.

You will only need to place an IRQ jumper on the first card at the IRQ

that you set in your CONFIG.SYS command line.  The second card does not

use an IRQ.

The optional DTR switch is used only for PCBoard and other BBS's that

perform a shell to dos for external protocols, doors, etc.