Instructions for using Og00_018.com Fossil with SmartCard 8 for OS/2.

The driver must initially be run to upload the operating system to the

SmartCard. Add the following line to your STARTUP.CMD file, since it

will run before the start window, and therefore load the operating system

to the card before bringing up the nodes.

Og00_018 2e0	;modify address as necessary, card ships at 2e0

;	218 is our second choice for no conflicts.

Each Node that you run must run the driver Og00_018 driver before

running the BBS Software.

You may do this by adding the following command line to your

autoexec.bat file.

Og00_018 p,2e0

The "p" option is required with os2. It causes Og00 to interrogate the

SmartCard before loading the Fossil Operating system to see if it is

already resident. If you don't have the "p" option, Og00 will load

the shared portion of the fossil every time it is invoked. This would

interfere with nodes already running. With the "p" option, Og00 loads

a 4k stub tsr in the window so that PCBoard or Wildcat can communicate

with the card.

Thus after a cold boot, the first time Og00 is run, Og00 loads the

COMMON portion of the SmartCard O.S. to the card as well as the local

portion. On successive loads with the "P" option, from other windows,

the COMMON portion is not reloaded.

This differs from the DOS/Windows version of G00 where the entire Fossil

is COMMON.

Og00 and G00 are not interchangable although G00 will work with a single

node under OS/2.

Known problems: As of 4/4/95

The PCBmodem program, used with PCBoard, for conditioning modems does

not operate with Fossil under OS/2. Solution- boot dos, run G00, run

PCBmodem to condition modems. We expect this problem should be fixed

sometime. Check for PCBmodem updates on SALTAIR BBS at 801-261-8976

4/17/95 word from pcboard is to expect fix shortly. See Steve Morris or

Stan Paulsen.

In your dos_settings for the BOARD.BAT node icons, set the idle_sensitivity

to 99 for each node.

In CONFIG.SYS file add the following

MAXWAIT=1

TIMESLICE=32,32

In AUTOEXEC.BAT file add the following

OG00_018 P,2E0

To lock your port at 57,600 add the following to your autoexec (after og00),

OSMART1 COM3:57600,N,8,1,L

In STARTUP.CMD  file add the following

OG00_018 2e0

Remember to select the "FOSSIL" port type in MAKEWILD or PCBSETUP, and

of course select the "Comm port number." No address or irq selection is

required.

Comm ports on the smartcard may begin with higher than 1.

og00_018 2e0 5   causes the comm ports to be com5-com12.

This can be useful if you have regular comm ports.

;;;;;;;;;;;;;;;;;;;

----------------------------------------------------------------------

...................................................................

OS/2 support 3/20/95

The g00 for os2 is called og00 (og00_017.com).

The smarts utility for os2 is osmart1.com.

NOTE: If Osmart is used to display port status, i.e.

"osmart s", it will display information about only those ports

which were set from the current dos session. It cannot tell

weather ports are on or off or locked or the baud rate or about

the handshake status in use except in the window in which those items

were operated on.

....................................................................

----------------------------------------------------------------------

problems as of 4/4/95

Wildcat 4.10 . The first release has a bug which is exhibited by the

following. Upon returning from a door such as WCCHAT or WCMAIL or any

other door, WILDCAT was not re-enabling the fossil port, there by

leaving the fossil disabled and the node dead in the water. WC was quick

to repair the problem, but if you have wildcat.exe and wildcat.ovr dated

prior to 3/28/95, you will have the problem. These files are available

from WC or call us. This problem should resolve itself shortly as they

ship the updated versions.

If you need a temporary solution, try the nasty mode, "N". Put it as

the first operand on the command line; G00 N 2e0. Among other things, this

causes a fossil port, once enabled, to stay enabled. In other words,

function 5 (Turn off fossil port) is disabled.

57600 is not a standard rate for standard fossil. Normal fossil calls max

out at 38,400. Wildcat uses an extended initialization protocol(function 1e),

supported by X00, which we support. To get 57,600 with PCBoard, you need to

use OSMART1 as described to LOCK the rate. It is recommended, however, that

with Wildcat also, if 57600 use is desired, that the rate be locked.

WCMODEM - When executed, this program shows the port type to be "SERIAL".

Set the port type to FOSSIL with function key 2.

MAKEWILD - Rememeber to set the port type to FOSSIL.

The port address and irq will be in reduced intensity once fossil

is selected, as they are meaningless.

----------------------------------------------------------------------

...................................................................

**** FIX FOR OS/2-WARP ******

***** 5/19/95           ******

The new versions og00_018.com and osmart2.com fix a problem where

an error message announcing that COM1 is already in use by another

application.  This message appears when RAY GUINN'S SIO and VSIO have

been loaded along with a mouse on COM1 and og00 is started.

GTEK,Inc.

#############################################################################################