GTEK SmartCard-8 Watchdog Driver 1.0 for OS/2                               29-8-95

This is a Watchdog driver for GTEK's SmartCard-8 eight port serial card.

It enables the hardware watchdog of the SmartCard-8 and refresh it

periodically as long as the computer is alive. If this refresh fails, because

the computer hangs, the watchdog triggers a hardware reset.

To install the driver run the ddinstal utitlity, and select the driver with

the port address of your SmartCard-8 card. The default address is

100.

The ddinstal copies the driver to the os2/boot dir and add the following

lines to the config.sys file.

DEVICE=C:\OS2\BOOT\WDOGSM8.SYS PORT=xxx

REIPL=ON

Where xxx is the port address of the SmartCard-8. If no port= statement

is specified, the address 2e0 is used.

The first runs the Watchdog driver, the second tells OS/2 to reboot

automatically, if a trap occurs, which would not trigger the Watchdog,

as the driver is still running after a trap.