REALmagic OS/2  Driver(PDD)  Beta Release  1.20

Date  : July 7,1995

The following Drivers and files are  upgraded  for the Beta release 1.20:

To install these drivers run:
          UPDATE.BAT  in the C: drive      from DOS

UPDATE.CMD in the C: drive      from OS2

I am working with IBM to put together a Corrective Service Diskette (CSD) or a

Mini-INSTALL program that will allow us to ship these drivers and files directly to our

Customers and Beta-Sites .

MPGIO.DLL

MPGDC.DLL

SVSH.DLL

IOPRNLS.DLL     To install this you need to either boot to DOS or do ALT+F1 and C  to go

to command prompt and then copy this.

VIDVCI.DLL

VIDRMS.SYS

VREEL.BIN

AUDIOSH.DLL

AUDIOIF.DLL

RMAUDIO.SYS

R0STUB.SYS

RMLBASE.SYS

Add the following line in the CONFIG.SYS:

BASEDEV=RMLBASE.SYS   /io:260   /i:11  /p:340  /sa:0000 /ct:2

As you see the iobase address and interrupt number , the CD-ROM address , SCSI

Address, CD-ROM type can be set/modified as desired.

DSP1.BIN and DSP2.BIN

mpgdev1.sys         This is only needed for DOS & WIN-OS/2 session

Please refer to OS2REL1. DOC for more information.

This Release(1.20) has the followings updates:

Native OS/2:

-Updates to ask the MPEG video device driver which DSP microcode file

-Smoothed out the jump in video playback after Pause/Stop/Rewind/Seeks.

- Move 8 bytes audio timing information into the IDC DDCMD_WRITE packet.

-Updated to fix problem with reading Japanese VIDEO CDs. They were not

recognized as MPEG data due to 7 sectors of leading zeros.

-Audio Stream Handler

updated to pass 8 bytes of MPEG Audio timing information

-Audio VSD

updated to pass 8 bytes of MPEG Audio timing information

- Updated driver to run on Orchid's Kelvin MPEG card

and DEC’s Full Video card.

- and more

Problems/Open issues:

1. Problem with stopping MPEG files after running the  files for few hours or more.

With the above bug fixes and upgrade , it seems this problem is diminishing.

2. They do have the support for the Video-CD (White-Book) which works fine with MOST of the

CD-ROMS(TOSHIBA,IBM,MITSUMI)

The current SONY DRIVER (SONY31A.ADD) which is provided by IBM does not play VIDEO-CD.

This is been reported to IBM and SONY. Sony is working on this problem. We sent them Two

REALmagic Boards on  June 29,1995

Also, the problem with Panasonic CD-ROM interface is reported to IBM.

3. OS/2 WARP does not have the support for CD-I movie(Green-Book file format.)

IBM is working on this. They will have a driver for playing CD-I movie soon.

4. The message regarding the RMAUDIO.SYS can not be loaded is due to the interrupt conflict.

Check the interrupt numbers(IRQ) , make sure there is not any conflict between the one

used by RMAUDIO and the one used by SoundBlaster. Remember the IRQ 15 which is the

default IRQ selected by OS/2 WARP setup for RMAUDIO does NOT work for REALmagic 2.

With the new RMAUDIO, DSP2.bin  and  a correct IRQ RMAUDIO should have no problem

getting loaded.