TITLE : 4019 SYSTEM BOARD IDENTIFICATION

MODELS 001 AND E01 SYSTEM BOARDS ARE PHYSICALLY INTERCHANGEABLE.

HOWEVER, WHEN REPLACING THE SYSTEM BOARD USE CAUTION IN PARTS

ORDERING TO ENSURE THE CUSTOMER'S PRINTER IS NOT "UPGRADED" OR

"DOWNGRADED" IN ERROR.

FURTHERMORE, DO NOT ATTEMPT TO UPGRADE A MODEL E01 SYSTEM BOARD

TO AN 001 SYSTEM BOARD BY REPLACING THE EPROMS.

PRINTERS MANUFACTURED AFTER DECEMBER, 1990 HAVE A RASTER PORT

CONNECTOR (J9) INSTALLED ADJACENT TO THE PARALLEL/SERIAL HOST

CONNECTION (REFER TO THE HM&S Z544-4195-02, PAGE 5-4, LOCATIONS.

NOTE:  THE DATE OF MANUFACTURE IS FOUND ON THE INFORMATION

PLATE LOCATED ON THE BACK OF THE PRINTER.

SYSTEM BOARD IDENTIFICATION/FRU PART NUMBER

EACH SYSTEM BOARD HAS AN ID TAG AT THE LOWER LEFT CORNER OF THE

BOARD.  BOARD IDENTIFICATION CAN BE ACCOMPLISHED BY LOOKING FOR

THE FOLLOWING NUMBERS TO BE INCLUDED WITHIN THE ID TAG NUMBER.

IDENTIFICATION                         FRU P/N

- 001 BOARD HAS THE NUMBER 1039034              1383462

- 001 W/RASTER PORT HAS THE NUMBER 1058200      1383901

- E01 BOARD HAS THE NUMBER 1320080              1383456

- E01 W/RASTER PORT THE NUMBER 1320060          1383902