IBM ISA Ethernet adapter Setting using ENETCFG.EXE

--------------------------------------------------

About ENETCFG

-------------

ENETCFG is a standalone DOS-based program that will configure the

IBM ISA Ethernet adapters.  The executable file - ENETCFG.EXE - is

being made available by IBM Ethernet Development.

ENETCFG can be executed from the DOS prompt.  To see the

available command line parameters that can be used with ENETCFG,

simply type at the DOS prompt: ENETCFG (upper

or lower case).  This will cause the program to display the

available parameters as shown here:

Ethernet Configuration Utility   Version 1.30

Copyright (c)  1995 IBM Corp.  All rights reserved.

Usage: ENETCFG options parameter

Options List:

-config          = Display Adapter Information Only

-enet <adapter>  = Select Ethernet Adapter (Default -enet 1)

-media <type>    = Select Media Type

-io <address>    = Select I/O Address

-prom <location> = Select Boot PROM Location

-int <level>     = Select Interrupt Level

-arch <mode>     = Select Architecture Mode

-chrdy <time>    = Select Channel Ready Timing (Default -chrdy normal)

-link <status>   = Select Link Pulse Status (Default -link enable)

Parameter List:

<adapter>  = 1, 2, 3, or 4

<type>     = tpi, thin, or thick

<mode>     = io_port or shared

<time>     = normal or fast

<address>  = disable, 240, 280, 2c0, 300, 320, 340, or 360

<location> = disable, c800, cc00, d000, d400, d800, or dc00

<level>    = disable, 2, 3, 4, 5, 7, 9, 10, or 11

<status>   = disable or enable

Example:  enetcfg -enet 2 -media tpi

ENETCFG can be used to write the configuration of the adapter

at every power-up to assure that the adapter is configured as

required.  The EEPROM on the adapter and the configuration

registers of the adapter's controller are written by ENETCFG

and the adapter will be set to configuration required for

the applications it is to be used for.