-------------------------------------------------------------------------

TITLE A: IBM PS/2 RUNNING UNDER OS/2 2.1

The IBM PS/2 system unit MUST have an XGA-2 adapter or XGA-2 on the

plannar.  XGA-2 is included on all 95xx systems.  85xx systems must be

upgraded with an XGA-2 Adapter or equivalent OEM adapter. The XGA-2

adapter is fully programmable and can support many modes, in contrast to

the older XGA adapter, which is fixed frequency and CANNOT support

'flicker free'.

OS/2 requires version 2.1. or version 2.0 with required CSDs applied and

the  system reconfigured (CSDs are available through System Extra).

1) OS/2 2.1 go into OS/2 SYSTEM -> SYSTEM SETUP -> SYSTEM folder. The

first two pages are called "SCREEN":

Page 1 contains a list of valid screen resolutions and

Page 2 is "Change XGA DMQS Display Configuration Profile".

2) Page 2, click the pull down box to see a list of monitor types.

For early levels of 2.1 select 9515. For the latest level of 2.1

select 6324, 6325 (14V and 15V) and 9524, 9525 (14P and 15P). Close

the folder, shut-down the system and reboot.

3) Return to Page 1 of the folder. Select resolution (1024 x 768 x 256

colors), close the folder, shut-down and reboot again.

The monitor will now be operating in Flicker-Free 1024 x 768at 76Hz.

-------------------------------------------------------------------------

TITLE B: IBM VALUEPOINT 6384-CXX (CIRRUS) RUNNING UNDER OS/2 2.1

1) From OS/2 2.1 Main Menu ... Select OS/2 Window.

2) Type DSPINSTL

3) Select 'Primary Monitor'

4) Select 'Install using Monitor Adapter Utility Program'

5) Insert 'Cirrus Windows Monitor Driver Disk 1' and

type A:\CLUTILS\CLMO DE.EXE

6) Select 'Monitor Type'

7) Select monitor type and refresh rate, refer to table on page 6 of

'IBM PS VP 386 SVGA Drivers / Utilities'.  Use TYPE 7 High Refresh

for all the above referenced monitors except 6312.  For 6312 use TYPE

5 High Refresh.

8) Exit CLMODE

9) Select Required resolution from table displayed.

10) At ? specify the source directory.  Insert IBM OS/2 2.1 Monitor

Driver Disk 1.  At prompt insert IBM OS/2 2.1 Monitor Driver Disk 2.

11) Perform Shutdown as directed.

12) Re-boot system. Mode and refresh will now be setup as selected.

-------------------------------------------------------------------------

TITLE C: IBM VALUEPOINT VP2-S3 (6382/4-F,K,M,W,6387-M,W) 16M DRIVER W/ OS2/2.1

1) Install drivers using information contained within 'Install

Instructions' in README.S3 file of S3-16M Drivers Diskette.

2) From OS2 Main Menu

3) Select OS2 Window

4) Type DSPINSTL

5) Select 'Primary Display'

6) Select 'Install using display adapter utility program'

7) Insert S3 DOSCAD DRIVERS diskette and type 'A:util refresh.exe

8) On refresh panel select 640x480,800x600 and 1024x768 to be 72Hz

(Note if using a 6312 1024x768 can only be set to 60Hz and thus is

not 'flicker-free').

9) Insert S3-16M Drivers Diskette and click on 'Install'.

10) Shutdown

11) Click on OS2 System.

12) Click on System Setup, Click on System

13) Select Resolution required from table provided.

14) Shutdown and Re-boot; Selected resolution and refresh should now be

displayed.

-------------------------------------------------------------------------

TITLE D: IBM VALUEPOINT 6384-CXX (CIRRUS) RUNNING UNDER DOS/WINDOWS

1) Install Drivers using "386 SVGA Drivers / Utilities" - Users Guide

Page 8 / 9 and "Monitor Drivers / Utilities Cirrus - Microsoft

Windows Monitor Driver" Diskettes 1 and 2.

TIP - if asked for Font files, these may be found on Windows

Diskettes 1 and/or 2.

2) From Windows Main Menu, click on Windows Set-up.

3) Click on Options and select Change System Settings.

4) Click on Down Arrow of Monitor section to list modes available,

select mode required and click.

5) On ? driver to use, Select current driver.

6) Restart Windows.

To modify refresh rate for FLICKER FREE operation:

7) Click on File and Exit Windows.

8) At Command Prompt enter C:\CLUTIL\CLMODE.EXE

9) Select Setup, select Monitor type.

10) Select Monitor Type and Refresh High / Low from table supplied.

Select TYPE 7 High Refresh for all of the above referenced monitors

except 6312.  For 6312, select TYPE 5 High Refresh.

11) Exit CLMODE

12) Restart Windows.  Mode and refresh rate will now be as selected.

-------------------------------------------------------------------------

TITLE E: IBM VALUEPOINT 6384-F,M,W XX (TSENG) RUNNING UNDER DOS/WINDOWS

1) Install drivers using '486 SVGA Drivers / Utilities Users Guide'

Page 33 / 34 and 'Display Drivers / Utilities Tseng - Microsoft

Windows 3.1 drivers' diskettes 1 and 2.

2) From Windows Main Menu click on 'Setup'.

3) Click on 'Options' and 'Change System Settings'.

4) Click on down arrow,list of modes / resolutions supported will appear,

click on your choice. Insert diskettes if prompted (for fonts etc).

5) Select ' Current Driver'

6) Restart Windows.

TO MODIFY REFRESH RATES USING VMODE.EXE

7) Exit Windows and return to DOS prompt.a:

8) Install VMODE software using USERS GUIDE Page 4.

9) Set VMODE scan rates within UTIL directory as per tables on USERS

GUIDE Page 6. eg command for 640x480 at 72Hz is VMODE 72Hz ;

command for 800x600 at 72Hz is VMODE 48K.

10) Return to Windows;display should now be functioning in 'flicker free'

mode.

--------------------------------------------------------------------------

TITLE F: HOW TO INSTALL XGA WINDOWS DRIVER v2.10 (GREEN SCREEN FIX)

IN THE FOLLOWING INSTRUCTIONS REPLACE Z: WITH THE DRIVE ON WHICH YOU HAVE

INSTALLED OS/2

1. INSERT THE XGA DEVICE DRIVERS v2.10 DISKETTE IN DRIVE A:

2. OPEN AN OS/2 WINDOW

3. TYPE THE FOLLOWING COMMANDS:

EXPAND A:\WINV31\XGACONF.CPL Z:\OS2\MDOS\WINOS2\SYSTEM\XGACONF.CPL

EXPAND A:\WINV31\XGACONF.HLP Z:\OS2\MDOS\WINOS2\SYSTEM\XGACONF.HLP

EXPAND A:\WINV31\XGAMSG.GML  Z:\OS2\MDOS\WINOS2\SYSTEM\XGAMSG.GML

EXPAND A:\WINV31\XGA210.DRV  Z:\OS2\MDOS\WINOS2\SYSTEM\XGA210.DRV

CD Z:\OS2\MDOS\WINOS2\SYSTEM

ENSURE THE NEXT LINES ARE TYPED INDIVIDUALLY - DO NOT USE WILDCARDS!

COPY XGAOEM.FON 8514OEM.FON

COPY XGASYS.FON 8514SYS.FON

COPY XGAFIX.FON 8514FIX.FON

CD ..

E SYSTEM.INI

CHANGE THE REFERENCE OF XGA.DRV TO XGA210.DRV.  SAVE & QUIT

RENAME Z:\OS2\DLL\WINPRF.DLL WINPRF2.DLL

E Z:\AUTOEXEC.BAT

ADD A LINE AS FOLLOWS:

SET DMQSPATH=Z:\XGADMQS

(ASSUMING YOUR *.DGS FILES ARE IN Z:\XGADMQS).  SAVE & QUIT

NOW START A FULL-SCREEN WIN-OS/2 SESSION. GO TO CONTROL PANEL AND START THE

NEW XGA SETUP ICON. FROM HERE YOU CAN CHANGE SCREEN ADDRESSABILITIES AND

COLOURS, AND DISPLAY TYPES. THESE CHANGE INDEPENDENTLY OF CHANGES TO THE

OS/2 DESKTOP SCREEN.

--------------------------------------------------------------------------