TITLE: MULTIPLE DDD (DEFUNCT) DISKS ON PC-SERVER 320/500/520/720

SYMPTOM:

SERVER 320/500/520/720 RAID ARRAY SYSTEMS MAY BE UNABLE TO START

THE FIXED DISK DRIVES IF MULTIPLE DRIVES ARE DDD (DEFUNCT).

THE FOLLOWING ERROR MESSAGE WILL BE REPORTED AFTER SELECTING

THE "REPLACE" OPTION IN THE RAID OPTION DISKETTE:

"ERROR IN STARTING DRIVE..."

PROBLEM ISOLATION AIDS:

IF MORE THAN ONE DRIVE APPEARS DEFUNCT, ALL DRIVES SHOULD

NOT BE DEFECTIVE; DATA MAY NOT BE LOST. IT IS POSSIBLE FOR

ONE FAILING DRIVE TO CAUSE ONE OR MORE OTHER DRIVES ON THE

SAME BACKPLANE TO APPEAR AS DDD.  THIS CAN OCCUR WHEN THE

USER PERFORMS ANY OPERATION THAT RESETS THE SCSI BUS AFTER

THE ORIGINAL DRIVE PROBLEM.

ENGINEERING IS INVESTIGATING THIS SITUATION; THIS TIP WILL

BE UPDATED AS NEW INFORMATION BECOMES AVAILABLE.

ALSO REFER TO RECORD H132397.

FIX:

USING THE PROCEDURE BELOW, ATTEMPT TO RESTART EACH DRIVE

WITH THE "REPLACE" OPTION ON THE RAID OPTION DISKETTE.  THE

"REPLACE" OPTION WILL BRING EVERY FUNCTIONAL DRIVE BACK TO

AN ONL STATE EXCEPT THE LAST DRIVE IN EACH ARRAY, WHICH MUST

BE REBUILT.

IF ANY DRIVE (OTHER THAN THE LAST ONE) FAILS TO RETURN TO AN

ONL STATUS, THAT DRIVE MAY REQUIRE REPLACEMENT.  LEAVE THE

FAILING DRIVE OUT OF THE ARRAY, RETURN THE OTHER FUNCTIONAL

DRIVES TO AN ONL STATUS, THEN INSTALL AND REBUILD THE

REPLACEMENT DRIVE.

A. PHYSICALLY DISCONNECT ALL DEFUNCT DRIVES, EXCEPT THE

ONE YOU ARE TRYING TO START.

B. WITH THE ONE DEFUNCT DRIVE INSTALLED, SELECT THE "REPLACE"

OPTION IN THE REPLACE/REBUILD MENU TO START THE DRIVE.

C. ONCE STARTED, YOU CAN INSERT THE NEXT DEFUNCT DRIVE

AND START IT.  USE THIS PROCEDURE TO ISOLATE AND REPLACE

A SUSPECTED FAILING DRIVE.

D. CONTINUE UNTIL ALL (FUNCTIONAL) DEFUNCT DRIVES HAVE BEEN

STARTED. THE LAST DRIVE OF EACH ARRAY WILL REPORT AN OFL

STATUS.  TO BRING THE DRIVE BACK TO AN ONL STATE, REBUILD

THE DRIVE USING THE "REBUILD" OPTION ON THE RAID UTILITY

DISKETTE.

NOTE: DDD MEANS DEFUNCT, NOT DEFECTIVE.  A DEFUNCT DRIVE WILL

NOT RESPOND TO COMMANDS FROM THE RAID CONTROLLER.  THIS

COULD BE CAUSED BY FACTORS EXTERNAL TO THE FIXED DISK

ITSELF.