�����������������������������������������������������������������ͻ

� To print this file type PRINT C:\PCINFO\H13697.DOC from a DOS   �

� or OS/2 command prompt.                                         �

�����������������������������������������������������������������ͼ

THE FOLLOWING PROCEDURE WAS DEVELOPED TO ASSIST IN IMPLEMENTING

FIXED DISK REPLACEMENT UNDER 8641 ECA002, WHICH PROVIDES FOR

THE REPLACEMENT OF SPECIFIC 2.25 GB FIXED DISK DRIVES.  (REFER

TO RETAIN RECORD # H13646 FOR IMPORTANT ADDITIONAL INFORMATION.)

HOWEVER, THIS PROCESS MAY PROVE TO BE BENEFICIAL AT ANY TIME

MULTIPLE FIXED DISKS ARE BEING REPLACED IN A RAID ARRAY.

VERY IMPORTANT NOTES:

NOTE 1:

THE MOST IMPORTANT POINTS OF THIS PROCEDURE ARE:

A. KEEPING TRACK OF WHICH DRIVES ARE LOCATED IN WHICH BAYS AND,

B. TO HAVE A BACKUP COPY OF THE CUSTOMER'S RAID CONFIGURATION.

NOTE 2:

THE DRIVES MAY HAVE BEEN SWAPPED OUT QUITE A BIT.  IN THE EVENT

OF DRIVE FAILURE, INSPECT THE CONNECTORS FOR DAMAGE.

NOTE 3:

IF THE FIXED DISKS ARE BEING REPLACED DUE TO SUSPECTED STICTION

PROBLEMS, THE MAIN IDEA IS NOT TO POWER DOWN THE SERVER.  IF THE

IBM F/W STREAMING RAID ADAPTER OPTION DISKETTE IS AT VERSION 2.3

IT IS STRONGLY RECOMMENDED THAT VERSION 2.23. BE OBTAINED AND

USED DURING THIS PROCEDURE.  VERSION 2.3 TURNS OFF ALL THE FIXED

DISKS IN THE ARRAY DURING THE REBUILD PROCESS.  THIS WILL EXPOSE

THE REMAINING DRIVES TO POSSIBLE STICTION.  REINSTALL VERSION

2.3 AFTER ALL DRIVES HAVE BEEN REPLACED.

"REBOOT," IN THIS PROCEDURE REFERS TO SOFT BOOTS (CTRL-ALT-DEL).

1.  MAKE A CHART OF BAYS 1 THROUGH 6 AND WRITE IN EACH BAY THE

ORIGINAL DISK DRIVE SERIAL NUMBER AND THE NEW DISK

DRIVE SERIAL NUMBER FOR THAT PARTICULAR BAY.

2.  WHILE STILL INSTALLED IN THE SYSTEM, PLACE STICKERS ON THE

DRIVES INDICATING WHAT BAYS THEY ARE IN.  DO THE SAME TO THE

NEW DRIVES ONCE THEY ARE INSTALLED.

3.  SOFT BOOT THE IBM F/W STREAMING RAID ADAPTER OPTION DISKETTE

VERSION 2.23.  (DO NOT USE V2.3.  SEE IMPORTANT NOTE ABOVE).

SELECT "INITIALIZE/SYNCHRONIZE ARRAY."  SELECT SYNCHRONIZE

LOGICAL DRIVE.  PRESS THE SPACEBAR TO SELECT EACH LOGICAL

DRIVE.  ONCE ALL LOGICAL DRIVES ARE HIGHLIGHTED,  PRESS

ENTER TO BEGIN SYNCHRONIZATION.

4.  AFTER THE SYNCHRONIZATION COMPLETES,  RETURN TO THE MAIN

MENU.  SELECT "ADVANCED OPTIONS."  THEN, SELECT "BACKUP

CONFIGURATION TO DISKETTE" FROM THIS MENU.  INSERT A BLANK

DISKETTE INTO DRIVE A: AND PUT IN A FILENAME FOR THIS

BACKUP.  THE DEFAULT FILENAME IS "CONFIG."  IF YOU USE THE

SAME BACKUP DISKETTE FOR MORE THAN ONE SYSTEM, BE SURE TO

GIVE EACH SYSTEM A DIFFERENT FILENAME AND KEEP TRACK OF THE

SPECIFIC FILENAMES.

5.  EXIT OUT OF ADVANCED OPTIONS.  SELECT "VIEW CONFIGURATION"

FROM THE "MAIN MENU."  IDENTIFY WHICH BAY CONTAINS THE

HOT SPARE DRIVE, IF APPLICABLE.

6.  PULL THE HOT SPARE DRIVE OUT OF ITS BAY.  EXIT OUT OF "VIEW

CONFIGURATION" AND THE "MAIN MENU."

7.  REBOOT, PRESS Y TO UPDATE THE CONFIGURATION WITH THE HSP

BAY AS "DDD".

8.  YOU ARE NOW READY TO PULL OUT THE FIRST DRIVE TO BE REPLACED

THE FIRST DRIVE TO REPLACE SHOULD BE THE DRIVE SUSPECTED OF

HAVING THE HIGHEST PROBABILITY OF STICTION.

IF THERE IS NOT A SUSPECT DRIVE, THEN CHOOSE AN ORDER YOU

WOULD LIKE TO REPLACE THEM IN, SUCH AS BAYS 1, 2, 3, 4, ETC.

JUST REMEMBER WHAT ORDER THAT IS.  CHECKING OFF THE BAYS ON

YOUR CHART AFTER THEY HAVE COMPLETED IS VERY HELPFUL IN

KEEPING TRACK.

NOTE:

REMEMBER, THE HOT SPARE DRIVE IS PULLED OUT FIRST AND IS LEFT

OUT DURING REPLACEMENT/REBUILDING OF THE OTHER DRIVES.  IT

THEN BECOMES THE LAST DRIVE REPLACED AND ITS STATUS TURNS

TO HSP.  THE HOT SPARE DOESN'T REQUIRE REBUILDING (IT'S A SPARE)

IN ORDER YOU HAVE CHOSEN,  FOLLOW STEPS 8 AND 9 FOR EACH

DRIVE.

9.  PULL OUT DRIVE AND REBOOT THE RAID SUPPORT DISKETTE.  "Y"

TO UPDATE CONFIGURATION AS DDD.  REBOOT.  INSTALL NEW DRIVE

INTO THE BAY.  SELECT "REBUILD DRIVE" OFF THE "MAIN MENU."

THEN, SELECT "REPLACE DRIVE."  HIGHLIGHT CORRECT DDD DRIVE

IN THE BAY YOU ARE REPLACING.  THE SYSTEM WILL START THE

DRIVE.  AFTER STARTING SUCCESSFULLY, THE DRIVE STATUS WILL

CHANGE FROM DDD TO OFL.

10. SELECT "REBUILD DRIVE" ON THE REBUILD MENU OFF THE MAIN MENU

HIGHLIGHT CORRECT OFL DRIVE IN THE BAY YOU ARE REBUILDING.

SYSTEM WILL REBUILD THE DRIVE FROM 0 TO 100%.  AFTER IT HAS

COMPLETED SUCCESSFULLY, IT WILL CHANGE THE DRIVE FROM OFL

TO ONL.

IF SOFT ERRORS OCCUR, DO THE FOLLOWING:

1.  BEFORE REBOOTING, SELECT DRIVE INFORMATION TO DETERMINE

WHICH DRIVE HAD SOFT ERRORS.  TAKE NOTE OF THE DRIVE THAT

HAD SOFT ERRORS.

NOTE:  SOFT ERRORS INITIALIZE TO ZERO UPON REBOOT.

2.  THE QUANTUM DRIVE THAT FAILED TO REBUILD WILL DISPLAY OFL

STATUS REPLACE THIS QUANTUM WITH THE STARFIRE DRIVE THAT WAS

ORIGINALLY IN THIS BAY.

3.  RESTORE THE CONFIGURATION THAT WAS BACKED UP IN STEP 4 BY

SELECTING ADVANCED FUNCTIONS ON MAIN MENU.  THEN, CHOOSE

RESTORE CONFIGURATION FROM DISKETTE.  ENTER THE FILENAME OF

THE SAVED CONFIGURATION FOR THIS SERVER.

4.  ONCE THE RAID CONFIGURATION HAS COMPLETED BEING RESTORED,

ALL DRIVES SHOULD INDICATE ONL.  AT THIS TIME, REMOVE THE

DRIVE THAT WAS NOTED AS CAUSING SOFT ERRORS.

5.  REBOOT TO RAID UTILITY.  SAY "YES" TO UPDATE THE RAID

CONFIGURATION WITH THE EMPTY BAY AS DDD. WHEN THE UPDATE

COMPLETES, THE RAID UTILTY WILL PROMPT YOU TO REBOOT THE

SYSTEM TO THE RAID UTILITY.

6.  INSTALL THE QUANTUM IN THE DDD BAY AND REBUILD IT.  WHEN

THIS DRIVE COMPLETES REBUILDING, YOUR SOFT ERROR DRIVE WILL

BE REPLACED. YOU ARE NOW PAST THE SOFT ERRORS AND CAN BEGIN

REPLACING THE REST FOLLOWING THE PROCEDURES.

IF FATAL ERROR OCCURS DO THE FOLLOWING:

REPLACE ONLY THE ORIGINAL DRIVE BACK INTO THE SYSTEM THAT HAS

NOT BEEN REBUILT SUCCESSFULLY YET.  THEN, RESTORE THE

CONFIGURATION FROM DISKETTE BY REBOOTING TO OPTION DISKETTE.

SELECT "RESTORE CONFIGURATION" FROM DISKETTE ON THE ADVANCED

OPTIONS MENU OFF THE  MAIN MENU.  PUT THE DISKETTE WITH THE

BACKUP CONFIGURATION INTO THE DISKETTE DRIVE AND TYPE IN THE

FILENAME FOR THAT PARTICULAR SYSTEM.  THEN, PROCEED REPLACING

AND REBUILDING DRIVES WHEREVER YOU LEFT OFF.

IF DURING THE RESTORING FROM DISKETTE AN, "ADAPTER IS NOT

RESPONDING TO CONTROLLER COMMANDS" MESSAGE APPEARS, DO THE

FOLLOWING:

REBOOT AND RESTORE THE CONFIGURATION FROM THE BACKUP RAID

SUPPORT DISKETTE AGAIN UNTIL IT COMES BACK AND SAYS RESTORATION

COMPLETED SUCCESSFULLY.

11.  ONCE ALL DRIVES ARE REPLACED AND REBUILT SUCCESSFULLY,

POWER OFF THE SERVER AND COLD BOOT THE SYSTEM.  THE SYSTEM

SHOULD NOW BE READY TO RETURN TO THE CUSTOMER.