HOW TO CONFIGURE THE AUTO ISA 16/4 TOKEN RING ADAPTER FROM THE

DOS COMMAND LINE.

1. PLACE THE LANAID DISKETTE V1.40 IN DRIVE A: AND BOOT.

2. AT THE FIRST MENU (CONFIG.SYS MENU) PRESS <F5> TO LOAD DOS

BYPASSING THE AUTOEXEC.BAT AND CONFIG.SYS. (DIRECT TO DOS

PROMPT)

3. EXECUTE THE LANAIDC PROGRAM WITH ONE OR MORE OF THE FOLLOWING

OPTIONS LISTED BELOW:

USAGE: LANAIDC  <OPTIONS>...  .40

/FAST=NW - NETWARE FASTPATH, ALL OPTIONS AUTOMATICALLY CHOSEN

/FAST=LSP - LSP FASTPATH, ALL OPTIONS AUTOMATICALLY CHOSEN

/FAST=CFGONLY - ALLOW ALL OPTIONS TO BE AUTOMATICALLY CHOSEN, CONFIGURE ONLY

/PNP=Y OR /PNP=N - PLUG AND PLAY CONFIGURATION / MANUAL CONFIGURATION

/LOCK=Y OR /LOCK=N - LOCK THE ADAPTER CONFIGURATION

/IO=<###> - SPECIFY HEX IO ADDRESS (A20 OR A24)

/INT=<##> - SPECIFY INTERRUPT LEVEL (3, 9, 10 OR 11)

/ROM=<#####> - SPECIFY HEX ROM ADDRESS (C8000-DE000)

/RAM=<#####> - SPECIFY HEX RAM ADDRESS (C0000-DE000)

/RAMSIZE=<##> - SPECIFY RAM SIZE IN KBYTES (8, 16, 32, OR 64)

/BUSW=<##> - SPECIFY BUS WIDTH (8 OR 16)

/RIPL=Y OR /RIPL=N - ENABLE/DISABLE REMOTE IPL

/RS=<##> - SPECIFY RING SPEED (4 OR 16)

/AUTO=Y OR /AUTO=N - ENABLE/DISABLE AUTO SENSE

/VIEW - VIEW THE CURRENT CONFIGURATION

/LSP - INSTALL LSP

/NWDIR=<DRIVE:\DIR> - INSTALL NETWARE CLIENT IN THIS DIRECTORY

/NOUPDATE - DO NOT ALLOW CONFIG.SYS AND AUTOEXEC.BAT TO BE UPDATED

/WINDIR=<DRIVE:\DIR> - WINDOWS DIRECTORY FOR WINDOWS SUPPORT

/FTMSB OR /FTLSB - SPECIFY FRAME TYPE = TOKEN-RING MSB OR LSB

/FTSNAPMSB OR /FTSNAPLSB - SPECIFY FRAME TYPE = TOKEN-RING SNAP MSB OR LSB

/MAXFRAME=<##> - SPECIFY MAXIMUM FRAME SIZE

/NODEADDR=<############X> - SPECIFY HEX NODE ADDRESS + OPTIONAL FORMAT (L,M)

4. EXECUTE LANAIDC /V TO VERIFY THAT THE ADAPTER SET CORRECTLY.

NOTE: WITH LANAID DISKETTE V1.4 A REBOOT AFTER CONFIGURATION IS NOT

NECESSARY