TITLE: PS/2 PASSWORD RESET PROCEDURES

OVERRIDE JUMPERS ON IML SYSTEMS               �         POWER-ON PASSWORD

�          OVERRIDE JUMPER

- THE SERVER 95A HAS NO SYSTEM PARTITION. IT   �   SYSTEM          �  JUMPER

LOADS THE IML & POST FROM EEPROM ON THE      �                   � LOCATION

SYSTEM OR PROCESSOR BOARD. IF A POWER-ON     � ������������������������������

PASSWORD IS SET AND THE REFERENCE DISKETTE   �  8525 SX          �  J2

IS IN THE DISKETTE DRIVE, MOVE THE PASSWORD  � ������������������������������

OVERRIDE JUMPER, THEN POWER ON THE COMPUTER. �  8525 & 8530 (286)�  J13

THE SYSTEM DEFAULT IS BYPASSED & THE SYSTEM  � ������������������������������

LOADS THE IML IMAGE FROM THE REFERENCE       �  8535, 8540       �  J14

DISKETTE. THE NEXT TIME YOU POWER ON THE     � ������������������������������

COMPUTER, THE IML & POST LOAD FROM EEPROM.   �  8556, 8557       �  J18

� ������������������������������

- THE SERVER 85 & SERVER 95 LOAD THE IML FROM  �  9533, 9535, 9540 �  J20

THE EEPROM, BUT THE POST IS IN THE SYSTEM    � ������������������������������

PARTITION. IF A POWER-ON PASSWORD IS SET &   �  9556, 9557, 9576 �  JMP1

THE REFERENCE DISKETTE IS IN THE DISKETTE    �  9577, 9595       �

DRIVE, MOVE THE PASSWORD OVERRIDE JUMPER,    � ������������������������������

THEN POWER ON THE COMPUTER.  THE SYSTEM      �  XX90             �  J10

DEFAULT IS BYPASSED AND THE SYSTEM LOADS THE � ������������������������������

IML IMAGE FROM THE REFERENCE DISKETTE.       �  SERVER 85        �  J16

�  SERVER 95        �

- ALL OTHER IML SYSTEMS LOAD THE IML & POST

FROM THE SYSTEM PARTITION.  IF A POWER-ON

PASSWORD IS SET & THE REFERENCE DISKETTE IS

IN THE DISKETTE DRIVE, MOVE THE PASSWORD

OVERRIDE JUMPER, THEN POWER ON.  THE SYSTEM

DEFAULT IS BYPASSED AND THE SYSTEM LOADS THE

IML IMAGE FROM THE REFERENCE DISKETTE. THE

NEXT TIME YOU POWER ON THE COMPUTER, THE IML

LOADS FROM THE SYSTEM PARTITION.

PRIVILEGED-ACCESS PASSWORD

NOTE: THE PRIVILEGED-ACCESS PASSWORD IS ALWAYS JMP2

THIS IS USED WITH OTHER SECURITY FEATURES TO MAKE THE SYSTEM INOPERATIVE AFTER

FORCED ENTRY, UNTIL THE PASSWORD IS TYPED.  IT ALSO OVERRIDES THE POWER-ON

PASSWORD, IF INSTALLED, AND RESTRICTS ACCESS TO THE SYSTEM PROGRAMS.

YOU CANNOT ERASE OR OVERRIDE THE PRIVILEGED-ACCESS PASSWORD.  THE USER OR

TECHNICAL SUPPORT PERSON AT THE ACCOUNT MUST HAVE THIS PASSWORD.  IF NOT, THE

ONLY WAY TO GAIN ACCESS TO THE SYSTEM PROGRAMS IS TO REPLACE THE SYSTEM BOARD

AT THE CUSTOMER'S EXPENSE.

THE SYSTEM IS SHIPPED WITH THE PRIVILEGED-ACCESS PASSWORD FEATURE DISABLED (NO

PASSWORD IS SET).  A JUMPER (JMP2) ON THE SYSTEM BOARD MUST BE MOVED BEFORE A

PRIVILEGED-ACCESS PASSWORD CAN BE SET FOR THE FIRST TIME. THE JUMPER CAN EITHER

BE IN THE LOCKED STATE OR THE CHANGE STATE.

THE LOCKED STATE PREVENTS A PRIVILEGED-ACCESS PASSWORD FROM BEING SET, CHANGED,

OR REMOVED.  THIS IS THE FACTORY-SET POSITION.  THE JUMPER IS IN POSITION 1

WHEN IT IS THE LOCKED STATE.  THE CHANGE STATE IS POSITION 0.

THE RULES FOR SETTING THE PRIVILEGED-ACCESS PASSWORD ARE THE SAME AS THE POWER-

ON PASSWORD (7 CHARACTERS, A-Z, A-Z, 0-9).  ONE IMPORTANT DIFFERENCE IS THAT A

PRIVILEGED-ACCESS PASSWORD CANNOT BE THE SAME AS THE POWER-ON PASSWORD.

~~~~~~

TITLE: PC SERVER 300 (8640) PENTIUM MODELS

THERE IS NO REFERENCE IN THE SERVICE OR USER DOCUMENTATION OF

THE SERVER 300 FOR SETTING OR REMOVING THE POWER-ON PASSWORD.

THERE IS NO JUMPER ON THE SYSTEM BOARD WHICH WILL CLEAR A

POWER-ON PASSWORD.

TO REMOVE A LOST POWER-ON PASSWORD, CMOS MEMORY ON THE SYSTEM

BOARD MUST BE CLEARED.  THE FOLLOWING PROCEDURE WILL CLEAR

THE POWER-ON PASSWORD, BUT WILL ALSO REQUIRE THE SYSTEM TO

BE TOTALLY RECONFIGURED.

A. REMOVE THE SRAM (STATIC RANDOM ACCESS MEMORY - CMOS) MODULE

FROM THE SOCKET AT U50, LOCATED JUST ABOVE THE IDE AND

DISKETTE DRIVE CONNECTORS (REFER TO PAGE 51 OF THE "USER'S

HANDBOOK" FOR THE SYSTEM BOARD CONNECTOR LOCATIONS.)

B. WAIT FOR 5 MINUTES (TO ALLOW THE MODULE DRAIN OR DISCHARGE),

C. RE-INSTALL THE SRAM MODULE IN ITS SOCKET, BEING CAREFUL

NOT TO BEND THE PINS.  ALSO BE CERTAIN THE MODULE IS PROPERLY

ORIENTED, OTHERWISE IT COULD BE DAMAGED.  SEE DIAGRAM BELOW:

D. THEN, RUN THE SETUP AGAIN.

���������Ŀ

� CLOCK   �

� MODULE  �  <---DO NOT REMOVE THIS MODULE

�����������

�����������Ŀ

�SRAM MODULE�

NOTE THE DOT--->�O          �

IN THIS CORNER  �������������

����������������Ŀ  ����������������Ŀ

� DISKETTE CONN. �  �IDE CONNECTOR   �

������������������  ������������������

CAUTION: REFER TO PAGE 16 OF THE USER'S HANDBOOK, SHIPPED

WITH EACH SYSTEM, AND OBSERVE THE PRECAUTIONS FOR HANDLING

STATIC SENSITIVE DEVICES WHILE HANDLING THE SYSTEM BOARD

AND OTHER COMPONENTS OF THE 8640.