Area Os2, Msg#1586, Sep-09-92 09:52PM

From: Wilson Navarro

To: All

Subject: 16550 under os2

I found this message on internet. I hope it helps someone understand com

ports under OS/2. ------------------------------------------------------------

------------ Article 11617 (102 more) in comp.os.os2.misc: From: db3l@ans.net

(David Bolen)

Subject: COM performance hints with 16550 Date: Thu, 3 Sep 1992 20:18:53 GMT

Organization: Advanced Network & Services, Inc. - Elmsford, NY Lines: 325

Warning: This is pretty long, but there's a summary at the end.  If you just

want to see a brief statement now:

If you have a 16550, have a serial device that keeps DSR high while

it is on (I think most modems do this by default, or are configurable),

doesn't do hardware flow control, or doesn't mind getting up to 15

characters after dropping CTS and/or DSR, make sure you set BUFFER=ON

when configuring your COM port with the MODE command.  With default

settings, BUFFER=AUTO is really no different than BUFFER=DISABLED.

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

I recently obtained a copy of the OS/2 physical device driver reference and

was reading through the information about the async drivers (COM/VCOM.SYS).

From the number of posts on the subject (some from myself), it seems that a

number of people are confused about what the various MODE COM settings mean,

and what sort of setup was the most efficient use of COM ports when a 16550

UART was present (which seems to be universally regarded as a good idea to

have in order to gain good performance).  I normally use OS/2 COM programs

with the RTS=HS,BUFFER=AUTO settings (as is normally recommended on the net

and in the FAQ) and wondered if I was fully exploiting the capabilities of my

16550.  In addition, I was still having problems with DOS environment

programs (notably kermit 3.11 that just dies a horrible death) and hoped to

learn a bit about how that might be improved.

After reading a bit, and performing some experimentation, I've come up with

the following information and conclusions that may help other people with the

16550s.  I don't place any guarantees on this information, other than that it

is accurate to the best of my knowledge and seems to be backed up by my

experimental results.  I hope it proves helpful, or at least helps to explain

the relationship between some of the MODE parameters and the use of the FIFO

buffers provided by the 16550.

(in the following, when I use the term 16550, the OS/2 drivers actually

require an NS-16550A or compatible chip, and I believe the current level

chips as discussed previously in the OS/2 groups are 16550AFN or 16550AF)

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

Introduction:

------------

The 16550 provides for a 16-character receive hardware buffer that can be set

to generate an interrupt every 1, 4, 8 or 14 characters.  It can also hold up

to 16 characters in a transmit buffer, generating an interrupt only when that

buffer is empty.  In other words, in its most efficient mode, the 16550 can

handle transmission and receipt of data in roughly 16-byte chunks, only

interrupting the CPU at the end of each chunk.

Note that even if you don't make use of the extended buffering from the point

of view of interrupt handling, but still receive an interrupt for every

character received or transmitted, the 16550 buffer can be helpful, as it can

buffer received characters if the COM handler misses an interrupt, thereby

preventing receive overruns.

The OS/2 COM driver can fully exploit these capabilities of the 16550

depending on how it is configured.  Here's the rub though - if you take the

default COM settings, and even if you use MODE to set the BUFFER parameter to

AUTO, the COM driver is only partially using the capabilities of the UART. In

other words, the settings that are normally posted to these groups, and are

in the FAQ are not the most efficient settings.  In fact, from what I can

tell, a BUFFER value of AUTO is no better than DISABLED with the other

defaults for the modem signaling values.

COM Driver Settings (MODE):

--------------------------

The extended hardware buffering mode of the async driver can be controlled

via the BUFFER parameter of the MODE command.  It can take on three values:

ON         Set transmit/receive triggers to fully exploit buffering.

(16-character transmit/receive queues for 16550).

OFF        Set transmit/receive triggers to a single character.

AUTO       "Automatic Protocol Override".  Adjust the transmit/receive

triggers according to handshaking protocols in effect.

The "handshaking protocols" for AUTO mode use the following signals:

RTS = Request To Send           ( Signals FROM OS/2 COM Driver )

DTR = Data Terminal Ready

CTS = Clear To Send             ( Signals TO OS/2 COM Driver )

DCD = Data Carrier Detect

DSR = Data Set Ready

and refer to the following:

* Input Sensitivity using DSR

- MODE option IDSR, default ON

- If this setting is ON, then the driver will only accept data from

the device while the DSR line is active.

* Output Handshaking using CTS, DSR, DCD

- MODE options OCTS, ODSR, default is both ON

- No MODE option for DCD - MODE always sets it OFF

- This setting controls whether CTS, DSR or DCD are used to control

the flow of data to the modem.  If any of these settings are ON,

the driver stops giving data to the transmit hardware as soon as

the corresponding control signal drops.

* RTS Control Mode

- MODE option RTS, default is ON.  Can be one of the following:

+ Enabled                                              [MODE=ON]

+ Disabled                                             [MODE=OFF]

+ Input Handshaking (RTS drops on input queue full)    [MODE=HS]

+ Toggling on Transmit (RTS drops unless transmitting) [MODE=TOG]

- This setting controls how the RTS signal is controlled by the

driver.  If ON, DTR is used to signal when the COM port is

active, while if HS, DTR is used to control the flow of data

from the modem.

* DTR Control Mode

- MODE option DTR, default is ON.  Can be one of the following:

+ Enabled                                              [MODE=ON]

+ Disabled                                             [MODE=OFF]

+ Input Handshaking (DTR drops on input queue full)    [MODE=HS]

- This setting controls how the DTR signal is used to control

the flow of data.

- This setting controls how the RTS signal is controlled by the

driver.  If ON, DTR is used to signal when the COM port is

active, while if HS, DTR is used to control the flow of data

from the modem.

A typical default configuration for the COM port is (ignoring some of the

MODE parameters not relevant to this post):

IDSR = ON               ODSR   = ON

OCTS = ON               DTR    = ON

RTS  = ON               BUFFER = AUTO

And most suggestions for configuring the COM port for better control

explicitly set RTS=HS, and BUFFER=AUTO, so I expect that many people are

running with the above settings, except with RTS=HS, allowing full hardware

flow control (RTS/CTS) to the modem.

Setting Interaction:

-------------------

For the purposes of this post (getting the most performance out of the

16550), the only settings that are really important from the above discussion

are the "Extended Hardware Buffering" (BUFFER), "Input Sensitivity using DSR"

(IDSR), and "Output Handshaking using CTS, DSR, DCD" (OCTS,ODSR).

Here's the basic problem.  In order to accomplish the latter two settings in

their default modes, the AUTO buffering mode basically stops using the FIFO

buffers.  In particular, the following adjustments are "automatically" made:

* If IDSR is enabled, then the COM driver is set up to respond

to a lowering of the DSR signal within one character time.  To ensure

this, the driver adjusts the receive trigger to be a single character.

In other words, it lets the 16550 generate an interrupt per character.

The full 16-character receive buffer is still available to prevent

receive overruns, and the transmit trigger is not adjusted.

* If either OCTS, ODSR (or ODCD - only changeable from a program) are

enabled, then the device driver will respond to a lowering of the

appropriate signal within a single character time.  To do this, the

transmit trigger is lowered to a single character - the 16550 is set

to generate a transmit interrupt for each character, although the

receive trigger is not adjusted.

The reason both of these automatic adjustments are made is to take the safe

course, and assume that anything using those signals has been designed to

assume that the signals take action within a single character time.  For

example, without fixing the transmit trigger, the modem could lower the CTS

signal, but the 16550 might continue transmitting up to 15 characters that

might still be in the FIFO transmit queue.

Of course, the problem is that with the default settings, both of these rules

come into effect, and the driver ends up getting an interrupt per character

for both transmitting and receiving characters.  In other words, you end up

using the 16550 just like any lower UART, with the single advantage that the

receive FIFO buffer on the chip helps avoid receive overruns.

Improving Settings

------------------

In order to make more effective use of the 16550, you can do one of three

things:

* Explicitly set the extended buffering on (MODE BUFFER=ON).

* Disable IDSR sensitivity (MODE IDSR=OFF)

* Disable any output flow control (MODE OCTS=OFF,ODSR=OFF)

Of the three, I generally prefer the former, but you may have reasons to do

one of the latter two.  By explicitly setting the buffering on, you will

override the rules above that adjust the receive and transmit triggers.  What

this means is that you may receive extraneous data if DSR is actually being

used to signal input and that if the modem asks the driver to stop sending

data (CTS), it may still receive up to 15 more characters.

I don't think either of these problems should hit most people using modems

today.  I believe most modems hold DSR high while they are on (or at least

are configurable to do this), and most modems that handle hardware flow

control (RTS/CTS) drop CTS before they are absolutely out of room.  For

example, the USR Courier v.32bis modem I have has a transmit buffer of 1.5K

(3.25K for error control connections) and drops CTS when the buffer is 90%

full (plenty of room for 15 extra characters).  But of course, for a specific

case the second or third choice can be made, which will make better use of

the 16550 than the default settings, but not the best.

Some experiments

----------------

Just to see if any of the stuff I was learning seemed to apply, I ran some

quick tests.  Nothing extremely scientific - just a couple of quick file

transfers.

Using a SLIP connection (TCP/IP over a serial line), with USR v.32bis modems

on either end, I transferred a reasonably sized (115K or so) file down from a

workstation.  My COM port was set at 38.4K.  The transfer was from the

workstation back to my machine so I was mostly testing the receive

characteristics although the program used (FTP) does transmit data during the

transfer.

I tested using both the normal COM port settings (plus RTS=HS), and then with

BUFFER=ON rather than AUTO.  Also, I first ran the transfers without anything

else occurring on the system, and then re-ran the same transfers with a PM

monitor (one of the ones from ftp-os2 - pmload11.zoo).  The results were

interesting:

FTP Transfer Time (binary mode, size = 114688 bytes)

MONITOR      BUFFER          Time (s)        Rate (cps)      Load (%)

---------------------------------------------------------------------

No           Auto            41              2797            N/A

No           On              41              2797            N/A

Yes          Auto            53              2164            98-100

Yes          On              41              2797            82-86

When nothing else was running, BUFFER didn't really make much of a

difference. I guess this just meant that my machine itself was fast enough to

keep up with an interrupt a character in general.  I presume though (given

the next set of results) that the load on the system in the On mode was much

less.

When something was running (even the load program, as a PM program, takes up

CPU time and interferes with running applications - I had it iconized, so it

had to update the icon each second), things were very different.  In Auto

mode, I got a much worse transfer time, and my system load was pretty much

pegged at 100%.  In On mode, I got as good a transfer time as when nothing

else was running, but at a much smaller load on the machine.

So I think this is pretty strong evidence that using BUFFER=ON is very

different than BUFFER=AUTO when the other signaling settings are using their

default values.

Virtual COM Driver (DOS Boxes)

------------------------------

After reading about VCOM (of which little is written), I don't see a magic

pill for DOS box users, other than that a BUFFER=ON setting should help lower

the CPU load and make it easier to keep the virtual interrupt rate up in the

DOS environment.

There was one discovery I did make however, that may help some people out.

VCOM supports two modes of accessing the COM ports - either through BIOS

calls or via virtual I/O ports.  The virtual I/O ports only emulate the lower

end INS8250 UART, and as others have noted, the virtual interrupt rate is

limited to around 1000Hz or so.

Since I figured it had something to do with the virtual I/O ports that was

causing the most problem with kermit, I reset kermit to use BIOS1 as its

method for accessing the COM port.  Although access through the BIOS is

technically limited to 9600 baud maximum, I was able to run kermit without a

failure, including file downloads.  When letting kermit use the virtual I/O

port, I'd generate traps at rates even as low as 2400 baud.

I think this behavior arises from the fact that when the application is

controlling the hardware directly, it is issuing a lot more I/O requests to

the virtual I/O port that must be handled by VCOM.  With the BIOS interface,

the application basically asks BIOS for a character, and gets them back one

at a time.  Since VCOM goes off to COM to get the character, all of the

hardware flow control and buffering used by COM is available to VCOM, and the

DOS application doesn't have to concern itself with anything.

So - if someone else is trying to run a COM program in a DOS box and it can

use BIOS to access the COM port rather than trying to run the hardware itself

you may be able to get it working reasonably.  At the very least, it gave me

a way to test the DOS version of kermit some of my users use from home at a

reasonable speed (although not as high as I would like) from within a DOS box.

Summary

-------

An important aspect of the support for 16550 UARTs within the OS/2 2.0 COM

driver is the interaction between various modem control handshaking settings

and the automatic control of extended buffering.  In particular, several of

the default COM port settings (IDSR=ON,OCTS=ON,ODSR=ON) when automatic

control (BUFFER=AUTO) is enabled force the driver not to use the enhanced

buffering of the 16550, and instead generate interrupts for each character

transmitted or received.  This can decrease performance and definitely places

a heavier load on the machine.

The easiest way to solve this is to use a setting of BUFFER=ON rather than

BUFFER=AUTO, as long as it won't cause problems overrunning your modem or

receiving extraneous information (see "Improving Settings" above).  I don't

think many users will encounter a problem if they make these changes.  From

my experiments, switching to BUFFER=ON can decrease the load on a 16MHz 386

by at least 15%, while allowing the same transfer rate on a busier system.

For the DOS Box, one hint to users trying to get reasonable performance out

of DOS COM applications - BUFFER=ON may also help this situation, but if you

can live with 9600 baud, and your application supports using the BIOS

interface to the COM port, try using that mode.  You may be to run the

application successfully at higher data rates through the emulated BIOS

interface than if the application tries to use the emulated COM hardware

directly.

I hope someone else finds this information useful - I know it's helped me to

get a better handle on how OS/2 manages the COM port, and how various

settings affect its use of the capabilities of my 16550 UARTs.

--

-- David

--

/-----------------------------------------------------------------------\

\              David Bolen             \  Internet: db3l@ans.net      /

|   Advanced Network & Services, Inc.   \   Phone: (914) 789-5327   |

/ 100 Clearbrook Road, Elmsford, NY 10523  \   Fax: (914) 789-5310    \ \----

-------------------------------------------------------------------/