RPLADD.EXE  LAN Server 4.0 Productivity Aids Program

This program, RPLADD.EXE, can be used to create LAN Server Version 4.0

or above, remote IPL client definitions from an OS/2 command line.

The files included in this package are:

RPLADD.EXE - Executable program

RPLADD.FIL - Sample input file

RPLADD.DOC - This README document

RPLADD.ICO - Icon file

USING RPLADD

============

RPLADD can be used to create remote IPL definitions from any OS/2 workstation

within the same domain.  The local workstation and the target remote IPL

server must both be running LAN Server Version 4.0 or above.  The local

workstation must be logged on with administrator level privilege.  The target

remote IPL server must have the Server service started, but does not require

that the Remoteboot service be started.

COMMAND LINE PARAMETERS

=======================

REQUIRED PARAMETERS

/F:input_filename

This required parameter specifies the path and file name of the source

input file containing the formatted records that will be used as

input to the RPLADD.EXE program.  See the INPUT FILE FORMAT section

of this document for an explanation of the input file format.

OPTIONAL PARAMETERS

/O:output_log_filename

This optional parameter specifies the path and file name of an ouput

log file that will be used to record all output from the RPLADD.EXE

program.  If this parameter is specified, all output that would normally

be displayed on the display, will be redirected to a file.

/Debug

This optional parameter specifies to display additional debug information

about the execution of the RPLADD.EXE program.  This parameter can be

useful for displaying information about the parameters that are parsed

from the input file.

INPUT FILE FORMAT

=================

RPLADD.EXE requires an input file containing formatted records.  A sample

input file, RPLADD.FIL, is supplied with this program.  Below is a brief

description of the input file format.

PARAMETER 1 = CREATE TYPE

CREATE TYPE = 2 is specified to create a new machine definition

and will inherit the default machine configuration.

CREATE TYPE = 12 is specified to create a new machine from an existing

model machine definition.  The new machine will inherit all of the

configuration files from the model machine definition.  The new

machine and the model machine must exist on the same server.

PARAMETER 2 = NAME

The name of the new remote IPL machine definition.  The machine name

must be a unique name within the Domain.  DOS remote IPL machine

names cannot exceed 8 characters in length.  Machine names defined

on a server with a FAT file system cannot exceed 8 characters in

length.

PARAMETER 3 = ADAPTER ADDRESS

The ADAPTER ADDRESS is the 12 character universal adapter address of

the remote IPL machine.

PARAMETER 4 = SERVER RECORD IDENTIFIER

The SERVER RECORD IDENTIFIER specifies the operating system plus

network adapter type configuration information.

The SERVER RECORD IDENTIFIER specified must exist as an enabled

server record identifier on the remote IPL server.

OS/2 PARAMETER 5 = OS/2 DRIVE (OS/2 PARAMETER ONLY)

The OS/2 DRIVE specifies the boot drive letter for the OS/2 remote IPL

machine.

NOTE:  USE THIS PARAMETER FOR OS/2 MACHINE DEFINITIONS ONLY!

OS/2 PARAMETER 6 = OS/2 CONFIG (OS/2 PARAMETER ONLY)

The OS/2 CONFIG parameter is used to specify OS/2 configuration

parameters for the OS/2 remote IPL machine.  A combination of 3

flags(4 flags for DBCS countries) are to be specified, one from

each of the catagories.  There should be no spaces between the flags.

SWAPPER.DAT LOCATION

L = SWAPPER.DAT file located on remote IPL machine hard disk

S = SWAPPER.DAT file located on server hard disk

BUS ARCHITECTURE

I = ISA bus architecture

M = Microchannel bus architecture

DISPLAY TYPE

3 = S-3 Super VGA display

8 = 8514 display

E = EGA display

U = Super VGA display(non S-3)

V = VGA display

X = XGA display

N = PS/55 16 color display(DBCS countries)

O = PS/55 256 color display(DBCS countries)

P = PS/55 256P color display(DBCS countries)

KEYBOARD TYPE (DBCS COUNTRIES ONLY)

A = DBCS AX keyboard

B = DBCS 101 keyboard

C = DBCS 103 keyboard

D = DBCS 106 keyboard

NOTE:  USE THIS PARAMETER FOR OS/2 MACHINE DEFINITIONS ONLY!

DOS PARAMETER 5 = DOS IMAGE (DOS PARAMETER ONLY)

The DOS IMAGE specifies the DOS image file for the remote IPL machine.

The DOS image file must exist on the target server before the DOS

remote IPL machine can be defined.  Do not specify a file name extention

for the DOS IMAGE parameter.

NOTE:  USE THIS PARAMETER FOR DOS MACHINE DEFINITIONS ONLY!

FINAL PARAMETER = SERVERNAME (Optional)

The SERVERNAME is the name of the target server where the remote IPL

machine will be created.  The server name specified must exist within

the same domain as the machine where RPLADD is being run.

The SERVERNAME parameter is optional if the remote IPL machines are

to be created on the same machine where RPLADD is being run.

Below are sample input lines...

;CREATE  NAME           ADAPTER       SERVER RECORD  OS/2   OS/2    DOS       [SERVERNAME]

;TYPE                   ADDRESS       IDENTIFIER     DRIVE  CONFIG  IMAGE     (Optional)

;------  -------------- ------------  -------------  -----  ------  --------  ------------

2        OS2_MACH       10005A111111  R_230_OTK      Z      LI3

2        DOS_MACH       10005A222222  R_DTK                         STD3HFUL

2        OS2JAPAN       10005A333333  R_21_OTK       J      SMND              SRV_JAPAN

;CREATE  NAME           ADAPTER       MODEL         [SERVERNAME]

;TYPE                   ADDRESS       NAME          (Optional)

;------  -------------  ------------  ------------  ------------

12       NEW_MACH       10005A444444  MODEL_MACH    RIPL_SERV

The results expected from each line above are...

A remote IPL definition for OS2_MACH will be created on the local server.

OS2_MACH will be defined as an OS/2 remote IPL machine and will be created

from the R_230_OTK standard configuration files.  OS2_MACH will boot from

drive letter "Z:" and will be configured for a local SWAPPER.DAT file,

ISA bus architecture and an S3 SVGA display.  Note that the remote IPL

server must be preconfigured to support SVGA display types.

A remote IPL definition for DOS_MACH will be created on the local server.

DOS_MACH will be defined as a DOS remote IPL machine and will be created

from the R_DTK standard configuration files.  DOS_MACH will boot using

the STD3HFUL DOS image file.

A remote IPL definition for OS2JAPAN will be created on SVR_JAPAN server.

OS2JAPAN will be defined as an OS/2 remote IPL machine and will be created

from the R_21_OTK standard configuration files.  OS2JAPAN will boot from

drive letter "J:" and will be configured for a server SWAPPER.DAT file,

microchannel bus architecture, PS/55 16 color display and DBCS 106 keyboard.

A remote IPL definition for NEW_MACH will be created on RIPL_SERV server.

NEW_MACH will inherit all the configuration information of MODEL_MACH.

NEW_MACH and MODEL_MACH will exist on the same remote IPL server.

End of RPLADD.DOC.