## PATHWORKS Client License Requestor Configuration Instructions

This document will provide information on how to configure PATHWORKS License Requesters.

The license server is assumed to be either a PATHWORKS for OpenVMS (LAN Manager) v5.0F or PATHWORKS for OpenVMS (Advanced Server) v6.0 server and has Client Access licenses (PWLMXXXCA06.00) available.

Client systems for which instructions are given include DOS, Windows, Windows for Workgroups, Windows 95, Windows NT v3.51 and 4.0, and OS/2.

In all cases, a client requires a Client Access license to access a PATHWORKS for OpenVMS server. This license is referred to in this document as PWLMXXXCA06.00.

If you are also using PATHWORKS client software (DECnet, LAT, etc) on your PC then it will also require an appropriate license. In this document, this is referred to as PWXXWINAT07.00.

For more information about PATHWORKS products and licenses, please visit our PATHWORKS InfoCenter on the Web at:

http://www.digital.com/info/PATHWORKS

Thank you for choosing DIGITAL as your information systems provider.

DOS and Windows 3.x

When using the real-mode license requester software available in PATHWORKS for DOS and Windows for the DOS or Windows 3.x environments, configure these clients to select a specified license using a license configuration file:

1)	Create a template as usual, using PWSETUP.

1)	Create a text file (usually called licfile.txt) with details about the licenses to be requested, as described in the PATHWORKS Guide to Managing Licenses. It should contain lines similar to this:

LICENSE00 = DECPWLMXXXCA06.00

LICENSE01 = DECPWXXWINAT07.00

1)	If you are configuring a DOS system, modify the {SW_PWLICLM} keyword, adding the /FILE:filename parameter, referring to the file created in step 2:

- Use EDIT or another text editor to open the template file, usually named CFGxxxx.TPL

- Move to the [Keywords] section and locate the {SW_PWLICLM} line.  If this line does not exist in your template, add it anywhere in the [Keywords] section.

- Add the /NEEDED (or /FILE) parameter to that line and save the file.

1)	If you are configuring a Windows for Workgroups system with the real-mode license requester (using WINSTART.BAT), modify WINSTART.BAT by adding the /FILE:filename parameter to the PWLICLM command, referring to the file created in step 2:

- Use EDIT or another text editor to open the WINSTART.BAT file.

- Locate the PWLICLM line.

- Add the /NEEDED (or /FILE) parameter to that line and save the file.

When using the protected-mode license requester software available in PATHWORKS for DOS and Windows for a Windows for Workgroups environment, configure these clients by modifying the [DECLICL] section of the \WINDOWS\SYSTEM.INI file. This section contains lines similar to this:

LICENSE00 = DECPWLMXXXFP05.00

LICENSE01 = DECPWXXWINAT07.00

- Use a text editor to open the SYSTEM.INI file.

- Locate the [DECLICL] section, if it exists.

- NOTE: If the [DECLICL] section does not exist, add it to the end of the file.

- In the [DECLICL] section, locate the LICENSE00 line.

- NOTE: If the LICENSExx lines do not exist, add them to this section.

- Modify the line as indicated and save the file.

Windows 95 Clients

Windows 95 systems can be configured to select a PATHWORKS Client Access license as follows:

1)	From the Start menu, select Settings and then Control Panel.

2)	In the Control Panel window, double-click on the Network icon.

3)	In the Network control panel, select "DEC LAN Manager Licensing Agent" and click on the Properties button (some versions will call this "PATHWORKS Licensing Agent (LAN Manager)").

4)	Select "License PAK 1" and enter "DEC-PWLMXXXCA06.00" in the Value field.

5)	If you are running the PATHWORKS client software (DECnet, LAT, etc), then you also need a client system license. Select "License PAK 2" and enter "DEC-PWXXWINAT07.00" in the Value field.

6)	Click on OK to close the Licensing Agent properties sheet.

7)	Click on OK to close the Network control panel.

8)	Reboot the PC.

Windows NT 3.51 Clients

Windows NT 3.51 systems can be configured to select a PATHWORKS Client Access license as follows:

1)	(optional) Configure your PATHWORKS for Windows NT client as usual, but remove the included license agent from the Network applet from the Control Panel.

2)	Install the Licensing Agent from version 1.4 or later of the DIGITAL PATHWORKS Licensing Software kit, available from your CSC or http://www.digital.com/pathworks.  Be sure to read the README.TXT file, especially step 5, which directs you to remove the trailing backslash from the source location.

3)	From the File menu of the Program Manager, select Run… and enter PNLCFG and press enter.

4)	Select any unwanted licenses and click the Remove button.

5)	Click the Add button and enter "DEC-PWLMXXXCA06.00" in the Value field.

6)	If you are running the PATHWORKS client software (DECnet, LAT, etc), then you also need a client system license. Click Add again and enter "DEC-PWXXWINAT07.00" in the Value field.

7)	Click OK to close the properties sheet, then OK to close the control panel.

8)

Reboot the computer.

Windows NT 4.0 Clients

Windows NT 4.0 systems running PATHWORKS 32 can be configured to select a PATHWORKS Client Access license as follows:

1)	Configure your PATHWORKS for Windows NT client as usual.

2)	Invoke the Network applet from the Control Panel menu.

- In the Network Control Panel, select the Services tab.

- Select "DEC LAN Manager Licensing Agent" and click on the Properties button

- In the “Select Licenses” section, remove any inappropriate licenses by selecting them and clicking the Remove button.

- Click the Add button and enter "DEC-PWLMXXXCA06.00" in the Value field.

- If you are running the PATHWORKS client software (DECnet, LAT, etc), then you also need a client system license. Click Add again and enter "DEC-PWXXWINAT07.00" in the Value field.

- Click OK to close the properties sheet, then OK to close the control panel.

Reboot the computer.

PATHWORKS for OS/2 Clients

OS/2 systems can be configured to select a PATHWORKS Client Access license as follows:

1)	(optional) Configure your PATHWORKS for OS/2 client as usual, but remove the included license agent.

2)	Install the Licensing Agent from version 1.4 or later of the DIGITAL PATHWORKS Licensing Software kit, available from your CSC or http://www.digital.com/pathworks. Instructions can be found in the \OS2\README.TXT file. This will have you add the following lines to your logon script (or startup.cmd):

detach pwliclmt

pwliclm [optional parameters] [license server]

Note that pwliclm can have optional parameters. You can use this to specify exactly which licenses to request. For example, a system using both Client Access Licenses and a client System License (required if you are running PATHWORKS for OS/2) and requesting licenses from a server named GNORM would use a line such as this:

Pwliclm /l:license.txt gnorm

and a license.txt file like this:

license00=decpwlmxxxca06.00

license01=decpwxxwinat07.00

which should result in this:

PATHWORKS V5.1 for OS/2 (LANMAN) Client License Requester V5.10.000.

Copyright (C) 1993-1994 by Digital Equipment Corporation.

All Rights Reserved.

This client obtained a license for PWXXWINAT07.00.

Client environment information:

Client ID: B85F912B0008.

NetBIOS name:     NOLES.

This client contains a license for PWLMXXXCA06.00.

from license server GNORM.

obtained from group default.

This client contains a license for PWXXWINAT07.00.

from license server GNORM.

obtained from group default.