ASPIDRIVER IBM V1.00 command line switches

------------------------------------------

DEVICE=ASPIIBM.SYS /l| /m| /n<slot>| /a<bus-on time>| /b<bus-off time>|

/i| /d <HAN>,<ID>,<BYTE>| /z <slot>| /s<scsi-speed>| /rw||

/L	enable nonzero LUN SCSI scanning at boot (for ASPI=1 GetDeviceType)

/M	disable full message display during boot

/I	disable int 13h module loading;by default int 13h module is loading

for first ibm host adapter recognized:if autoscanning is enabled

(no /n option set) the driver assigns as the first adapter the ibm

adapter at the LOWEST slot number

NOTE : if you are booting from a SCSI hard disk and you are loading

ASPIIBM.SYS DO NOT USE /I option

/Z<slot>exclude slot from install (MCA BUS)

/N<slot>install slot number (MCA BUS) - can be given multiple times

if no /n option exists the driver is autoscanning the SCSI bus and

installing all supported cards on MCA BUS-slot increasing order

/R	do SCSI hard reset to all found host adapters.This option is useful

after a host adapter previous "hang" (the host adapter LED remains

on after reset,the card is not recognised at boot by the driver,etc)

/RW	do SCSI hard reset and wait 5 seconds before any SCSI request is made

to the host adapter (for slow old SCSI devices that could "hang" the

host adapter after SCSI bus reset is sent)

/D<HAN>,<ID>,<BYTE> set unique parameter byte configuration option for

the specified host adapter and SCSI ID.You can set the following bits

bit2-prevent target disconnect

bit3-no autosense

bit6-disable special ibm cached read/write commands for int 13h

For details about the configuration byte see ASP=5 Set Host Adapter

parameters

/A<bus on  time decimal> - specifies the amount of relative time the host

adapter bus master is allowed to hold the bus

/B<bus off time decimal> - specifies the amount of relative time the host

adapter bus master must release the bus

On the IBM card the buson value means the percentage of 100% of the

MCA bus bandwidth to be used:tbuson+tbusoff=100 (if not it is displayed

an appropiate error);

NOTE:BOTH switches must be specified for programming the DMA pacing

factor

/S<SCSI synchronuous transfer speed hexa> - specifies the SCSI timing of the host adapter

the allowed values are

scsi speed	scsi sync rate (IBM) (MB/s)

0		 5.00

1		 4.00

2		 3.33

3		 2.86

4		 2.50

5		 2.22

6 		 2.00

7		 1.82

ASPIDRIVER IBM V1.00 initialization and mesages

-----------------------------------------------

Note : most of the initialization messages can be disabled to be

displayed with the /M option.

After the initial message

"CorelASPI for DOS - device driver V1.00"

"Copyright (c) 1992-1993 Corel Corp."

the driver starts processing of the config.sys line. If any i/o slots

are specified the driver tries installing only the cards that

correspond to the specified slots;else the driver scans the MCA

bus for valid SCSI supported cards and installs them all. If none is

found the driver exits without installing with the message

"No supported SCSI cards located"

"Driver is not installed"

At the end of the initialization the exit message displays the

number of SCSI cards installed.

"Installation complete : x SCSI  card(s)   located"

x-number of installed cards

Any config.sys command error is ignored with the message

"Unknown command line errors ignored"

The per card host adapter specific initialization starts with the

message

"Initialising host adapter : x"

"Configuration I/O : yyyy, IDD : d, IRQ : bb, DMA : a"

x-host adapter number (0 based)

yyyy-base I/O port for the card

d-the bord ID byte as returned by adapter inquiry command

bb-interrupt level for the card

a-dma channel for the bus master chip

Also the slot number is displayed

"Microchannel slot : x"

x-slot number (1-8)

Then the driver is displaying the IBM host type and bus width

"  Host : IBMSCSI/xx Va.bb"

xx=16/32 bits the bus width

a.bb the firmware revision number

The driver is scanning the SCSI bus for present devices and displayes

the results. If the /L option is specified the scanning is made for

every SCSI LUN on a given SCSI ID, else the inquiry is made only for

LUN = 0 on a given ID.

The /A /B /S command line switches display the programming of the

specified parameters. These parameters are GLOBAL for EVERY installed

card.

"    Configuring DMA pacing factor to xxx%, host : yy"

"     Configuring SCSI synchronuous speed to zz.z MB/s"

xxx-percentage of bus used

yy=host adapter number

zz.z-value in mbytes/second of the scsi synchrounuous transfer rate

If the per card initialization is succesfull the last message is

"Initialising complete host adapter : x"

else

"Initialising error host adapter : x error : yyyy"

x-host adapter number (0 based)

yyys-the error code

In this last case the driver displays also a corresponding error

string.Also in should be noted that in this last case the driver is

not installing FOR the specified card.

If there are any present SCSI hard disks, the driver changes the

hard disk interrupt services vector. If there are any CMOS installed

hard disks, the driver installs the vector after them.Remember that

the /I option disables the loading of the int 13h vector. Use this

option if you have no SCSI hard drives	in your system, so that the

driver do not keep the code for int 13h resident.

Upon succesfull instalation of the vector the driver displays the

message

"ASPI - hard disk interrupt SCSI module  V1.00"

"INT 13H redirected - for X:,Y drive(s)"

X=start drive for which the vector is installed (C: or D:)

Y=number of SCSI hard drives for which the vector is installed

Following are the int 13h error messages

"SCSI device id 0 lun 0 not a direct access device"

"INT 13H function error"-the BIOS read drive parameters failed

"Read test INT 13h error"-the verify test with the new int 13h

vector failed.

The autoscan on MCA bus is in the order slot #1 to slot #8.

If the slot in which the SCSI boot hard disk is present is AFTER

another IBM SCSI card slot (slot 4 non bootable SCSI card, SCSI 6 hard

disk to boot from), the int 13h vector will not install properly

unless you specify on the command line /n6 /n4 as in our

example, to force the installation of the cards in your own order.

This will ensure that the boot host adapter number IS zero

and the int 13h vector installs by default.Alternately you can assign

in a lower slot the IBM card that has the bootable hard drive;

Attention : for correct functioning with more than one IBM card in the

system you must insure that the BOOTABLE hard drive is always on the

first IBM host adapter

NOTE:	After installing Windows you may REM the config.sys line

device=c:windowssmartdrv.exe /DOUBLE_BUFFER.This option is

not needed by the ASPI MANAGER

NOTE:	MCA bus = IBM MicroChannel PS/2 I/O bus

NOTE:	NEW FEATURES

1.support for > 1 GB hard drives if BIOS supports it also

2.support for > 2 hard drives if DOS 5.0 or higher present

on multiple host adapters;for each host adapter the drives

letters are assigned in SCSI ID decreasing order, and for

multiple hosts they are assigned in slot increasing order

SCSI ID's must not be contiguous.

NOTE:	Through IBM configuration from the reference diskette you

can change the hard disk boot device : for example if you have

two hard disks SCSI ID=6 and ID=4 the normal boot drive is from

ID=6;with Set features menu you can specify in Set startup

sequence to boot from drive at ID=4;When loading ASPIDRV.SYS

DO NOT USE THIS OPTION;ASPIDRV.SYS will always try to boot

from hard disk ID=6 (as in our example).