Advanced Installation Guide for BocaRam/AT Plus  (BRAT80/BRAT90)

Before installing the BocaRam/AT Plus:

Be sure the board is populated with at least 2MB RAM.

Populate the board starting with Bank 0 and Bank 1

See manual for illustrations.

Installation Guide:

Insert board in a 16-bit slot.

Power the machine on with a "plain vanilla" boot diskette.

(one with no CONFIG.SYS or AUTOEXEC.BAT file)

After boot is complete, insert the BocaRam/AT Plus

installation disk.

Type ADVINST at the A:\> prompt

Program Guide:

Screen 1: Introduction, press any key to continue.

Screen 2: This screen should tell how much memory you have

installed on the BocaRam/AT Plus board.  If the amount is

correct, press "Y" for yes.

If the installation program does not recognize the amount of

memory on the Boca memory board then you may have a problem.  To

continue, enter the amount of RAM you have installed and then

press ESC to move to the next screen.  Remember, RAM must be

installed in 2MB increments.

Screen 3: CPU and Bus speeds.  Both of these questions should be

answered by the program.  If they are correct, then answer YES to

confirm each question.

If the program is incorrect or asking for the proper speeds,

consult your computer's documentation for the proper answers.

Normally, the Bus speed is 1/2 to 1/3 the speed of the CPU clock

speed. i.e. 286-12Mhz, CPU 12Mhz, BUS 6 or 8Mhz.  If in doubt

choose 8Mhz as your Bus speed.

Screen 4: Memory Allocation Menu.  For this task you will use the

arrow keys and the plus (+) and Minus (-) keys to allocate memory

that fits your needs.  This screen only shows the memory on the

BocaRAM/AT Plus board.  It will not show existing memory on the

motherboard.

If you have only 512K memory on the motherboard, you must

back-fill conventional memory.  Increase the conventional memory

option to 128K, which gives you 640K total.  Then allocate the

rest of the memory to Extended and/or Expanded memory based on

your present needs.  To set the Extended Memory Start, accept the

default or see document on "Selecting the Proper Start Address".

Extended memory stop is set automatically.

Screen 5: RAM speed and wait states.  The RAM Access Speed should

match the speed of the RAM if it is 80ns to 120ns, 100ns is the

default.  If the RAM is faster than 80ns (or a lower number) then

the RAM Access Speed should be set to 80ns.  The board is

designed to handle RAM that is 80ns to 120ns in speed.

If the BUS Speed in Screen 3 was 4Mhz to 8Mhz the Total Wait

States can be set to Zero.  If the BUS Speed is 10Mhz or higher

then the Total Wait State should be at least 1. Set it to 2 total

wait states if the BUS is 12Mhz or higher.  Press ESC when

complete.

If the RAM access speed is 0ns then you may have a problem.

Set the Access speed to the proper setting.  Setting the total

wait state to a higher number can help the board overcome bus

noise and interference.

Screen 6: PARITY?  Always say no to this question.  Parity is

only used for older RAM chips that require that sort of testing.

The basic installation program sets parity to NO as a default.

Extended Memory

Screen 7a: Update CMOS.  If you configure the BocaRam/AT Plus for

all Extended Memory, this would be the last screen you'll see in

the installation process.  You say yes to updating the CMOS

configuration.  This will update your computers memory setup and

reboot.  During the Boot process the machine will count the extra

installed memory.

If you have installed Expanded Memory, you will be asked to

choose the proper addresses that the manager will use.

Expanded Memory

Screen 7b: I/O Address selection.  The default is 268h.  Unless

this is a known conflict with another device in your machine,

accept it, as is.

Screen 8: Page Frame Address:  This board provides up to 128K to

address your page frame.  The minimum requirement is 64K.  That

translates to four 16K pages in a row.

The addresses that are available are C000, C400, C800, CC00,

D000, D400, D800, DC00.  You will need to choose at least four of

the pages the are continuous.

NOTE:  If you have VGA installed, C000 and C400 will not be

available for use by the memory board.  Network boards also use

an address in this area and should be inspected before

configuring this board as Expanded Memory.

Follow the instructions on the screen to select the page

frames you would like to use for the memory specification.  Then

press ESC to continue.

Screen 9: Update CONFIG.SYS file:  Choose yes to updating the

file.  If you have already done this it will not damage any files

to do it again.  After the installation program is complete, you

may want to inspect the CONFIG.SYS file to ensure that the

appropriate lines were added.

Screen 10: Update CMOS.  See screen 7a above.

Installation is Complete.

If the program did not follow this guide smoothly then you

may be facing one the many pitfalls in memory upgrading.  For

example, if the Advanced Installation Program does not recognize

how much memory is on the board, you may have the RAM installed

improperly or the RAM may be defective.  If the installation runs

smoothly and there is a problem with the application your are

running, then run the BRPTEST program on the Boca diskette.  The

RAM chips or the address space on the motherboard may be

defective.  Remember to always boot with a plain vanilla DOS

diskette (one with no AUTOEXEC.BAT or CONFIG.SYS) when working

with the board.

Severe Error 002:  This is a message that informs of an

inability to program the EEPROM on the BocaRam/AT Plus board.

Either the 300h address is in conflict with another board or the

jumper to enable/disable the EEPROM is set improperly.

If problems persist beyond the issues described in this

document, call the Technical Support Department.  The phone

number is found in the product manual.