HOW TO FIND A STARTING ADDRESS

Trying to calculate a start address for Extended memory can be a confusing

ordeal.  Having everything measured in Kilobytes (K) doesn't help either.  This is

written to answer the questions and aid in the selection of proper addresses when

installing a memory board.

In any discussion it's always best to know what you are talking about.  The

following definitions should be of use during this exercise.

Conventional Memory

This type of memory is also known as Base Memory and is a maximum of

640K. Base memory is used by programs and TSR (Terminate and Stay Resident)

drivers.  If there is not enough Base memory to load a program, DOS will announce

an error of "Not enough memory to load program".  Although programs do use other

types of memory for their work, they can only load into Conventional Memory.

Extended Memory

Extended memory (also referred to as Expansion Memory) is physically

addressed on the motherboard starting at 1024K (1 Megabyte) and occupying every

consecutive address above it, until you run out of existing RAM.  When you add

memory expansion boards, this is where your start address resides.

Expanded Memory

Expanded memory is not formally recognized by DOS and system and requires

the use of a manager program.  This program creates a 64K page frame in which the

memory is "paged" through.  This page frame window is located in area of address

space known as Reserved Memory. Much like a lazy-susan, you only have access to

64K of the memory at a time.  When the computer has a need for information that is

located in another portion of the memory, the manager pages to that section.  This

all happens unseen to the computer operator.

���������������������� 16MB

�

�

�

� Extended Memory               Memory address space provided by 286 ATs and up

�

�

�

���������������������� 1024K

�

� Reserved Memory Address       RAM cannot be addressed to the Reserved Area

�

���������������������� 640K

�

�

� Base Memory                   Programs can only use Base Memory

�

�

���������������������� 0K

Reserved Address Space

This is the area located between the 640K and 1024K address marks.  The IBM

tandard defined this area to be used for system use. TSR drivers, expanded memory

managers, video and network card I/O addresses can all be addressed in the 384K

reserved area.

Selecting Start Addresses

When selecting a start address for your memory board, you must first

determine the configuration of your existing memory. This is done by converting the

RAM amount into its' literal size.  1MB = 1024K, 2MB = 2048K, etc.

When you have RAM on your motherboard the computer has to map it to

existing address spaces in order to make use of it.  The address spaces were outlined

in the first part of this article.

Once you are aware of the amount of RAM that is installed on the motherboard,

it becomes a matter of just matching numbers.  In a system with 1MB of RAM

installed, there would be 640K Base and 384K Extended.  The Extended memory

starts at the 1024K address mark and occupies 384K of space.  This memory would

stop at 1408K address mark. 1024K start + 384K RAM = 1408K.  Mapping the

memory in this way means totally skipping the Reserved Address Area used by the

system.

Memory has to occupy consecutive addresses, but it will NOT occupy the

reserved area.  What results is a 384K offset when mapping memory that exceeds

640K.  The following chart may help to visualize this concept.

MEMORY                    START

SIZE          ADDRESS

640K      1MB+0K=   1024K

1024K     1MB+384K= 1408K

2048K     2MB+384K= 2792K

3072K     3MB+384K= 3456K

4096K     4MB+384K= 4480K

5120K     5MB+384K= 5504K

Exceptions to the rule are always present.  In this case they take the form of

shadow RAM, reserved memory, and proprietary addressing schemes.

With shadow RAM and reserved memory (not to be confused with reserved

address space), the system recalls a portion of the mapped RAM for it's own use.

How and why make no difference right now, but the fact that the memory is no

longer being used as RAM does.

When this RAM is recalled for other uses you must make the adjustment to

your starting address.  For instance, some IBM micro-channel model computers

reserve 128K for use with OS/2.  If you have 2MB installed on the motherboard, your

start address would be 2MB+384K or 2792K.  If the computer is reserving 128K for

other uses you must subtract 128K from the total start address.

Example:

IBM Model 80 w/2MB 2048K RAM

BocaRam/2 Plus w/4MB 4096K RAM

After the installation procedure is completed you should check the start

address.  If it is wrong, the RAM will overlap, and that will cause an error.

This machine reserves 128K for OS/2 because the total usable memory states

1920K (that's 2048K - 128K).  Be sure the start address is set to 2MB + 256K (or

2664K).  The memory on the board will start where the motherboard left off.