OrCAD

OrCAD version 4.x supports a resolution of 1024x768 with 16 colors.

Beginning with version 4.04 of OrCAD, 800x600/16 colors is directly

supported using the VESA standard driver which is included with the

program. An 800x600 driver for OrCAD can also be built using a tool

supplied with OrCAD called GENDRIVE.EXE, if the version you are

running does not include the VESA standard driver. SuperVGA boards

with one megabyte of video memory can use the 1024x768/16 colors

driver.

Installing OrCAD drivers

The 1024x768 OrCAD display driver called WD1K.DRV is installed by

the ORCAD installation program. It can be selected by marking the

label 1024x768 16 Western Digital VGA from the menu of display

drivers presented when installing OrCAD. The VESA standard 800x600

driver included with OrCAD is copied by the installation program.

Simply mark the driver labelled 800x600 16 VESA SuperVGA in the

menu of display drivers when installing the OrCAD program.

Using the OrCAD GENDRIVE utility

GENDRIVE.EXE is a utility supplied by OrCAD that has the ability to

create an 800x600 driver. The utility is located on the OrCAD

driver diskette that is shipped with OrCAD. GENDRIVE is

automatically copied to the OrCAD directory which contains display

drivers, typically ORCADESP\DRV.  The driver may be created as

follows:

1. Make the OrCAD driver directory your current working directory

by typing:

CD\ORCADESP\DRV    [Enter]

2. Run the GENDRIVE utility by typing GENDRIVE and then press

[Enter], which displays the following message:

Enter a Display Card Number or S(Special), M(More) or Q(Quit):

3. Press S which displays:

Number of Screen Columns (640, 800):

4. Type 800 and then press [Enter] to create a 800x600 driver. This

displays:

Number of Screen Rows (350, 480, 600):

5. Type 600 [Enter] to create a 800x600 driver, which displays:

AX Value for Initializing INT 10H (hex):

6. Type 6A [Enter] to create a 800x600 driver, which displays:

BX Value for Initializing INT 10H (hex):

7. Type 0 [Enter] which displays:

Identification displayed during driver initialization: Driver Name:

8. Type OrCAD 800x600 GENERIC DRIVER to create an 800x600 driver in

response to the following message:

9. Press Q to abandon redefinition of EGA/VGA Palette Definition.

Please Enter Filename for updated driver:

10. Type ORCAD800.DRV [Enter]. The OrCAD display driver has now

been created.

Configuring OrCAD

When OrCAD is first used with its display driver, it must be

configured for the computer's hardware. OrCAD may be configured for

a display driver as follows:

1. Run OrCAD by typing ORCAD [Enter].

2. Select CONFIGURE ESP from the main menu.

3. Select the driver from the menu of display drivers presented, if

it is present. Otherwise place the cursor in the field containing

the name of the current display driver and delete the name

displayed there. Type:

VESA800.DRV for the VESA standard 800x600 driver, or

WD1K.DRV for the Western Digital VGA 1024x768 driver, or

ORCAD800.DRV to select the driver created with GENDRIVE as

described above.

4. Return to the Main Menu and exit OrCAD.

5. Restart OrCAD to use the driver selected.