Boca Internal Modem and Fax/Modem Installation Hint

Subject: Troubles finding a COM port.

This describes how to use DEBUG from DOS to find an available Com Port address for

your Boca Internal Modem and Fax/Modem.

With the modem removed from the system, boot the machine with a plain DOS boot

disk.  One with no AUTOEXEC.BAT and CONFIG.SYS.

Change your prompt to the DOS directory and type DEBUG and press <RETURN>.

At the Dash Prompt type D 40:0.  Debug will generate an entire paragraph of address,

for our purposes we will only use the first half of the first line.  This portion denotes

the COM addresses.  This diagram describes how your screeen should look.

C:\DOS>DEBUG

-D 40:0

0040:0000 F8 03 F8 02 E8 03 E8 02-

Explanation:

0040:0000 Location Address Byte and Offset

F8 03 inverted is COM1 03F8h

F8 02 inverted is COM2 02F8h

E8 03 inverted is COM3 03E8h

E8 02 inverted is COM4 02E8h

If any of these sets of addresses are zeros then the com port is available for use by

your Boca Internal Modem or Fax/Modem.  Below is a possible scenario:

Your computer has one two serial ports installed.  You have a mouse on COM1 and

COM2 is not being used by a device.  Running DEBUG gives you the following results.

0040:0000 F8 03 F8 02 00 00 00 00-

0040:0000 Location Address Byte and Offset

F8 03 inverted is COM1 03F8h

F8 02 inverted is COM2 02F8h

The zeros located in COM3 and COM4 spots denote the absence of those ports.  You

have these address open to use for your modem.  Set the modem for COM3 or COM4

along with the appropriate IRQ and install your software.  (Remember, High-speed

modems do not like to share IRQs.  It is suggested to use IRQ5 for COM3 and IRQ7

for COM4.)

Change your prompt to the DOS directory and type DEBUG and press <RETURN>.

At the Dash Prompt type D 40:0.  Debug will generate an entire paragraph of address,

for our purposes we will only use the first half of the first line.  This portion denotes

the COM addresses.

The following chart displays the possible options when running DEBUG.

If DEBUG says this...                       You have this installed.

0040:0000 F8 03 00 00 00 00 00 00-          COM1

0040:0000 F8 02 00 00 00 00 00 00-          COM2

0040:0000 E8 03 00 00 00 00 00 00-          COM3

0040:0000 E8 02 00 00 00 00 00 00-          COM4

0040:0000 F8 03 F8 02 00 00 00 00-          COM1, COM2

0040:0000 F8 03 E8 03 00 00 00 00-          COM1, COM3

0040:0000 F8 03 E8 02 00 00 00 00-          COM1, COM4

0040:0000 F8 02 E8 03 00 00 00 00-          COM2, COM3

0040:0000 F8 02 E8 02 00 00 00 00-          COM2, COM4

0040:0000 E8 03 E8 02 00 00 00 00-          COM3, COM4

0040:0000 F8 03 F8 02 E8 03 00 00-          COM1, COM2, COM3

0040:0000 F8 03 F8 02 E8 02 00 00-          COM1, COM2, COM4

0040:0000 F8 03 E8 03 E8 02 00 00-          COM1, COM3, COM4

0040:0000 F8 02 E8 03 E8 02 00 00-          COM2, COM3, COM4

0040:0000 F8 03 F8 02 E8 03 E8 02-          COM1, COM2, COM3, COM4

It is important to have the modem removed from the system when running DEBUG.

If you have two objects addressed for the same COM port you will only see one

address.  DEBUG does not report conflicts, only what addresses are being used.