IDE Plus CONTROLLER - VL by Boca Research, Inc.

This file contains additional installation details; the latest additions

to the manual; and technical specifications for getting the best performance

from your IDE Plus Controller - VL (Product Code IDEVL1).

This file contains 2 sections:

Section One

-----------

I............................... Software Installation Details.

II..................................Motherboard Speed Selection.

III.....................................Obtaining Driver Updates.

Section Two

-----------

I...............................Multiple Sector Transfer (MST).

II.......32-bit Operation by Device Driver or Extended ROM BIOS.

III.......................................IDE Drive Cycle Timing.

IV...........................Motherboard BIOS vs. Device Driver.

V........Data Transfer Cycle:  Concurrent Read/Write Operation.

VI......What Affects IDE Drive Performance (Data Transfer Rate).

VII.........................How To Make a Performance Comparison.

VIII.............................................Seek Performance.

Section One

Detailed Installation Guidelines for the IDEVL1

===============================================

I.   Software Installation Details.

The INSTALL utility on the distribution diskette makes the following

installations for you. The following instructions are for users who

wisht to manually install the software drivers.

A. DOS Device Driver Installation

2015.SYS is the DOS device driver supplied with the IDEVL1. Add the

following line to the very beginning of you CONFIG.SYS file.

Device=C:\2015.SYS

You may place 2015.SYS  in one of your sub-directories, but make

sure that the proper directory path is specified in the DEVICE= command

line.

The DOS device driver replaces the ROM-BIOS routines on your PC's

system board with routines that can take advantage of the capabilities

of the IDEVL1 controller.

B. Microsoft Windows Driver Installation

Microsoft Windows version 3.1 supports 32-bit disk access, but the

disk data transfer between host and the IDE adapter is still limited

by the 16-bit ISA bus. If you want to speed up your hard drives under

Windows through the 32-bit VL-Bus, you will need to modify the

SYSTEM.INI in your Windows SYSTEM directory (this file primarily

contains settings that customize Windows to meet your system's

hardware needs).  If you do not modify the SYSTEM.INI file, your system

will still function, but performance will not be optimized.

Please follow the following steps to modify your SYSTEM.INI file:

1. Use any ASCII word processor to open your SYSTEM.INI file.

Search the [386 Enh] section to find the following lines:

[386Enh]

device=*int13

32BitDiskAccess=on

device=*wdctrl

2. If the statement "32BitDiskAccess=Off", change it to "ON".

3. Insert a ";" before the line "device=*wdctrl" to disable the

wdctrl setting, so the previous lines should be:

[386Enh]

device=*int13

32BitDiskAccess=on

;device=*wdctrl

4. Add the following line to the [386Enh] Section

device=C:\windows\system\2015.386

You may place DC50P.386  in any one of your sub-directories, but

make sure that the proper directory path is specified in the

DEVICE= command.

The lines above should now read:

[386Enh]

device=*int13

32BitDiskAccess=on

;device=*wdctrl

device=C:\windows\system\2015.386

5. Save the new SYSTEM.INI file.

6. Copy the file 2015.386 to the directory you specified in step 4,

usually C:\windows\system

7. Load or restart Windows to bring up your PC with the new system

configuration.

II.    Motherboard Speed Selection

The 2015P driver will attempt to program the IDEVL1

to the best speed setting for each drive.  But it does this while

making the assumption that the motherboard is running at 50MHz.  If

the 2015P is being used on a 33MHz motherboard, some drives may get

better results using the 33MHz timing tables.

If the 2015P is running on a 33MHz motherboard, then you can

tell the IDEVL1 to use the 33MHz timing table by adding the

following switch to the "device=" line in the CONFIG.SYS file:

DEVICE=C:\2015P.SYS /P:33

The "/P:33" option tells the device driver to use the 33MHz timing

table instead of the default 50MHz timing table.

III. Information on obtaining updated drivers.

From time to time, Boca Research may provide updates to the IDEVL1 Drivers.

You may use your modem to dial the BOCA RESEARCH B.B.S. and download the

drivers to your computer.  After downloading drivers, make sure to read any

'read.me' files supplied with the new drivers for installation notes and

instructions.

The Boca Research B.B.S. supports all modem speeds. Set your communications

software  to 8 Databits, 1 Stop Bits, No Parity.

Dial  (407) 241-1601

The Boca Research B.B.S. is available 24 hours per day.

Section Two

Technical Specifications of the IDEVL1

===============================================

The following outline discusses the issues most commonly faced

when dealing with how the IDEVL1 integrates into the architecture

of the VESA local bus to IDE hard drive interface.

I.   Multiple Sector Transfer (MST).

The conventional AT BIOS handles hard drive data in a 512

byte cluster called a sector.  Each time the system

transfers a sector to or from the hard drive, some time is

spent establishing and finishing the transfer.  These

operations can be grouped together and considered as the

"transfer overhead".  This transfer overhead remains the

same regardless of the size of the sector.  During our

testing, we have found that in many systems this overhead

takes about the same time as the actual data transfer.

Thus, eliminating the transfer overhead would nearly double

the transfer rate.

The ANSI ATA standard supports "Multiple Sector Transfers"

(MST).  This groups a number of sectors together into one

package.  This package can then be transferred using a single

"transfer overhead".  If a drive is capable of transferring

64 sectors in one MST package, then the transfer overhead

would be one-sixty-fourth of what it would be if each sector

were to be transferred individually.

Some final notes that are important to keep in mind:

1.   Not all IDE drives support MST.

2.   Not all IDE drives that support MST can do it very

efficiently.

3.   The BIOS on current motherboards does not support MST.

4.   MST factors of 2,4,8,16,32 and 64 are defined by the

ANSI ATA standard.

II.  32-bit Operation by Device Driver or Extended ROM BIOS.

Currently, IDE drives have a 16-bit interface for data

transfers and an 8-bit interface for programmed I/O.  Also,

the motherboard BIOS supports only 16-bit IDE routines.  To

take advantage of the VESA 32-bit bus, a 32-bit instruction

based hard drive routine can replace the motherboard's

original 16-bit instruction based BIOS routine.

The IDEVL1 interface chip provides the interface between the

32-bit local bus and the 16-bit IDE drive, by automatically

translating a 32-bit local bus cycle into two consecutive

16-bit IDE cycles with a minimum of overhead on both the

VESA and IDE sides.

Usually, it will take a device driver or an extended ROM

BIOS to replace the motherboard BIOS IDE routine to realize

the 32-bit access.

III. IDE Drive Cycle Timing.

The ANSI ATA revision 3.0 specifies three types of IDE cycle

timing as described in the following table.  Each drive

manufacturer tries to minimize the data transfer timing in

order to provide a faster transfer.  Different IDE drives

will come with their own timing specifications which are at

least the same or faster than the timing defined by one of

the 3 IDE timing types.

In theory, an IDE drive with a 300ns cycle time is faster

than an IDE drive with 400ns cycle time.  However, on the

ISA bus, there is very little we can do to improve the IDE

cycle timing since the ISA bus is running at a very low

speed to insure compatibility with the slowest type of IDE

timing.  Some motherboards offer a more efficient ISA cycle

that can actually run at a faster IDE cycle timing than

other motherboards, but the improvement is limited.

However, on a VL-bus, we can tailor the IDE timing to the

individual IDE specification and actually take advantage of

the faster IDE data transfer timing offered by aggressive

IDE drive manufacturers.

Table of IDE Cycle Timing, ANSI ATA revision 3.0:

+----------------------+----------------------+

| 16-bit Data Transfer | 8-bit Programmed I/O |

+----------------------+----------------------+

Drive  |   Active    Cycle    |   Active    Cycle    |

Type   |   pulse     time     |   pulse     time     |

+--------+----------------------+----------------------+

| Type 2 |   100       240      |   290      >290      |

| Type 1 |   125       383      |   290       383      |

| Type 0 |   165       600      |   290       600      |

+--------+----------------------+----------------------+

IV.  Motherboard BIOS vs. Device Driver.

The original motherboard BIOS's were developed to work with

all existing IDE drives, so they tended to have very

conservative timing and thus had very slow access times.

The latest versions have improved the IDE data transfer

efficiency, but are still limited to 16-bit ISA bus

operation and, as mentioned earlier, they do not support

MST.

Since the motherboard BIOS works together with the

motherboard chipset, different combinations of

BIOS/chipset's produce different IDE data transfer

performances.  In other words, the same drive/controller

will produce different results on different motherboards.

To overcome these limitations, the motherboard's IDE BIOS

routines are replaced with the 2015p.sys routines when the

2015p.sys driver is loaded.

In summary, the reasons for replace the BIOS routines are as

follows:

1.   Provide more efficient 32-bit transfer routines.

2.   To enable MST operation, if the IDE drive supports it.

3.   Identify the IDE drive in use to select the fastest

possible IDE data transfer cycle timing.

V.  Data Transfer Cycle:  Concurrent Read/Write Operation.

To read a block of data, the CPU must transfer it 32 bits at

a time.  For the typical 512 byte sector, this means cycling

through a transfer loop 128 times.  If a block (one or more

sectors) is to be read from the IDE drive buffer into

memory, then the transfer consists of an I/O read from the

drive buffer, followed by a memory write.  To write a block

from memory to the drive buffer, the sequence is just

reversed:  a memory read followed by an I/O write.

During a read cycle, the IDEVL1 fetches the 32-bit chunk of

data from the IDE drive buffer and presents it to the CPU

through the I/O channel.  Then the CPU writes the data to

the main memory.  The functions are just reversed during a

write cycle.

Since the IDEVL1 is running on the local CPU bus, the timing

between the IDEVL1 and the CPU is so closely tied as to appear

to be synchronous.  Because of this synchronous timing, the

IDEVL1 does not have to wait for the CPU to finish writing the

data to memory before the IDEVL1 retrieves the next 32-bit

chunk of data.  Thus, this transfer can be said to be

concurrent.  That is, while the CPU is reading or writing

memory, the IDEVL1 is loading or unloading the IDE drive

catch.  This parallel operation technique reduces the

transfer time.

VI. What Affects IDE Drive Performance (Data Transfer Rate)

1.   The IDE Hard Drive Routines.

As discussed in Section IV, the motherboard BIOS

routines are not sophisticated enough to take advantage

of the newer high-performance IDE drives.  Replacing

these low-end routines with higher performance routines

results in significant transfer rate improvements.  The

IDEVL1 is packaged with a Device Driver or a ROM BIOS

that takes advantage of these high performance drive

features.

2.   Multiple Sector Transfer (MST) Factor.

As discussed in Section I, a communication link (hand-

shaking) must be established each time data needs to be

transferred.  The larger the data package, the less time

spent hand-shaking, and the more time spent working.

Thus, the larger the MST factor, the better the data

transfer rate.

The size of the drive's data buffer as well as the

drive's internal controller algorithm will determine

the MST factor.

3.   The IDE Drive Access Time.

The Access Time is the delay between the time data is

requested, and the drive says that it has the data

ready.  From the perspective of the IDEVL1, the hard

drive is considered a 'black box'.  This means that it

is not known what the drive does to get the data that

is requested.  The IDEVL1 just deals with the drive's

data buffer.  This data buffer is the window through

which all data is transferred.  The IDEVL1 requests the

data from the drive and at some time later the drive

says that it has the data ready in the buffer.  This

delay is called the Access Time of the drive.

4.   The Data Transfer Cycle Time.

The Data Transfer Time counts how long it takes to move

one 32-bit chunk of data from the IDE drive buffer to

the CPU and then to the main memory.  This process must

cycle through 128 times to transfer one 512 Byte

sector.  There are two distinct concerns for

performance as follows:

A.   The IDE Drive Data Transfer Cycle.

The electrical characteristics of the drive's data

buffer will determine the fastest possible speed

at which the data can be transferred to or from the

buffer to the I/O bus.

B.   The Motherboard Local Bus Cycle Efficiency.

There are two factors that determine how long the

motherboard takes to complete its' side of the

data transfer.

i.   The motherboard chipset design will determine

how efficiently the local bus transfer is

handled.  Some motherboards introduce some

timing overhead to provide more tolerance.

ii.  The wait states imposed on main memory access

will determine how long it takes to complete

the memory read or write.

Usually these factors combine to produce a Data Transfer

Cycle Time that is faster than most IDE drives.  But on some

systems, an unusually high number of DRAM wait states and an

inefficient chipset design can create an environment in

which the IDEVL1 will need to be run at a slower than normal

speed setting.

VII. How To Make a Performance Comparison.

In general, using the same motherboard, drive and testing

program, you should get a consistent performance grading:

The IDEVL1 with Driver will be the fastest;  The IDEVL1 using

the generic motherboard IDE BIOS routines will be second;

And the ISA IDE IO card with the motherboard IDE BIOS

routines is the slowest.

In rare cases one may see little or no difference.  This is

due to a variety of factors relating to the hard drive as

discussed in Sections VI.2, VI.3 and VI.4.A.

Essentially, the IDE drive itself is the bottleneck, not the

bus.  Keep in mind the 'black box' idea:  the drive

controller does not have any control over how the IDE drive

stores or retrieves that data.  The IDEVL1 deals only with the

IDE drive buffer, the window through which all of the data

must pass.

A final point on this subject, the purpose of the IDEVL1 is to

take advantage of the latest high performance IDE drives.

If no improvement is seen, then the drive being used does

not use any of the latest high performance features.

VIII.  Seek Performance.

Some performance rating programs will conduct a "Seek

Performance Test" to see how fast the drive seeks a

particular sector.  This is essentially testing the Access

Time of the drive as discussed in Section VII.3.  This type

of test is usually one of several tests run to gage the

over-all performance of the drive.

The seek operation is important for non-IDE hard drives

because the hard drive controller must first seek the

targeted sector before the transfer can be started.  With

an IDE drive, all that is needed is a request for a

particular sector;  The intelligent controller built onto

the drive takes care of seeking and retrieving the sector

and thus a seek command is not necessary for normal IDE hard

drive operation.

The Device Driver for the IDEVL1 processes a seek command by

verifying with the drive controller that the sector is

valid, and then it immediately reports back that the seek is

done.  No access time is actually involved and thus the seek

time is essentially zero.

We have noticed that this 'zero seek time' creates a problem

with the Norton hard drive performance utility.  Norton

makes the assumption that the seek operation will take at

least a certain minimum time.  Because of this assumption,

it ignores the immediately received 'seek done' status,

thinking that the status must be invalid.  At some point

later it decides that the status must be valid, notes the

time (absurdly long) and thus ends up giving erroneous

results.

----*----