Daniela's YMF724 patch utility

The current driver for the Yamaha WaveForce 192 sound board supplied by IBM

supports the YMF724D chip only. This patch utility modifies the driver to

accept other compatible chips instead of the YMF724D. These are

PCI vendor ID: 1073h (Yamaha)

PCI device IDs:

YMF724D (DS-1)  0004h

YMF724F (DS-1)  000Dh

YMF740C (DS-1L) 000Ch

YMF744B (DS-1S) 0010h

YMF754  (DS-1E) 0012h

Usage:

There are 3 methods to invoke the patch utility:

1) 'PatchYMF'

This performs a scan of the PCI bus for YAMAHA PCI sound chips only.

2) 'PatchYMF target_file'

This performs a scan of the PCI bus for YAMAHA PCI sound chips, and asks

the user to select one of the found. The given target file is then modified

*in-place* to accept the selected chip instead of the original one.

Example: 'PatchYMF YMF724.SYS'

3) 'PatchYMF target_file device_ID'

The given target file is modified *in-place* to accept the YAMAHA PCI sound chip

with PCI device ID as provided by the user. The ID may be a decimal value, or

a hexadecimal one preceeded by 0x.

Example: 'PatchYMF YMF724.SYS 0x000C'

Copyright 1999 Daniela Engert <dani@ngrt.de>