Daniela's S506 ADD - Release 1.6.7

----------------------------------

NAME

DaniS506.ADD  -  replacement for IBM1S506.ADD

SYNOPSIS

EIDE driver with support for Intel, VIA, ALi, SiS, CMD, Promise, Cyrix,

HighPoint, AMD, Artop, SMSC, ServerWorks, Opti, Nvidia, National Semiconduc-

tor, and SiliconImage chipsets, and generic busmaster support for all

SFF-8038i compliant chips.

This covers virtually *any* EIDE controller built/sold in the past few

years and even some just about to be released in the near future. Don't be

confused by the generic chip names below! If you are uncertain about your

actual controller to be supported, run the DumpIDE utility - it will tell

you.

###  This driver is *not* compatible to any of the software RAID setups offered

###  by some so called IDE RAID controllers (f.e. HPT, Promise FastTrack, AMI)!

###  Of course, you may run these chips in non-RAID BIOS setups (i.e. JBOD).

***  Changes to the previous release are denoted by "***" tags.

DESCRIPTION

Since the original IBM1S506.ADD doesn't support the IDE controller

found in the VIA 82C586(/A/B) south bridge beyond the basic features,

I wasn't very happy with the performance of my 10Gig DTTA-351010 drive.

The driver from VIA itself was no solution because it doesn't support

all the advanced features found in todays hardware and locked up my

system completely. So I sat down....

This release of my S506 driver is no longer based on the sources

provided by IBM on their DDK site. It has the latest feature set

as found in the current fixpacks and fixes later than these.

My enhancements are:

- (supports the Intel PIIX, just like IBM1S506 does) plus some PIIX4

variants unsupported by IBM1S506

- supports the Intel ICH/ICH0/ICH2/ICH3/ICH4/C-ICH/ICH5/ICH5R IDE

controllers

***  - supports the Intel ICH5/ICH5R SATA controllers

- supports the VIA IDE controllers

***  - supports the VIA PATA/SATA RAID controllers (experimental)(see note ###)

- supports the ALi IDE controllers

- supports the SiS 5513/5517/5518 IDE controllers

- supports the CMD 643/646/648/649 IDE controllers

(this includes OEMs like AMI)

- supports the Promise Ultra33/66/100/TX2/133 IDE controllers

- supports the Cyrix 5530 IDE controller (Cyrix MediaGX)

- supports the HighPoint HPT302/366/368/370/371/372/374 IDE controllers

- supports the AMD756/766/768/8111 IDE controllers

***  - supports the ACARD ATP850/860/865 IDE controllers (ATP860 experimental)

- supports the SMSC SLC90E66 IDE controller (PIIX4 look-alike) (exp.)

- supports the ServerWorks IDE controllers

- supports the Opti IDE controllers

- supports the Nvidia nForce/nForce2 IDE controller

- supports the National Semiconductor Geode SCx200 IDE controller

- supports the Silicon Image 680 IDE controller

- supports the Silicon Image 3112 Serial ATA controller

(this includes OEMs like Adaptec)

***  - supports the ITE 8212 IDE controller (experimental)(see note ###)

- generic busmaster support for SFF-8038i compliant controllers

***  - supports SATA <-> PATA adapters

- supports PCCard (formerly: PCMCIA) ATA/ATAPI controllers

- supports ATA-Flash, Compact-Flash, CF+, SmartMedia, etc. in PCCard slots

- supports internal DriveBays

- supports the external DriveBay and other PCCard attached units

- no need for chipset initialization by the BIOS

(Intel, VIA, ALi, SiS, CMD, Promise, and HighPoint)

- Ultra33-DMA on all of the above chipsets which can do Ultra-DMA

- Ultra ATA-66 on AMD, VIA, SiS, Promise, HighPoint, Intel, CMD, ALi,

SMSC, and Acard

- Ultra ATA-100 on HighPoint, Promise, VIA, CMD, Intel, AMD, SiS, Nvidia,

***    ACARD (these all tested), ITE, VIA RAID and ALi (these experimental).

***  - Ultra ATA-133 on ACARD, ALi, ITE and VIA RAID (experimental)

Intel, VIA, SiI, HighPoint, Promise, SiS, and Nvidia (tested)

***  - SATA-150 on Silicon Image, Intel (these tested), VIA RAID and HighPoint

(these experimental)

- faster PIO due to 32bit IO

- sets device inactivity timers (POWERSV.SYS is no longer needed)

- adjustable SMS length

- adjustable drive noise level

- optional write protection of the first track of a disk drive

- unlocks full capacity of disks which are programmed to report a lower

capacity to be compatible with older BIOSes

- improved support for removable media drives (in particular

Castlewood ORB EIDE and the Fujitsu M2541B MO drive)

- can change removable media drives to fixed media drives and vice-versa

- supports devices which don't spin up automatically

- optional limit data transfer rates for more reliability

- adapts (feature removed: auto-adapts) chip timings to PCI clock

- supposed to work with drive capacities up to 2TiB (OS/2 ADD limit),

tested with drives up to 200GB now

- improved compatibility with current BIOSes (IBM1S506.ADD fails on an

increasing number of recent BIOS implementations)

- performance and feature tuning options

- up to 8 EIDE channels

- enhanced S.M.A.R.T support

- some fixes for bugs introduced with the last release

- some fixes for bugs and omissions in the IBM code ;-)

- work around some BIOS geometry issues

- fixes possible data corruption with some VIA chips.

###  Caution: users of DaniBoot v1.0 need to upgrade to version 1.0.2!

COMMAND LINE OPTIONS

This driver supports most switches and options that IBM1S506 supports. For

an explanation of these look into the OS/2 online help (by picking

'OS/2 Commands by name'/ 'BASEDEV...'/'IDE...' or by "help ibm1s506"

from the command line).

COMMAND LINE OVERVIEW

Gerald Eischer did us the favour of creating a syntax diagram and a

short overview of the command line options implemented in the driver

Syntax Diagram

--------------

+------------------+ +-- /I --+  +---------------+

V		      | V	 |  V		    |

BASEDEV=DaniS506.ADD --+----------------++--- /A:n -+---+-------------++->

+-+- /!V -+------+	^	     +- /!R -------+

| +- /!W -+      |	|	     +- /P:hhh ----+

+--- /!AA -------+	+--------+   +- /IRQ:n ----+

+--- /PCLK:x ----+		 |   +- /!BM ------+

+-+- /GBM -----+-+		 |   +- /FORCEGBM -+

| +- /FORCEGBM + |		 |   +- /PF:xxxx --+

+--- /SHUTDOWN --+		 |   +- /80WIRE ---+

+--- /MGAFIX ----+		 |   +- /TO:nn ----+

+--- /DEBUG:n ---+		 |   +- /BAY ------+

+--- /!BIOS -----+		 |

+--- /PCS:n -----+		 |

+--- /!BM -------+		 |

+--- /PF:xxxx ---+		 |

>--------+----------------------------------------+------------------|

|				       ^

| +----------------------------------+ |

| +--/I---+ +----------------------+ | |

| V	  | V			   | | |

+-+- /U:n +--+-------------------+-+-+-+

+- /T:n ------------+

+- /!SMS -----------+

+- /!DM ------------+

+- /IT:x -----------+

+- /MR:udp ---------+

+- /PF:xxxx --------+

+- /NL:n -----------+

+- /!RMV -----------+

+- /!SETMAX --------+

+- /PROT0 ----------+

+- /!BM ------------+

In most cases, options prefaced with ! can be used with the ! modifier to

explicitly disable the option.  Options are valid both with and without

the ! modifer.

The values in parentheses are the default values if the option is not

present.

Options may be prefixed by "-" instead of "/" as well.

General options

---------------

/!V /VL /VLL					    (quiet)

(not) Verbose mode (less/lesser verbose)

/!W /WL /WLL					    (quiet)

same as /!V, but pauses screen

/!AA						    (beep)

No Audible Alerts

/PCLK:x						    (33MHz)

PCI Clock (DECIMAL)

/GBM						    (off)

Generic Busmaster Mode

/FORCEGBM						    (off)

Force Generic Busmaster mode

/!SHUTDOWN 					    (not)

(not) Shutdown each attached device

/MGAFIX						    (off)

Fix problem with Matrox drivers

/DEBUG:n						    (0)

Set debug level (DECIMAL)

/!BIOS						    (call BIOS

Disable all BIOS Int13 calls			       if required)

/PCS						    (0)

Support all found PCCard Slots

/PCS:n						    (0)

Support at most n PCCard Slots

/!BM						    (ATA  : enable)

(not) Enable BusMaster DMA on all adapters	    (ATAPI: enable)

/PF:xxxx						    (0)

Register access timings

Adapter options

---------------

/A:n						    (n/a)

Adapter number (0, 1, .. 7)

/!R						    (enable)

Disable adapter resets

/P:hhh						    (PCI, Snooper)

Base port address (HEX)

/PORT:hhh

same as /P

/IRQ:n						    (PCI, Snooper)

IRQ level (DECIMAL)

/I 						    (off)

Ignore adapter

/!BM						    (ATA  : enable)

(not) Enable BusMaster DMA			    (ATAPI: enable)

/PF:xxxx						    (0)

Register access timings

/80WIRE						    (autodetect)

80 wire cable present

/FORCEGBM						    (off)

Force Generic Busmaster mode

/TO:n (DECIMAL)					    (5s/dynamic)

Interrupt timeout

/BAY						    (off)

Adapter is connected to a drive bay

Unit options

------------

/U:n						    (n/a)

Unit number (0, 1)

/UNIT:n

same as /U

/I 						    (off)

Ignore unit

/T:n						    (5s)

Error retry time in seconds (DECIMAL)

/!SMS						    (autodetect)

Disable block mode

/SMS:n						    (autodetect)

Set SMS block length to n

/!DM						    (off)

Prevents OS2DASD.DMD from managing the device

/IT:x						    (BIOS)

Inactivity timer (DECIMAL)

/MR:udp						    (maximum)

Limit data rate of a device

/PF:xxxx						    (0)

Register access timings

/!RMV						    (device default)

(not) removable media drive

/NL:n						    (current device

noise level, n = 0 (faint) ... 126 (noisy)			setting)

/!SETMAX						    (enable unlock)

disable unlocking of full drive capacity

/PROT0						    (off)

enable write protection of first disk track

/!BM						    (ATA  : enable)

(not) Enable BusMaster DMA			    (ATAPI: enable)

Deprecated options

------------------

/FORCE

/ATAPI

/!ATAPI

/PCMCIA

Obsolete options

----------------

/GEO

/LBA

/!CYLA

/DC:n

/!DSG

/DT:x

/NEC

/PCLK:0

Undocumented options

--------------------

/LAT

/FIXES

/DEBUG

COMMAND LINE DETAILS

Beginning with release 1.0.8, this includes the undocumented switch

/!SHUTDOWN found in the latest IBM1S506.ADD. At system shutdown, this

IBM1S506 issues shutdown commands to each attached device. This behaviour

is different from earlier releases.

I've seen many reports about problems when the drives are shutdown. So I

decided to no longer be compatible with IBM1S506 here. DANIS506 *tolerates*

the option /!SHUTDOWN on the commandline, but it does nothing. If you want

this new shutdown behaviour (on notebooks perhaps), then add the new option

/SHUTDOWN.

Rule of thumb: if your system doesn't suffer from any (possibly unknown)

flaws, not a single command line option is required. The driver adapts

itself to the environment and runs at maximum performance.

In some situations, you may need some of the following options to achieve

optimum performance:

- ATAPI devices are initialized to PIO mode only if they appear to support

only very early ATAPI specifications; the same is true if the IDE chip is

known not to support PCI busmastering with ATAPI devices. In the former

case you may try to run particular ATAPI devices using busmaster DMA by

applying the proper /A:x /U:y /BM options.

- a few chips need the /80WIRE option to enable ATA66 operation (see below).

- if the DumpIDE utility detects an unsupported busmaster capable chip

and thus recommends to set /GBM (see below).

- you intend to hot-swap devices or use PCCard devices

-------------------------------------------------------------------------

For all of us who get the impression today's computers are way too fast to

keep up with, IBM did us the favour of adding another switch, /W (available

for more than one year now). This switch replaces the /V switch, and just

like /V, /W shows some very useful info about the EIDE system at boot time.

But instead of throwing all of this stuff onto the screen and scrolling it

off, /W writes it to the display in reasonable chunks and then *waits* for

the user to hit a key after he/she has learned it by heart ;-)

In order for /W to work you must have pausing on errors enabled in

CONFIG.SYS, either by "PAUSEONERROR=yes" or by no PAUSEONERROR setting at

all.

To reduce screen clutter and avoid possible SYS2068 messages, some lines

of the boot message may be suppressed by /VL (/WL) instead of /V (/W). If

you prefer even less info at boot time, /VLL (/WLL) will print out the

most basic messages only. Here is an example:

With /V or /W it looks like this:

Daniela's Bus Master IDE Driver for OS/2 Version 1.3.8

Controller:0  Port:01F0 IRQ:0E  Status:OK Bus Master DMA Scatter/Gather

PCI Rev: 2.10 VIA 571 PCI to IDE IC Rev:06 VID:1106 DID:0571 @33MHz

Unit:0 Status:OK ATAPI Device Bus Master DMA DMA1/PIO3

Model:IOMEGA  ZIP 250	   ATAPI	      51.G

Controller:1  Port:0170 IRQ:0F  Status:OK Bus Master DMA Scatter/Gather

PCI Rev: 2.10 VIA 571 PCI to IDE IC Rev:06 VID:1106 DID:0571 @33MHz

Unit:0 Status:OK ATAPI Device Bus Master DMA Ultra DMA2/PIO4

Model:Pioneer DVD-ROM ATAPIModel DVD-115  0108 E1.08

Unit:1 Status:OK SMS:16 LBA Bus Master DMA DMA2/PIO4 Identify

Model:CASTLEWOOD ORB2-E			      USR-4.08

OS2:log   phys BIOS:log  BPB:log  IDE:log   phys     Total Sectors

C    534   4273	      0 	      534   4273   Avail   4307184

H    128     16	      0        0      128     16   OS2	   4306176

S     63     63	      0        0       63     63   % Used    99.98

Controller:2  Port:A000 IRQ:09  Status:OK Bus Master DMA Scatter/Gather

PCI Rev: 2.10 CMD 649 PCI to IDE IC Rev:02 VID:1095 DID:0649 @33MHz

Unit:0 Status:OK SMS:16 LBA Bus Master DMA Ultra DMA4/PIO4 BPB

Model:IBM-DJNA-352030			      J58OA30K

OS2:log   phys BIOS:log  BPB:log  IDE:log   phys     Total Sectors

C   2482  39560	   1023 	    39560  16383   Avail  39876480

H    255     16	    255      255       16     16   OS2	  39873330

S     63     63	     63       63       63     63   % Used    99.99

Controller:3  Port:A800 IRQ:09  Status:OK Bus Master DMA Scatter/Gather

PCI Rev: 2.10 CMD 649 PCI to IDE IC Rev:02 VID:1095 DID:0649 @33MHz

Unit:0 Status:OK SMS:16 LBA NL:64 Bus Master DMA Ultra DMA5/PIO4 BIOS

Model:IBM-DTLA-307030			      TX4OA50C

OS2:log   phys BIOS:log  BPB:log  IDE:log   phys     Total Sectors

C  14890  59560	    620 	    59560  16383   Avail  60036480

H     64     16	     64        0       16     16   OS2	  60036480

S     63     63	     63        0       63     63   % Used   100.00

Controller:4  Port:B400 IRQ:09  Status:OK Bus Master DMA Scatter/Gather

PCI Rev: 2.10 HPT 370 PCI to IDE IC Rev:03 VID:1103 DID:0004 @33MHz

Unit:0 Status:OK SMS:16 LBA Bus Master DMA DMA2/PIO4 BPB

Model:QUANTUM FIREBALL_TM1280A		      A6B.2400

OS2:log   phys BIOS:log  BPB:log  IDE:log   phys     Total Sectors

C    621   2484	    524 	      621   2484   Avail   2503872

H     64     16	    255       64       64     16   OS2	   2503872

S     63     63	     63       63       63     63   % Used   100.00

Controller:5  Port:BC00 IRQ:09  Status:OK Bus Master DMA Scatter/Gather

PCI Rev: 2.10 HPT 370 PCI to IDE IC Rev:03 VID:1103 DID:0004 @33MHz

Unit:0 Status:OK SMS:16 LBA Bus Master DMA Ultra DMA2/PIO4 BPB

Model:ST34310A				      3.07

OS2:log   phys BIOS:log  BPB:log  IDE:log   phys     Total Sectors

C    524   8354	    534 	      524   8354   Avail   8420832

H    255     16	    128      255      255     16   OS2	   8418060

S     63     63	     63       63       63     63   % Used    99.97

But with /VL or /WL it's a little more comprehensive:

Daniela's Bus Master IDE Driver for OS/2 Version 1.3.8

Controller:0  Port:01F0 IRQ:0E  Status:OK Bus Master DMA Scatter/Gather

Unit:0 Status:OK ATAPI Device Bus Master DMA DMA1/PIO3

Model:IOMEGA  ZIP 250	   ATAPI	      51.G

Controller:1  Port:0170 IRQ:0F  Status:OK Bus Master DMA Scatter/Gather

Unit:0 Status:OK ATAPI Device Bus Master DMA Ultra DMA2/PIO4

Model:Pioneer DVD-ROM ATAPIModel DVD-115  0108 E1.08

Unit:1 Status:OK SMS:16 LBA Bus Master DMA DMA2/PIO4 Identify

Model:CASTLEWOOD ORB2-E			      USR-4.08

Controller:2  Port:A000 IRQ:09  Status:OK Bus Master DMA Scatter/Gather

Unit:0 Status:OK SMS:16 LBA Bus Master DMA Ultra DMA4/PIO4 BPB

Model:IBM-DJNA-352030			      J58OA30K

Controller:3  Port:A800 IRQ:09  Status:OK Bus Master DMA Scatter/Gather

Unit:0 Status:OK SMS:16 LBA NL:64 Bus Master DMA Ultra DMA5/PIO4 BIOS

Model:IBM-DTLA-307030			      TX4OA50C

Controller:4  Port:B400 IRQ:09  Status:OK Bus Master DMA Scatter/Gather

Unit:0 Status:OK SMS:16 LBA Bus Master DMA DMA2/PIO4 BPB

Model:QUANTUM FIREBALL_TM1280A		      A6B.2400

Controller:5  Port:BC00 IRQ:09  Status:OK Bus Master DMA Scatter/Gather

Unit:0 Status:OK SMS:16 LBA Bus Master DMA Ultra DMA2/PIO4 BPB

Model:ST34310A				      3.07

And finally, with /VLL or /WLL it becomes very terse:

Daniela's Bus Master IDE Driver for OS/2 Version 1.3.8

Model:IOMEGA  ZIP 250	   ATAPI	      51.G

Model:Pioneer DVD-ROM ATAPIModel DVD-115  0108 E1.08

Model:CASTLEWOOD ORB2-E			      USR-4.08

Model:IBM-DJNA-352030			      J58OA30K

Model:IBM-DTLA-307030			      TX4OA50C

Model:QUANTUM FIREBALL_TM1280A		      A6B.2400

Model:ST34310A				      3.07

No matter of the verbosity level which may suppress some lines on screen,

*all* messages are stored internally for later retrieval!

Beginning with beta 7, the driver saves all messages possibly shown at

boot-up internally. Users may redisplay these messages, or store them into

a file later with commands like these:

COPY IBMS506$ con:     or

COPY IBMS506$ anyfile

Please, check the spelling!

Some time ago, IBM decided to enable /SMS by default on most controller

hardware. The /!SMS switch then disables this controller based default. But

if the harddisk reports to be SMS capable, SMS is enabled regardless of the

/!SMS switch. Since this might cause sporadic failures on some hardware as

a few users reported, I changed the behaviour of the /!SMS switch to a full

override which is honoured in any case.

Sometimes it's not necessary to disable SMS mode, but just to limit the

number of sectors transferred in one run to a lower amount. To implement

this feature, the /SMS option is changed into /SMS:n, where n is the

maximum number of sectors in one run. If n is 0 or 1, this is equivalent

to /!SMS. Every harddisk has a maximum SMS run length defined by the

manufacturer specific to each model. You cannot set a value above this

individual maximum SMS run length. The driver sets up the actual SMS run

length for a specific drive to the minimum of the user supplied value n

and the maximum value reported by the drive.

-------------------------------------------------------------------------

Some notes on the /FORCE and /ATAPI options:

These options are no longer supported beginning with driver version 1.6.0;

the same is true with option /PCMCIA !

The related functionality is implemented now by means of the options /BAY

and /PCS with I hope to be easier to use.

-------------------------------------------------------------------------

To take advantage of Ultra DMA mode 3 (44MB/s), Ultra DMA mode 4

(66MB/s), Ultra DMA mode 5 (100MB/s), or Ultra DMA mode 6 (133MB/s),

*all* of these prerequisites must be true:

- the EIDE controller must support the ATA-66/100 enhancements

- the device must support the ATA-66/100 enhancements

- the cable must be a 80 wire type (recommended in any case)

- the motherboard must support the detection of the 80 wire cable (see #)

- the EIDE controller must report a 80 wire cable present

- the device must report a 80 wire cable present

# some chipset manufacturers still don't assign dedicated pins to enable

the detection of the cable type installed, and therefore don't define a

standard procedure for driver writers to acquire this *vital*

information. Any motherboard with a VIA596B, VIA686, or AMD756 south

bridge is affected by that: the ATA-66 features aren't enabled by default

in the driver because it has no way to detect the presence of a 80-wire

cable reliably. In this case add /80WIRE to the *adapter* options if

there is a 80-wire cable installed on that particular EIDE channel. This

decision is completely up to *you*. Don't blame me for lost or corrupted

data if you are wrong!

As a workaround to solve this inconvenience with the chips mentioned,

beginning with release 1.3.0, the driver checks the controller setup as

done by the BIOS. If it finds a particular channel initialized to ATA-66

operation it assumes the BIOS positively checked for the presence of a

80 wire cable.

-------------------------------------------------------------------------

The following options are features added by me:

-------------------------------------------------------------------------

Support for drive bays and PCCard slots

Many notebooks/laptops have so called drive bays built in to swap different

devices (floppy, disc, CD/DVD, etc.) while powered on state. The option

/BAY prepares the related IDE adapter for such a hot-swap. To make this

safely work on OS/2, the support for APM (by means of DEVICE=APM.SYS) has

to be installed. During APM suspend the device in the drive bay may be

swapped The floppy driver and the different ATAPI filters made by IBM check

the presence of appropriate devices after APM resume and operate

accordingly.

Beginning with release 1.1.9, DaniS506.ADD improves this hot-swap support.

After APM resume the driver inspects these "special" adapters and

reconfigures both the device and the controller exactly as it does at boot

time. It then reports its findings to interested filter drivers controlling

the units so that the filter driver can reconfigure itself appropriately.

At present, only DaniATAPI.FLT supports this advanced reconfiguration

protocol.

Example:

A notebook has got a drive bay with a swappable floppy drive, disk, ZIP,

and DVD-CDR/W. The following lines in CONFIG.SYS make all these devices

swappable and operational without powering down:

BASEDEV=IBM1FLPY.ADD /A:0 /FORCE:1 /U:0 /F:1.44MB

BASEDEV=DaniS506.ADD [other options] /A:1 /BAY  (see note #)

BASEDEV=DaniATAPI.FLT /A:1 /U:0 /TYPE:ZW

note #: this assumes the secondary channel to be connected to the drive bay

The ZIP drive and the hard disk show up as removable media unit in the

drives folder after a "refresh removable media" command. Important: before

swapping the units there *must* be an eject of the media (either by the

"EJECT x:" command from the command line or fro the context menu of the

drive icon)! Forgetting to do that may cause data loss because the file

system caches may not yet be written back to the media. With some luck the

command "ChkDsk x: /F" will repairs such damage without data loss.

Beginning with release 1.3.0, the driver supports PCCard slots as well.

This support will be enabled by means of the options /PCS or /PCS:n. With

/PCS the driver checks how many PCCard slots are available from the socket

services driver and creates the same number of "virtual" IDE adapters. If

less IDE adapters for PCCards shall be created, then the required amount n

may be set by the option /PCS:n. At most the corresponding number of

PCCards with IDE units attached may be inserted into the PCCard slots at

the same time.

A "virtual" PCCard IDE adapter is implemented by a PCCard (the actual IDE

controller) which is inserted into any one of the PCCard slots. Each

supported PCCard inserted *concurrently* counts as one IDE channel. The

driver attaches to OS/2's PCMCIA subsystem and takes over all PCCards with

a function id of 4 (disk) and function extension 1 (ATA interface); this

includes PCCard ATAPI devices as well (like the PCMCIA ZIP-250 or the

Click!40 drives). Beginning with release 1.3.10 both ATA and ATAPI devices

are supported (if DaniATAPI.FLT rev 0.3.0 or later is installed). PCCards

may be arbitrarily inserted to and removed from any PCCard socket. The only

prerequisite is a functional socket services driver and PCMCIA.SYS. Beware:

older revisions of IBM's PCMCIA subsystem have defects which may cause

failures to configure cards on sockets other than the first one.

After inserting a PCCard into the slot, you'll likely hear some noises from

the PCCard or its attached unit. Shortly after that you should hear a high

pitched beep. This beep indicates that the driver has accepted the PCCard,

has successfully assigned resource to it and initialized it, and was

successful communicating with the attached unit. If you don't hear this

beep, try reinserting the PCCard. A failed initialization will result in a

two times low pitched beep.

If you intend to remove a PCCard, don't forget to close the file system

on the attached device by issuing "EJECT x:" from the command line or the

drive's context menu. Failing to do so will leave the file system in an

inconsistent "dirty" state which most likely needs to be corrected by

CHKDSK to make the medium accessable again before the next use. "Eject"

doesn't necessarily unload the medium (f.e. in case of the Click! drive),

so don't be confused. Some drive activity will indicate the flushing of

still unwritten file system cache contents back to the medium. The end

of the flush operation is indicated by a middle pitched beep. Afterwards

it is safe to remove the PCCard from the socket or swap the medium.

Beginning with version 1.6.0 the driver handles PCCards from which the

system ist going to be booted from and which are already initialized by the

BIOS (for example the Sony Vaio Crusoe subnotebook with a bootable external

CD-Rom). To do that some technical prerequisites must apply (the PCCard

slot needs to be ExCA register compliant) and there must be no socket

services driver loaded; the latter one is needed only because of errors in

the socket services drivers which make booting from a BIOS initialized

PCCard impossible.

-------------------------------------------------------------------------

To use the inactivity timer feature simply add /IT:x to the *unit*

parameters. x is the timeout value in minutes. A 0 disables the spin down

completely. If you leave out this option, some devices will still

spin down after a certain default inactivity time.

For proper driver operation of this function, all of the following

should be true:

- in BIOS, APM must be enabled

- in BIOS, hard disk power down timers must be disabled

- in CONFIG.SYS, at least the statement DEVICE=APM.SYS must be present

(in other words: OS/2 APM support must be installed)

If these prerequisites are true, the driver can keep track of disk power

down even in case of intermittant system APM suspends. This ensures quick

error recovery by short command timeouts, but sufficiently long timeouts

for drive spin-ups.

If APM is not available in OS/2, the driver will prolong command timeouts

to 20s for each command if a drive reports active power down modes or

the user adds an /IT option; all other drives will be sent a command to

disable automatic spin-down.

-------------------------------------------------------------------------

To limit the data rate a device communicates at, add /MR:udp to the *unit*

parameters. Choose the values for u, d, and p from this table:

u: Ultra DMA

0  -  disable  Ultra DMA

1  -  limit to Ultra DMA mode 0

2  -  limit to Ultra DMA mode 1

3  -  limit to Ultra DMA mode 2

4  -  limit to Ultra DMA mode 3

5  -  limit to Ultra DMA mode 4

6  -  limit to Ultra DMA mode 5

d: Multi Word DMA

0  -  disable  MW DMA

1  -  limit to MW DMA mode 0 (in S506.ADD effectively the same as 0)

2  -  limit to MW DMA mode 1

3  -  limit to MW DMA mode 2

p: PIO

0  -  limit to PIO mode 0

1  -  limit to PIO mode 1 (in S506.ADD effectively PIO 0)

2  -  limit to PIO mode 2 (in S506.ADD effectively PIO 0)

3  -  limit to PIO mode 3 (with flow control)

4  -  limit to PIO mode 4 (with flow control)

If you set u and d to 0 at the same time, this effectively disables

busmastering on this device since no DMA mode is left to use. Leading

zeroes may be left out.

Many EIDE controllers don't have the capability to select PIO modes and

multiword DMA modes independantly because theses mode share the same timing

registers. This is for historic reasons and the similarity of cycle

timings. This applies to all controller chips mentioned above except for

Promise Ultra and Cyrix MediaGX.

Note: this applies to mixing transfer modes for the *same* device! The

devices on an EIDE channel are independent from each other regarding the

transfer modes (except for Intel 82371FB).

To deal with this situation the driver selects the slower cycle time from

the proposed PIO and MW-DMA modes. The net result can be seen from the

table below:

| no DMA    MW-DMA 1       MW-DMA 2	    Ultra-DMA x

------+----------------------------------------------------------------

PIO 0 |  PIO0    PIO0/DMA1      PIO0/DMA2	    PIO0/UDMAx	 (Intel,Promise,

|							  Cyrix)

PIO 0 |  PIO0    PIO0/no DMA    PIO0/no DMA    PIO0/UDMAx	 (others)

------+----------------------------------------------------------------

PIO 3 |  PIO3    PIO3/DMA1      PIO3/DMA2	    PIO3/UDMAx	 (Promise,Cyrix)

PIO 3 |  PIO3    PIO3/DMA1      PIO3/DMA1	    PIO3/UDMAx	 (others)

------+----------------------------------------------------------------

PIO 4 |  PIO4    PIO4/DMA1      PIO4/DMA2	    PIO4/UDMAx	 (Promise,Cyrix)

PIO 4 |  PIO4    PIO3/DMA1      PIO4/DMA2	    PIO4/UDMAx	 (others)

-------------------------------------------------------------------------

Some users are forced (or feel themselves forced ;-) to run the PCI bus on

their motherboards at speeds other than the optimum 33MHz. For example,

many Cyrix processors need very uncommon front side bus speeds which cannot

be easily divided into 33MHz. This leads to either too slow EIDE timings or

too fast ones. The former case may be disappointing, but the latter

overclocks the EIDE interfaces and my introduce data distortions. If you

know the actual PCI clock of your motherboard, you may add an additional

driver option /PCLK:x (x is the PCI clock in MHz). The user supplied PCI

clock is then sorted into one of four speed classes (25MHz, 33MHz, 37MHz,

and 41MHz) which is reported at boot time. For each of these speed classes,

the driver *tries* to select the optimum chip timings, but due to chipset

limitations this is not possible in all cases.

As most motherboards run their PCI bus at the optimum 33MHz clock today,

you should use this switch only if really necessary!

The technical background of this option:

The so called Front Side Bus (FSB) is the bus the CPU communicates with the

north bridge (which contains the memory controller, the cache controller,

the AGP bridge, the PCI arbiter, the PCI bridge, and optionally some more

devices). Depending on the design of the north bridge, the PCI clock is

derived from the FSB clock by dividing this clock (synchronous design) by

either an integral number (2, 3), or a fractional number (2, 2.5, 3), or

it is derived from an independent clock source (asynchronous design, very

uncommon). The PCI 2.1 spec (without its extensions) allows for 33MHz max.

PCI clock.

The VIA, the ALi, and most likely the SiS chipsets are capable of

fractional dividers. The Intel chipsets are not.

If you know, your PCI bus runs at the typical 33MHz clock (true for most of

the mobo/CPU combinations sold today), don't apply this switch.

If you know the PCI clock in advance (or like to tune the EIDE timings

for some reason), you apply the /PCLK:n switch where n is the known PCI

clock. The driver then selects the speed class from this value in the

same way as from an autodetected value.

These speed classes are the result of the typical FSB clocks and dividers:

FSB clock / divider -> PCI clock -> class

50MHz    /   2	 ->  25MHz    -> 25MHz

60MHz    /   2	 ->  30MHz    -> 33MHz

66MHz    /   2	 ->  33MHz    -> 33MHz

75MHz    /   2	 ->  37.5MHz  -> 37MHz

75MHz    /   2.5	 ->  30MHz    -> 33MHz

75MHz    /   3	 ->  25MHz    -> 25MHz

83MHz    /   2	 ->  41.5MHz  -> 41MHz

83MHz    /   2.5	 ->  33MHz    -> 33MHz

95MHz    /   3	 ->  31.7MHz  -> 33MHz

100MHz    /   3	 ->  33MHz    -> 33MHz

112MHz    /   3	 ->  37.3MHz  -> 37MHz

124MHz    /   3	 ->  41.3MHz  -> 41MHz

For the chosen speed class, the driver tries to select EIDE controller

register values which don't exceed the PIO/DMA/Ultra ratings. My former

driver revisions and IBM1S506 only know the proper register values for the

33MHz class. This runs the EIDE interface slower than possible in case of a

25MHz PCI clock, but exceeds the allowed timings in the 37MHz/41MHz cases.

The former one is uncritical, but the latter one may cause failure of

operation (better) or data corruption (worse).

To give you an example how to use command line switches, look here:

BASEDEV=DaniS506.ADD /V /A:0 /U:1 /MR:223 /IT:5 /A:1 /I

This shows some info at boot time, limits the data rates to communicate

with the primary slave to UDMA1/MWDMA1/PIO3 and stops the spindle motor

after 5 minutes of inactivity on this device (which happens to be the

IBM-DTTA351010 drive). It tells the driver also not to touch the secondary

channel (adapter 1).

-------------------------------------------------------------------------

The /GBM option enables the scan for busmaster capable PCI EIDE controllers

if none of the 'well-known' chipsets (currently INTEL, VIA, ALi, SiS, CMD,

Promise, HighPoint, AMD, SMSC, Artop, ServerWorks, CMD640, and RZ1000, the

latter two are not busmaster capable!) is found. Before a chip is accepted

by the driver to be operated in generic busmaster mode, it has to pass some

sanity checks. Some BIOSes simply fail to initialize the controller appro-

priately (seen on a board with an UMC8886BF). A rule of thumb: if Linux

doesn't run a chip in DMA mode, it is not generic busmaster compatible.

At present, EIDE controllers not supported by any S506 ADD run attached

devices in PIO mode only, and with timings setup by the BIOS (most probably

PIO0 if the device is hidden from the BIOS). The generic busmaster option

enables usage of the busmaster machinery and device DMA modes, thus

reducing CPU load by a large amount. In this mode, the ADD doesn't touch

any chip registers other than the standard PCI command register (to enable

PCI busmastering) and the registers associated with the busmaster engine.

It has *no* way to enable Ultra-DMA modes in hardware if not done by

the BIOS.

If you think this should be the default behaviour of my driver, then

simply change the /GBM option into the /FORCEGBM option. Then, *every*

EIDE chip is treated as if it's not a 'well-known' chip. All programming

of device timings in the EIDE controller has to be done by the BIOS.

/FORCEGBM comes in two flavours, as a global option, and as an adapter

option. If /FORCEGBM preeceds all /A:x options, it applies to all adapters;

if it follows an /A:x option, it applies to adapter x only.

In some cases, /FORCEGBM can help to overcome stability problems because

of the spec-conforming timings that the driver programs into the controller

registers. In the recent past, some batches of boards popped up which are

"tuned-down" by their BIOS to work reliably. In these cases, the "correct"

timings are too fast and the boot process fails if these timings are set up

by DaniS506.ADD. Adding /FORCEGBM prevents the driver from adjusting the

timings so that the "special" BIOS timings are used instead.

Examples:

BASEDEV=DaniS506.ADD /W /GBM /A:0 /U:0 /MR:023

BASEDEV=DaniS506.ADD /W /FORCEGBM /A:0 /U:0 /MR:023 /A:0 /U:1 /MR:034 /IT:5

-------------------------------------------------------------------------

There is a new option /PF:xxxx which may be applied to an adapter or

individual units. This option is meant to fine-tune register access timings

and enable chipset-performance features. The option value is a bit field,

whose individual bits need to be put together into the final option value.

Bits 0-7 are meant for all PCI chipsets, bits 8-15 are chipset specific.

In this driver release, the following settings are available:

+---+---+---+---+---+---+---+---+---+---+---+---+---+---+-------+

|15  14  13  12  11  10	9   8 | 7   6	5   4	3   2	1   0 |

+---+---+---+---+---+---+---+---+---+---+---+---+---+---+-------+

| 	 unused 	      |        unused	      |  (1)  |

+---+---+---+---+---+---+---+---+---+---+---+---+---+---+-------+

1) Register access delay:

0 : 1000 ns  (default, fixed for all non-PCI systems)

1 :  500 ns

2 :  250 ns

3 : no delay (works on most systems, check out yourself)

The actual access delay used for a particular adapter channel is the

largest of all attached units!

BASEDEV=DaniS506.ADD /A:2 /PF:2 /U:0 /PF:3

If there is no slave device, then no register access delays are incurred

because the individual unit setting /PF:3 overrides the adapter setting

/PF:2. But if there is an additional slave device, this unit inherits the

adapter option /PF:2. The largest delay of both units is the resulting one

and thus each register access is delayed by 250ns.

-------------------------------------------------------------------------

Modern ATA spec 6 compatible drives may support the so called acoustic

management feature set. Using this you can trade off performance vs. noise

(mostly noticable during quick sequences of random seeks). The new unit

option /NL:n allows you to control this feature: the value n may be set to

0 ... 126, where '0' selects the faintest noise level with the biggest

performance impact, while '126' selects best performance at the expense of

noisiest operation. Drives are not required to support any value in

between, but to select the next supported one. Drives store the latest

programmed value internally and preserve it over power outages and resets.

The driver will tell the current value read from the drive if you don't

use /NL option, or the user selected value n if /NL:n is present.

-------------------------------------------------------------------------

If you have a removable drive which you want to boot from, this reportedly

fails. To enable booting from this unit it has to be treated as a fixed

one. This can be done with the option /!RMV applied to the proper unit.

It is reported also that some Syquest drives have buggy firmware which

doesn't report properly the support of the removable feature set. To

announce these drives to OS/2 a removable drives add the /RMV option to

the appropriate unit.

-------------------------------------------------------------------------

If there is a PCI EIDE controller in your system, and you happen to

have a Matrox video board, then the boot process might just stop at a

certain point or fail with a trap 8. This is due to a horribly nasty

behaviour of the Matrox driver wich scans all ports with addresses Cx0yh

(x = 0..F, y = 0..3) at initialization of the base video handler even if

these addresses are not assigned to the video board. I wouldn't call it

playing by the rules beating the bush and looking if some sort of MGA

drops out of it.

If any other PCI device gets assigned addresses matching the above pattern,

*anything* can happen. To work around this problem, I have added a new

option /MGAFIX which detects EIDE hardware with addresses affected by that,

and tries to push them away a little by reprogramming the address decoders

so that they are no longer potential scan targets.

Caution: this is a last-resort option. Use it *only when necessary* !

If you can boot without this option, or cannot boot with this option,

remove it! Its side effects may cause other problems.

-------------------------------------------------------------------------

Beginning with gamma 4 I introduced an additional 'safety net'. Some

hardware tells to be perfectly capable of doing DMA transfer operations but

fails miserably when it comes to a real life test. In such a case booting

will succeed eventually but with a retry in PIO mode for each chunk of data

(and a very upset driver beeping all of the time).

To prevent such an outcome, at the very end of driver initialization an

additional check is carried out. If a device was elegible for DMA

transfers, a DMA mode was chosen and the device commanded to do so, it is

tested for a successful mode switch. If this test turns out positive and if

the device is a harddisk, a read operation with the selected DMA mode and

very tight timings is issued. If this succeeds as well, then the driver

assumes busmaster DMA is safe to be used. If either test fails, busmaster

DMA is shut down for the affected device and PIO mode transfers are used

instead. This is *not* reflected in the messages shown at boot time;

'DiskInfo v' tells the actual device transfer mode after bootup.

Oddly enough, some devices do in fact switch into the correct DMA transfer

mode, but don't show the selected DMA mode in their INQUIRY data: some tell

there is no DMA mode selected at all, others tell multiple DMA modes to be

selected. The driver then (correctly) assumes something is wrong with this

device and - as described above - denies busmaster DMA despite the fact

that the device can do DMA. If one of your devices fits into this category

then override the 'safety net' by applying /BM to the affected unit.

-------------------------------------------------------------------------

One user reported a boot failure in a particular situation which involved

SCSI and ATA/ATAPI units. The solution was to avoid BIOS INT13 calls. The

driver asks the BIOS about the geometry properties of the ATA drives. In

general, this information is not required because the most authorative

source of the geometry info is the drive itself (by inspecting the MBR and

BPBs). This implies that no BIOS INT13 calls are required in almost any

scenario. The only exception is if your disk is completely blank, and you

want to partition this blank disk by OS/2 FDISK with a geometry compatible

to the BIOS one.

So, if you prefer to deny the driver calling the BIOS, add the option

/!BIOS. When doing so, the "strange" VDM seen with some process management

utilities is gone as well.

-------------------------------------------------------------------------

Later versions of FDISK and LVM tend to overwrite bootmanagers other than

OS/2 BootManager on every write operation to the partition/LVM tables.

In such cases it might be appropriate to write protect track zero of the

boot drive (which contains the bootmanager code). This is possible either

at bootup using the unit option /A:x /U:y /PROT0 ("PROT zero"), or later

by means of the DiskInfo utility:

DiskInfo <unit> p 1	enable write protection on <unit>

DiskInfo <unit> p 0	disable write protection on <unit>

"DiskInfo ?" shows all options in detail.

-------------------------------------------------------------------------

Todays large disk drives often don't work properly with older BIOSes.

Either the BIOS detects the wrong disk capacity, or sometimes the PC

doesn't boot at all. In such cases, the cure of the problem is either to

set a jumper at the disk or reprogram the disk such that it reports a

smaller, troublefree capacity to the BIOS.

Beginning with version 1.3.8 the driver is able to use the full disk

capacity even if the disk is programmed to report a smaller capacity. This

is done automatically; if you don't like that then you have to prevent the

unlocking of the full capacity by means of the unit option /A:x /U:y

/!SETMAX.

To program a disk to report a smaller capacity, I recommend this process:

1) remove the disk from the BIOS setting. The PC will no langer hang.

2) boot from an OS/2 boot disk (or the like).

3) program a smaller disk capacity by means of the Diskinfo utility:

DiskInfo <unit> m <new capacity>,  f.e.

DiskInfo 0m m 32G

For details see "DiskInfo ?"

4) reboot

5) if you like you may add the disk to the BIOS settings again

INSTALLATION

For an upgrade from a previously installed IDE driver (most likely

IBM1S506.ADD) just copy DaniS506.ADD to bootdrive:\OS2\BOOT and rename

IBM1S506.ADD (or the name of the previous IDE driver) to DaniS506.ADD in

your CONFIG.SYS.

For a first time install of an IDE driver you'll have to add this line

to CONFIG.SYS:

BASEDEV=DaniS506.ADD

In this case your boot device probably is SCSI. To avoid confusion make

sure to move the mentioned line *below* the SCSI driver.

If you prefer chasing icons, you can do the copy and the modification of

CONFIG.SYS using DDINSTAL ("OS/2 System"/"System Setup"/"Install/Remove"

/"Device Driver"). Due to limits in DDINSTAL you have to disable the

previously installed S506 base driver in CONFIG.SYS manually, either by

deleting that line or putting a 'rem' statement in front of it.

FILES

DaniS506.ADD  The driver itself.

DaniS506.DDP  The device driver profile for DDINSTAL

DaniS506.DOC  The file you are reading right now :-)

DumpIDE.EXE   A utility to dump the current IDE registers

DiskInfo.EXE  A utility to show some information about the attached devices

\nodebug\DaniS506.ADD

A smaller driver version without debug traces

\nodebug\cmprssd\DaniS506.ADD

The same as above, but LXlite compressed. This one is *not*

guaranteed to work on every system.

\source\DiskInfo.c

Guess ;-)

BUGS

Hopefully none, but who knows ...

!!!  Bug reports without IDE configuration and transfer modes involved, and

!!!  without the output from the dump utility, are pretty useless. You should

!!!  create these outputs in a running *and* the faulty system. The easiest way

!!!  to do this is to redirect the screen output from the dump utility to a file

like this:

DumpIDE > Good.dmp   or   DumpIDE > Bad.dmp

Many people refer to driver revisions by file dates. Don't do that for

these reasons:

- I don't keep track of file dates

- I build several drivers almost each day

- file dates can easly be changed

- there are many different calendars in this world

The only valid reference is the driver's version number. It is shown on

boot up if the /V or /W switch is given. If you can boot to a command line

!!!  the output of "COPY IBMS506$ anyfile" will list the driver version in

!!!  the first line. If you cannot boot, then run "BLDLEVEL DaniS506.ADD" which

will output something like this:

Signature:       @#DANI:1.2#@##1## 4.7.2000 10:45:10 ....

Vendor:	      DANI

Revision:	      1.02

Date/Time:       4.7.2000 10:45:10

Build Machine:   Nachtigall

File Version:    1.2.5   <------------------------------+

Description:     Adapter Driver for ST506/IDE DASD      |

|

This is the full version number  -----------------------+

I have no means to identify traps occuring outside this driver (module name

different from 'IBMS506$' or not present). If a trap occurs within this

driver, I need the following information: CS:EIP, EAX-EDI, DS and ES. My

test systems at home (VIA and ALi) and a couple of other machines at work

(different INTEL chips) run flawlessly with this driver, so I have

virtually no chance to reproduce errors.

If any of your EIDE devices is not detected at boot-up, please contact me.

The driver has special debug code built in; I will tell you how to enable

and gather these traces.

If you experience problems with your hard disks after OS2DASD.DMD has been

loaded (to be seen by pressing ALT-F2 when the white box appears on the

screen), read the following section *very carefully*, it applies to you.

I cannot solve your hardware problems with software. If you think your

problems arise from bugs in the EIDE controller chip, you may contact me

after gathering detailed info about this bug and its software solution.

At present, I have a recent chip bug list from ALi and another one for

the VIA 586 chipset created by myself.

SOME WORDS OF WISDOM

Much EIDE hardware you can buy (or already have bought) has *very*

little design margins and is *very* susceptable to even the slightest

signal distortion on the cables. Unlike SCSI, the EIDE bus has no

termination resistors on both ends of the cable to reduce ringing or

other signal reflections. Noise and crosstalk are other sources for

bit errors during data transmission; careful grounding (both the data and

the power connection) is paramount if you intend more than simple PIO

speed. Power supplies are not uncommon for inducing noise!

A very patient user found another source of data distortions after some

very long and tedious reliability tests: the cache subsystem of his

motherboard. He experienced bit errors which vanished after some fiddling

within the BIOS settings of the cache timings (pipelining was the culprit

in this case). This isn't something magic! Consider, a busmaster -

especially when running at high transfer rates - puts an additional heavy

burden on the cache logic in the chipset and the cache RAMs. Many random

failures of OS/2 systems - even without busmastering of EIDE or SCSI

controllers - have their origin here.

If you think your data is trashed due to bugs in this driver, please

take this into account:

I made a lot of changes to IBM's source code in the driver initialization

portion to support ALi, SiS, VIA, CMD, and Promise chipsets, and program

spindown timers in the devices, and to select data transfer rates other

than the fastest ones. Some of these changes could lead to traps during

bootup depending on the actual parameter values used. I hope to have them

fixed. These traps (identifiable by the module name 'IBMS506$') cannot

corrupt any of the data because they happen before any write operation is

issued to any drive. I'm sorry if you happen to have such an inconvenience.

BUT: the code to carry out read and write requests either by PIO or

busmaster assisted DMA is *unchanged*. In fact, all data transfers from the

device into host memory or vice-versa are done 100% by the drive

electronics and the chipset hardware on your motherboard (or 99% plus *one*

CPU instruction in case of PIO). Driver code only steps in when any part of

this hardware reports an error or wasn't able to fulfill the request.

Keep in mind: PIO and MWDMA transfer modes have no means of detecting bit

errors during data transfers, Ultra-DMA does CRC checking to maintain data

integrity.

So, instead of rushing to the fastest data transfer modes possible, limit

the negotiated data transfer speed to the slowest available in each

category (/MR:120). Using PIO4 on other than rock solid hardware which

passed reliability tests is *very* dangerous. Since large portions of the

OS2*.INI files cannot be flushed to disk at shutdown with error-checked

Ultra-DMA, the ADD has to revert to the error-prone PIO mode. I consider

this an OS/2 design flaw.

If you run SYSBench or DISKIO, and experience unstable or rather low

performance figures in the maximum data transfer rate test, you are seeing

the effects of retries and interface resets. This is an indication of bit

errors!

As a reference, in the DISKIO 'Drive cache/bus transfer rate' test you

should see the following results:

Ultra-DMA mode 5 : 66MB/s or more

Ultra-DMA mode 4 : 44MB/s or more

Ultra-DMA mode 3 : 34MB/s or more

Ultra-DMA mode 2 : 24MB/s or more

Ultra-DMA mode 1 : 17MB/s or more

Ultra-DMA mode 0 : 13MB/s or more

MW-DMA mode 2    : 11MB/s or more

MW-DMA mode 1    :  9MB/s or more

SYSBench is based on the DISKIO code with some modifications. DISKIO does

the worst case test on the last cylinder (falsely called track in both

programs) using forward reads, SYSBench uses backward reads. Remember,

different drives have different cache strategies built in, and reading

backwards is a pretty uncommon access strategy with file system caches, so

some drives are fooled in this test and the result is overly pessimistic.

If you like to tune the data transfer speed on the cables to your drive,

then run DISKIO or SYSBench without DMA rate limitation and look at the

results reported for cylinder (track) 0. This is the maximum data transfer

rate your drive is able to read off the media surface. Then select a

transfer mode which matches this speed. I recommend Ultra DMA modes over

MW-DMA modes. If the drive is DMA capable, select PIO mode 0 as non-DMA

mode (for the rare cases where busmaster DMA cannot be used).

If you see false device detects or even lockups at boot time, don't

expect reliable data transfers thereafter.

In any case, if you think you are experiencing data corruption, look at the

output generated by the /W or /V switch, and then look for the data

transfer modes used with your device. The first remedy to try will be

disabling all fast transfer modes (by /!BM and/or /MR:).

If you hear short beeps, then the data transfer state-machine encountered

an exception: a high beep indicates a UDMA CRC failure, a low beep an error

reported by the hardware which leads to a interface reset eventually,

and a middle beep indicates a command timeout (usually caused by a

spun-down harddisk). So, if your hardware works 'flawlessly' you will hear

*no* sounds from the driver (except for spin-down events of course). With

'flawlessly' I mean 'without errors detectable by any instance'. If you

prefer to ignore hardware errors indicated by these beeps, add the

/!AA (no audible alerts) option.

To protect the current working desktop from corruption, consider creating

test copies of the x:\Desktop directory tree and the OS2*.INI files

and pointing to these in a modified copy of your CONFIG.SYS located

in x:\OS2\BOOT. For example, rename this copy to CONFIG.T and select

this test configuration within the recovery choices screen by hitting

the 'T' key. This way, you always have a working default boot system

and a independent test system to play with.

QUESTIONS & ANSWERS

1) Both my EIDE controller and my harddisc is ATA/66 (or ATA/100) capable,

and a 80wire cable is installed. No matter what I am trying to do (reading

the docs, applying the /80WIRE override switch), the driver allows no more

than UltraDMA mode 2. What's wrong ?

Most likely, your harddisk is setup to hide its ATA/66 (or ATA/100)

capabilities. Some models are delivered with such a factory setup to

prevent confusion of older BIOSes or controllers. Run "DiskInfo i" and look

at word 88/058 reported from the disk in question. If it it something like

xx07, the disk reports ATA/33 capability; if it is xx1F, it can do ATA/66,

and if it is xx3F, then ATA/100 is available. If the reported capability is

less than advertised, contact the disk manufacturer for a utility to enable

the missing capabilities (often to be found on their web sites).

2) I experience random failures after wakeup from suspend. What can I do ?

Toggle the "Delayed Transaction" setting at the PCI page in the BIOS. Some

hardware doesn't implement this feature correctly.

3) I have a large (> 33GB) disk but FDISK or LVM report only a fraction of the

true disk capacity. Where does that come from ?

Run "COPY IBMS506$ con:" and look at the geometry values (example):

Model:WDC WD400BB-00AUA1 		     18.20D18

OS2:log   phys BIOS:log  BPB:log  IDE:log   phys     Total Sectors

C   4865  65535	  16383 	    65535  16383   Avail  78165360

H    255     16	     16      255       16     16   OS2	  78156225

S     63     63	     63       63       63     63   % Used    99.98

If the "OS2:log" column shows a C value of 65535 then there is most likely

a 16 bit overflow due to a badly chosen translation mode. Presumably, the

H value will be 16 instead of the much more appropriate 255 (or 240).

If you see the same H value in the "BPB:log" column then this is the

translation geometry stored on the disk itself, in the partition table

and/or the BPB sectors. In this case you *must* zero out the partition

table to get rid of this unwise translation mode, a mere change in the BIOS

settings is not sufficient!

If there are only zeroes in the "BPB:log" column, then the disks doesn't

have a valid partition table yet and the translation mode is taken from the

BIOS. Change the BIOS setting to "LBA translation" mode.

4) I have problems reading CDs now which was working fine with earlier driver

versions. Why ?

Later drivers enable DMA modes on ATAPI devices now if the seem to follow

sufficiently recent ATAPI implementation standards. Some ATAPI units still

fail running on DMA, so you have to disable DMA busmastering for these

units (/A:x /U:x /!BM).

CREDITS

I'd like to express my thanks to some outstanding people who made this

piece of code a better one:

Helen Terbizan   for her neverending efforts to make ALi support possible

and her patience and perseverance during the tests.

Mark Dodel       for just the same patience and perseverance.

Jens Glathe      for his helpful tips and the courage to test the SiS code.

Piotr Oniszczuk  for his persistent reliability tests.

Fred Springfield for his debugging of this pile of words.

Gerald Eischer   for creating the command line options diagram and table

TO DO

***  Still left is to persuade IBM to incorporate these features

***  in their distribution.

***  done! IBM has officially taken the v1.6.6 sources.

COPYRIGHT

DaniS506 - Copyright (c) 1999-2003 Daniela Engert. All rights reserved.

STATUS OF PROGRAM

DaniS506 is freeware.

DISCLAIMER

YOU ARE USING THIS PROGRAM AT YOUR OWN RISK! I don't take any

responsibillity for damages, problems, custodies, marital dis-

putes, etc. resulting from use, inability to use, misuse, pos-

session or non-possession of this program directly or indirectly.

I also don't give any warranty for bug-free operation, fitness for

a particular purpose or the appropriate behaviour of the program

concerning animals, programers and little children.

THE SOFTWARE IS PROVIDED ``AS IS'' AND WITHOUT ANY EXPRESS OR

IMPLIED WARRANTIES, INCLUDING, WITHOUT LIMITATION, THE IMPLIED

WARRANTIES OF MERCHANTABILITY AND FITNESS FOR A PARTICULAR

PURPOSE. THE ENTIRE RISK AS TO THE QUALITY AND PERFORMANCE OF THE

PROGRAM IS WITH YOU.  SHOULD THE PROGRAM PROVE DEFECTIVE, YOU

ASSUME THE COST OF ALL NECESSARY SERVICING, REPAIR OR CORRECTION.

Or, in a few words:

If its good, I am responsible.

If its bad, its all your fault. ;)

Permission is granted to redistribute this program free of charge,

provided it is distributed in the full archive with unmodified

contents and no profit beyond the price of the media on which it

is distributed is made. Exception to the last rule: It may be

included on freeware/shareware collections on CD-ROM, as well as

on magazine cover CD-ROMs.

All trademarks mentioned anywhere around here are property of their

owners and the like ...

AUTHOR

Daniela Engert

Internet: "Daniela Engert" <dani@ngrt.de>        (preferred)

Fidonet: 2:2490/2575 (no debug support here)

EPILOGUE

Suggestions and bug-reports are always welcome. Well ... bug-reports

are perhaps not *that* welcome ... ;)