STANDARD MICROSYSTEMS CORPORATION

IBM PC LAN Program Environment

For Ethernet LAN Adapters

INSTALLATION GUIDE

This software is licensed by SMC for use by its customers only.

Copyright (c) 1993 Standard Microsystems Corporation.

All rights reserved.

This installation guide describes:

o  How to use EZStart to copy the IBM PC LAN program (PCLP)

environment driver from the SuperDisk to your PC.

o  How to install the IBM PC LAN program environment driver on

a workstation and a PC.

COPYING THE DRIVER FILES

From the EZStart Driver Installation screen, select 'Copy Files'

to copy all the files used by this driver from the SuperDisk to the

Destination Directory recommended by EZStart, or to a different

directory that you specify.

The EZStart default Destination Directory is a:\NDIS.  Please make

a backup copy of your SuperDisk before EZStart copies files to the

SuperDisk to create an "option" diskette.

INSTALLING PC LAN PROGRAM WORKSTATION

1. At the DOS prompt, insert the SMC or IBM LAN Support Program (LSP)

Version 1.30 diskette.

2. Type: DXMAID

3. Press the Enter key three times.

The Environment Information screen appears.

4. Specify the following values for the fields on this screen:

Configuration for this computer?                Yes

Use existing configuration information?         No

Do you have adapter option diskettes?           Yes

Are you configuring for two adapters?           No

Source for LSP                                  A:\

Target for new configuration                    C:\lsp

5. Press Enter to store your choices.

The program prompts you to install the "option" diskette.

6. Insert the SuperDisk in your source drive and specify

the path: A:\NDIS.

7. Press Enter.

The necessary files are copied from the SuperDisk to your target

directory on your computer's hard disk.

8. Press Enter.

9. Insert the LSP diskette and press Enter.

The Current Configuration screen appears with "Western Digital

EtherCard PLUS Family LAN Adapters." Note, however, that the

correct driver will be installed.

10. Press F4. LSP is installed.

11. Use any text editor to edit the CONFIG.SYS file. Specify at least

20 files in the FILES= line; for example:

files=20

Once you save your CONFIG.SYS file, reset your computer.

12. Insert the PC LAN Install Diskette in your computer's diskette

drive.

13. Type: INSTALL

14. Press Enter.

15. Press Enter three more times.

16. Select the level of function for this workstation, either "Base

Services Only" or "Extended Services".  Most of the time, select

the second option.

17. Specify "Workstation".

18. Specify either "Send messages" or "Send, receive, and log

messages" options. Most of the time, select the second option.

19. Specify the target path for the PC LAN files. The default

pathname is C:\DOS.

20. Specify the machine's ID and the domain ID. The domain ID must

be the domain server ID and must be the same for all workstations

on the network.

21. Follow the prompts to complete the installation until you reach

the Change System Configuration File screen. Usually, you should

accept the default for the system configuration file.

The Change System Batch File screen appears. Usually you should

accept the default.

INSTALLING PC LAN PROGRAM SERVER

The following section provides instructions for installing PC LAN

on a server.  This procedure uses the LAN Support Program (LSP)

to facilitate the installation process.

1. At the DOS prompt, insert the LSP diskette.

2. Type: DXMAID

3. Press Enter.

4. Press Enter twice.

The Environment Information screen appears.

5. Specify the following values for the fields on this screen:

Configuration for this computer?                Yes

Use existing configuration information?         No

Do you have adapter option diskettes?           Yes

Are you configuring for two adapters?           No

Source for LSP                                  A:\

Target for new configuration                    C:\lsp

6. Press Enter to store your choices.

The program prompts you to install the "option" diskette.

7. Insert the SuperDisk in your source drive and specify

the path: A:\NDIS.

8. Press Enter.

The necessary files are copied from the SuperDisk to your

target directory on the computer's hard disk.

9. Press Enter three times.

10. Insert the LSP diskette into your computer's disk drive.

11. Press Enter.

The Current Configuration screen appears with the SMC Ethernet

adapter and protocols already selected.

12. Press F4. LSP is installed.

13. Use any text editor to edit the CONFIG.SYS file. Specify at

least 20 files in the FILES= line; for example:

files=20

Once you have saved the CONFIG.SYS file, reset the computer.

14. Insert the PC LAN Install Diskette in your computer's disk drive.

15. Type: INSTALL

16. Press Enter.

17. Press Enter three more times.

18. Select the level of function for this workstation: either "Basic

Services Only" or "Extended Services."  Most of the time, select the

second option.

19. Specify "Server".

20. Specify either "Send messages" or "Send, receive, and log

messages" options. Most of the time, select the second option.

21. Specify the target path for the PC LAN files. The default

pathname is C:\DOS.

22. Specify the machine's ID and the domain ID. The domain ID must

be the domain server ID and must be used by all workstations

on the network to communicate with the server.

23. Follow the prompts to complete the installation until you reach

the Change System Configuration File screen. Usually, you should

accept the default for the system configuration file.

The Change System Batch File screen appears. Usually, you should

accept the default.

SMC and Standard Microsystems are registered trademarks; and

EtherCard Elite 16 Ultra, EtherCard Elite 16T Ultra, EtherCard

Elite 16C Ultra, Elite Series, EliteView, EZStart, SuperDisk, and

UltraChip are trademarks of Standard Microsystems Corporation.

Other products and company names are registered trademarks or

trademarks of their respective holders.