PCM2WAV Short documentation

PCM2WAV replaces WAV2ZVD and has (at least) the same capabilities.

pcm2wav [option] [option] [...]  Jul  2 2000, Version 0.05c,

Options: -i<source filename(s)>

-o<path [and name] for target>

-s<samplerate [Hz] for target>

-k<Type of conversion>

0 ..... any known format to RIFF PCM WAV

1 ..... WAV to 2 Bit Zyxel ADPCM

2 ..... WAV to 3 Bit Zyxel ADPCM

3 ..... WAV to 4 Bit Zyxel ADPCM

9 ..... WAV to 4 Bit Zyxel DVI ADPCM

10 .... WAV to Zyxel 8 Bit PCM A-Law

11 .... WAV to Zyxel 8 Bit PCM �-Law

*          21 .... WAV to 2 Bit Rockwell ADPCM

*          22 .... WAV to 3 Bit Rockwell ADPCM

*          23 .... WAV to 4 Bit Rockwell ADPCM

24 .... WAV to 8 Bit Rockwell PCM

31 .... Raw 2 Bit Rockwell ADPCM to RIFF WAV

32 .... Raw 3 Bit Rockwell ADPCM to RIFF WAV

33 .... Raw 4 Bit Rockwell ADPCM to RIFF WAV

100 ... Raw 8 Bit WAV to RIFF WAV

200 ... Raw 16 Bit WAV to RIFF WAV

300 ... Raw 16 Bit CD Audio to RIFF WAV

301 ... WAV to Raw 16 Bit CD Audio

**         400 ... Raw USR GSM to RIFF WAV

**         401 ... WAV to GSM (XTreem/VoiceConnect)

**         402 ... WAV to USR GSM 6.10

***        403 ... WAV to USR 4 bit G.721 ADPCM

404 ... WAV to USR 4 bit IMA ADPCM

500 ... Raw 16 Bit DAT Audio to RIFF WAV

-b<bits> ........ create 8 or 16 bit PCM WAV

-w .............. target RIFF WAV type [1]

1 ............. 8 or 16 Bit PCM

6 ............. 8 Bit A-Law PCM

7 ............. 8 Bit �-Law PCM

17 ............ 4 Bit IMA ADPCM

**         49............. MS GSM 6.10

**         771............ embedded USR GSM 6.10 (RapidCommVoice)

-a .............. remove DC component

-g<gain> ........ apply gain (0.1 - 10.0)

-n .............. smooth (noisefilter)

-h<frequence> ... apply HighPass filter

-l<frequence> ... apply LowPass filter

-f .............. apply 300 - 3400 Hz Bandpassfilter

-t<type of filter>

1 ... critical damping

2 ... Bessel

3 ... Butterworth

4 ... Chebyshew 0.5 dB overshoot

5 ... Chebyshew 1.0 dB overshoot

6 ... Chebyshew 2.0 dB overshoot

7 ... Chebyshew 3.0 dB overshoot

-mono ............ make mono WAV from stereo

-stereo .......... make stereo WAV

-stream .......... force to use streaming mode

-nostream ........ disable streaming at all

-p ............... show peak level

-q ............... quiet mode

-d ............... delete source after conversion

-h or /h or -? or /? shows this.

*)   only displayed (and useable) if ADPCMDLL.DLL is detected.

**)  only displayed (and useable) if GSMDLL.DLL is detected.

***) only displayed (and useable) if G72XDLL.DLL is detected.

Due to copyright and patent restrictions, neither of the above mentioned DLL is

distributed. But sourcecode which makes somebody able to create these DLLs is

freely available in internet.

Without ADPCM.DLL, Rockwell 2, 3 and 4 bit ADPCM is decoded with a simple,

no patented, methode. The result is not so good, but speach is well

understandable enough.

The '-w771' option is experimental. This format is not documented anywhere,

but PCM2WAV can read and convert such RIFF WAV type files (thanks to Dmitri

Novoselov for supplying some examples).

PCM2WAV does voice format conversion 'in memory', so large files will result

in huge memory need and/or will increase OS/2 swap file enormous. Stream

conversion (multithreaded) is under study.

The above statement is no longer true :-)

Multithreaded conversion is already implemented and streaming mode is

(hopefully) working well.

PCM2WAV will convert voice data as long as possible 'in memory' (in large

allocated memory blocks) but if size of allocated block exceeds 'available'

memory than sound data is transported directly from/to diskfile.

The decision, when changing from memorymode to streammode is done by checking

total physical memory.

Use '-stream' commandline switch to force streammode, if PCM2WAV misbehaves.

All I had done was, to check conversion time in memorymode against

streammode depending on installed physical RAM (16Mb, 32Mb and 64Mb).

As a rule of thumb: If sound file size is less half the size of installed RAM,

than let decide PCM2WAV what mode it will use, else force streammode

by '-stream' if OS/2 behaves odd.

An example, how to convert a two channel (stereo) WAV file to a sampling rate

of 44100 samples/sec:

PCM2WAV MYWAV.WAV -oSAMPLE.WAV -b16 -stereo -stream -s44100

And preparing the same file for Audio CD Rom:

PCM2WAV MYWAV.WAV -oTRACK01.CDR -k301 -stream

======================================================

Version: 0.05c (July 5, 2000)

Features as described above.

Version: 0.06?

The '-g' option has the ability to use dB values by '+' or '-' sign:

-g10 means factor 10, -g+10 means 10 dB amplification...

Version: 0.07c (April 12, 2003)

Added '-a' option to remove DC component (offset) off audio signal.

Version: 0.07f (May 2003)

Added ability to read directly from a DAT streamer, if capable for audio tape.

Reading is performed by using ASPIROUT.SYS which has to be installed!

It is also possible to read a raw file, containing the audio data (obtained

with 'tape read <filename>.

If reading audio DAT tape use:

pcm2wav -iTAPE0 -o<filename> -k500 -b16 -s44100

TAPE0 is the special name for first detected DAT device (TAPE1 for 2nd ...)