WBIDEOS2 Device Driver Documentation

------------------------------------

Enclosed is the WBIDEOS2.ADD. It contains a number

of changes from the previous driver. This note is

intended to help explain those changes.

Summary of Changes

------------------

Multiple Controller support

- 2 independent controllers can be supported per instance

of this driver.

- Multiple instances of the driver may be loaded

- Support ATA-2 PIO Mode 3

Command Line Switches

---------------------

Sample Command Lines:

BASEDEV=WBIDEOS2.ADD /V /A:0 /U:0 /T:10 /GEO:(1314,7,26,-1) /A:1 /I

BASEDEV=WBIDEOS2.ADD /V /A:1 /P:120 /U:0 /T:10

BASEDEV=WBIDEOS2.ADD /V /A:1 /U:0 /GEO:20

Switch definitions:

/V = Verbose Mode

Displays controller status and drive geometry information

during OS/2 initialization.

/A:0 = Adapter number

Adapter 0 has default parameters for a primary adapter.

Adapter 1 has default parameters for a secondary adapter.

/U:0 = Unit Id

Unit 0 indicates parameters apply to the master unit.

Unit 1 indicates parameters apply to the slave unit.

/T:10 = Error retry time in seconds. (dec)

Indicates the total allowable error recover time for

a request. Error recovery times < 5 seconds will be

ignored. This parameter defaults to 30 seconds. A shorter

interval may be desirable for fault tolerant applications.

/GEO:(1314,7,26,-1) or /GEO:20 = Drive geometry override (dec)

Indicates the Cylinder/Head/Sector geometry for the drive.

The fourth parameter is the Write Precompensation Cylinder

which may be omitted for drives which do not require

precompensation.

As an alternate format standard BIOS drive types may

be used. Types (0-47) are supported. User defined types

48, 49 should be entered directly by in the previous

format.

When specified, this parameter overrides other geometry sources.

If this parameter is absent, other geometry sources discussed

above will be used.

/I = Ignore adapter

The driver automatically attempts to locate and initialize

both the primary and secondary adapters. In some cases other

DASD controllers may appear between the primary and secondary

IDE controllers. In these cases the system should be configured

as follows:

BASEDEV=WBIDEOS2.ADD /V /A:1 /I

BASEDEV=MOREDASD.ADD

BASEDEV=WBIDEOS2.ADD /V /A:0 /I

/CLK:xx = Use in VL-BUS to set VL-BUS clock speed

(Setting this option in VL-Bus is preferred )

xx = 20, 25, 33, 40, 50