Panasonic / MKE device driver information:

Copyright (c) Matsushita Kotobuki Electronics Industries 1989-1995

*****************************************************

*        ATAPI CD-ROM drive setup information       *

*****************************************************

Files on this Disc:

README.DOC	This text file for DOS and OS/2

[Required files for DOS installation]

INSTALL.EXE	Executable file for Installation for DOS

INSTALL.DAT	Setup file for customized executable file

CR_ATAPI.SYS	Device driver file (Version 5.11) for DOS

WDCTRL.386	Device driver for Windows 3.1

[Required files for OS/2 installation]

INSTOS2.EXE	Executable file for installation for OS/2

INSTOS2.DAT	Setup file for customized executable file

MKECR57x.ADD	Device driver file (Version 1.11) for OS/2

MKECR57x.DDP	Setup file for device driver of OS/2

MKECR57x.DOC	Decsription document file for OS/2

/////////////////////////////////////////////////////

///                                               ///

///     Installation Procedure for DOS & OS/2     ///

///                                               ///

///       	 Date:  2/20/95                   ///

/// 	  Written by :  MKE(Panasonic)            ///

/////////////////////////////////////////////////////

*************************************************************

(A) DOS Installation Procedure:

( also supported for Windows 3.1)

*************************************************************

1. Supported CD-ROM drive:

<ATAPI Interface CD-ROM>

CR-572-B (double speed, tray loading type)

CR-574-B (quadruple speed, tray loading type)

2. Installer file name:

INSTALL.EXE (Version 1.00, common file for installation)

INSTALL.DAT (installation's data file for ATAPI CD-ROM)

3. Description for execution:

Please type following from the DOS prompt:

A: or B: INSTALL [return]

*************************************************************

(B) OS/2 2.1, 2.11 and OS/2 Warp installation procedure:

*************************************************************

1. Supported CD-ROM drive:

<ATAPI CD-ROM>

CR-572-B (double speed, tray loading type)

CR-574-B (quadruple speed, tray loading type)

2. Installer file name:

INSTOS2.EXE (Version 2.00, common file for installation)

INSTOS2.DAT (installation's data file for ATAPI CD-ROM)

3. Description for execution:

Please type following:

A: or B: INSTOS2 [return]

/////////////////////////////////////////////////////

///                                               ///

/// Device Driver support for ATAPI CD-ROM drive  ///

///                                               ///

///             Making date:  3/3/95              ///

///             Written by :  MKE(Panasonic)      ///

/////////////////////////////////////////////////////

*************************************************************

(C) DOS device driver support:

( also supported for Windows 3.1)

*************************************************************

1. Supported CD-ROM drive:

CR-572-B (double speed, tray loading type)

CR-574-B (quadruple speed, tray loading type)

2. Device driver name:

CR_ATAPI.SYS (Version 5.11)

3. Driver description:

[/D:xxxxxxx]	---> Device name option

example: DEVICE=CR_ATAPI.SYS /D:MSCD001

*[/P:nnn]	---> ATAPI base I/O Port Address(hex) option

ex.)DEVICE=CR_ATAPI.SYS /D:MSCD000 /P:nnn

nnn==1F0, 170, 1E8 or 168

Primary connection:	/P:1F0

Secondary connection:	/P:170

Tertiary connection:	/P:1E8(Sound Blaster IDE)

Quaternary connection:	/P:168(Sound Blaster IDE)

*[/I:xx]	---> Interrupt number option

ex.)DEVICE=CR_ATAPI.SYS /D:MSCD000 /I:xx

Primary connection:	xx=/I:10,11,12,14 or 15

Secondary connection:	xx=/I:10,11,12,14 or 15

If you use Sound Blaster IDE, please set following

Tertiary connection:	xx=/I:10,11,12,14 or 15

Quaternary connection:	xx=/I:10,11,12,14 or 15

*[/W:n]		---> Waiting value option

When PC reads data from CD-ROM drive, Driver insert waiting

value into read cycle one by one.

If you are using a PC that does not support IOCHRDY, please set

this value(waiting value).

example: DEVICE=CR_ATAPI.SYS /D:MSCD000 /W:n

n==0, 1, 2, ... 255

(106usec/1 wait in 486DX2 66MHz)

*[/Q]			---> Non stop option

Non stop(no message <A>bort, <R>etry) in initializing driver.

ex.)DEVICE=CR_ATAPI.SYS /D:MSCD000 /Q

* : You can omit this option.

*************************************************************

(D) OS/2 2.1, 2.11 and OS/2 Warp device driver support:

*************************************************************

1. Supported CD-ROM drive:

CR-572-B (double speed, tray loading type)

CR-574-B (quadruple speed, tray loading type)

2. Device driver name:

MKECR57X.ADD (Version 1.11)

3. Driver description:

*[/V]		---> Verbose Mode option

Displays controller status and drive type during OS/2

initialization.

*[/A:n]		---> Adapter Number option

Adapter 0 has default parameters for the primary adapter.

Adapter 1 has default parameters for the secondary adapter.

*[/P:nnn]	---> ATAPI base I/O Port Address(hex) option

This parameter overrides the default base I/O port address for

the adapter indicated. The default I/O address for primary is

(1F0) and for seconday is (170).

If you set this parameter, then, you should set /I: parameter.

*[/I:xx]	---> Interrupt number option

This parameter overrides the default IRQ Number for the adapter

indicated.The default IRQ level for primary is (14) and for

secondary is (15).If you set this parameter, then, you should set

/P: parameter.

*[/NI]		---> No Interrupt Mode( Polling Mode ) option

When you use IDE HDD and ATAPI CD-ROM drive at the same adapter

(primary or secondary), the MKECR57X.ADD driver is automatically

used this mode.

*[/IOR]		---> I/O Channel Ready Supported Mode option

For the adapter with I/O channel ready, the MKECR57X.ADD driver

can designate the fastest read mode.

* : You can omit this option.

Examples:

; scan to primary and secondary adapter.

BASEDEV=MKECR57X.ADD /V

; scan to secondary adapter with I/O channel ready.

BASEDEV=MKECR57X.ADD /V /A:1 /IOR

; scan to another adapter(I/O address 1E8  IRQ 11) with I/O channel ready.

BASEDEV=MKECR57X.ADD /V /P:1E8 /I:11 /IOR

!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!

CAUTION:

OS/2 Warp has ATAPI CD-ROM filter driver(IBMIDECD.FLT). If problems occur

using the IBMIDECD filter, modify your OS/2 CONFIG.SYS file as follows:

rem BASEDEV=IBMIDECD.FLT

At the end of the file, add the following statement

BASEDEV=MKECR57X.ADD /V

and copy to root directory on the OS/2 system drive.

In this time, if the CR-ROM drive is attached as master to primary or

secondary, modify your OS/2 CONFIG.SYS file as follows:

BASEDEV=IBM1S506.ADD /A:x /I

The /A parameter refers to the adapter number that the CD-ROM is attached

to.The /I parameter indicates that the IBM1S506 driver should not attempt

to initialize the adapter indicated.

!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!

Trademarks and Copyrights:

OS/2 is a trademarks of International Business Machines Corporation.

Sound Blaster is a trademark of Creative Labs, Inc.

Windows 3.1 is a trademark of Microsoft Corporation.