WARP Celebrity Device Driver Documentation  24/2/96

===================================================

VERSION 1.003H

===============

Prepared by :

Tim Snape

Abbotsbury Software Ltd.

Abbotsbury,

DORSET DT3 4JT

United Kingdom

Tel: (+44) 1305 871543

Fax: (+44) 1305 871688

e-mail abbot1@ibm.net

System Requirements

===================

This version of the driver + the enclosed executables will work on any

version of OS/2 2.X or upwards.

The MCI interfaces to this driver will only work with the latest release

of the multimedia products (Warp GA). ie if you are going to use the enclosed

utilities everything "should" work on all versions of OS/2, if you are going

to be using the MCI interfaces then you will need Warp GA.

Installation

============

PLEASE MAKE SURE EARLY VERSIONS OF THIS DRIVER ARE BACKED UP. THIS

VERSION REQUIRES :

1) .DLL files to be placed in \MMOS2\DLL

2) .INF & .HLP files       in \MMOS2\HELP

3) .SYS file               in \MMOS2

4) .PRO file               in \MMOS2

5) .EXE anywhere

Install the Hauppauge board into the system as described in the Hauppauge

documentation.

Multimedia Installation

=======================

The complete set of driver files can be installed directly from disk

using the multimedia installation utility MINSTALL.

MINSTALL will prompt you for all necessary parameters and is very

easy to use.

Installation by Hand

====================

It will always be preferable to install using the minstall utility however

there may be instances when this is not possible. This section explains

how to install the driver the hard way by hand.

For OS/2, create an entry in the config.sys file of the form :

DEVICE=d:\mmos2\celeb.sys

The driver will use default initial values, you may override these with

a set of parameters on the config.sys line :

DEVICE=d:\mmos2\celeb.sys /P=520 /M=0xb8000

All parameters are introduced by a /, an alpha letter and an = sign (ie /M=).

Numeric parameters are always supplied in hex format, a leading 0x is

optional. In all cases alpha characters can be upper or lower case. Parameters

may be supplied in any order.

The supported parameters are :

/N=n     Change the name of the driver to - CELEB n$. The parameter n can

be any value in the range 1 to 8. The default value is 1, this

parameter is only required if support for more than one board is

required.

/P=nnn   Set the IO port address for the TV board to nnn. The default value

is 0x500 this parameter is only required if support for more than one

board is required or the board is mapped to an address other than

0x500. For information on setting the hardware IO port address refer to

Hauppauge documentation. N.B. The Hauppauge docs refer to the address

ranges as being :

51c

53c

etc.

or :

516

536

etc.

The address ranges this driver requires start at 0x500 and increments

in steps of 0x20 :

500

520

etc.

The current release of the celebrity driver ignores the low 5 bits in

the IO port address. In other words :

500 is equivalent to all numbers in the range 500 -> 51F.

/I=nn    Set the interrupt address for the TV board to nn. The default value

is 0x10 this parameter is only required if support for more than one

board is required or the board is mapped to an address other than 0x10.

For information on setting the hardware interrupt address refer to

Hauppauge documentation.

N.B. This version of the driver does not make use of IRQ's, (and

would offer no advantages if it did) so to save precious IRQ numbers

this option has been disabled.

/M=nnnnn Set the memory address for the TV board to nnnnn. The default value

is 0xb8000 this parameter is only required if support for more than one

board is required.

If your system hangs on reboot, it may be due to an address conflict

between the Celebrity card & another adapter on your system.

/F=file  Set the full path name for the device drivers ini file. The default

file name is C:\CELEB 1.INI. Because this file name contains embedded

spaces it is important to enclose the file name in quotes :

/F='c:\mmos2\celeb 1.ini'

This ini file contains a list of initial values

for device driver variables. They are held in the form :

XOffset=148

YOffset=124

HScale=183

VScale=188

InputFormat=0

InterlaceVGA=1

SLDivider=1247

SLVCOGain=0

SLPhDetGain=3

HSyncVGA=0

VSyncVGA=0

SLAuto=0

NumColours=256

ScreenWidth=1024

ScreenHeight=768

These variables will normally be set up by the configuration utility

program. If you do not have a .ini file at start up then the

latest version of the driver will create one. If there is no .ini

file then MMPM/2 will crash silently for no reason (thats why it is

needed).

The driver can be installed several times to support multiple TV cards.

The drivers should have parameters set correctly for the multiple

instances of the TV hardware

DEVICE=d:\mmos2\celeb.sys /N=1 /P=500 /M=b0000 /F='d:\mmos2\celeb 1.ini'

DEVICE=d:\mmos2\celeb.sys /N=2 /P=500 /M=b8000 /F='d:\mmos2\celeb 2.ini'

DEVICE=d:\mmos2\celeb.sys /N=3 /P=500 /M=c0000 /F='d:\mmos2\celeb 3.ini'

DEVICE=d:\mmos2\celeb.sys /N=4 /P=500 /M=c8000 /F='d:\mmos2\celeb 4.ini'

DEVICE=d:\mmos2\celeb.sys /N=5 /P=500 /M=b0000 /F='d:\mmos2\celeb 5.ini'

DEVICE=d:\mmos2\celeb.sys /N=6 /P=500 /M=b8000 /F='d:\mmos2\celeb 6.ini'

DEVICE=d:\mmos2\celeb.sys /N=7 /P=500 /M=c0000 /F='d:\mmos2\celeb 7.ini'

DEVICE=d:\mmos2\celeb.sys /N=8 /P=500 /M=c8000 /F='d:\mmos2\celeb 8.ini'

Utilities

=========

The file vidtlkt.zip contains a tool kit of useful utilities, documentation

and source code examples. You should download the .zip from :

ftp.wdi.co.uk

username:anonymous

password:your-email-id

The file is called vidtlkt.zip. When you unzip it, unzip to a floppy

for subsequent installation :

pkunzip -d vidtlkt *.* a:

To install from the a: drive, change to the a: drive and invoke :

install <dest_directory>

install c:\vidtlkt

The documentation for the toolkit is extensive and is included with the

.zip file.

Bugs Problems & things To Be Done

=================================

Stereo audio support with latest release of Rev E board - still not

implemented - I am waiting for Hauppauge to send me a board !

Register overflow with multi-monitor configurations - I'll be looking

at this next week.

Setting up the web site + FAQ fora - I'm doing it.

This driver has been tested for all resolutions upto & including 1280 by 1024.

We can only guarantee operation on 640 by 480, but hopefully the driver

should work with most display controllers at most resolutions.

When setting up for display controller work from 640 * 480 upwards. As 640*480

is the easiest to start from.

When selecting a scanning frequency always start from the lowest value.

Support Policy

==============

We believe that all outstanding

problems/bugs/issues not addressed here have been resolved. We are interested

to know of any problems you are experiencing, this can be done :

1) Directly with Hauppauge

2) Direct to the authors (address at the top of this file).

Before using any of these support lines, please ensure that you are using the

latest release of the driver.

The latest release of the driver will always be available on the web site :

http://www.wdi.co.uk/os2tv

or via anonymous ftp access at

ftp.wdi.co.uk

The drivers may be available elsewhere, but since we have no control over

other sites we cannot offer any commitments.

Good Luck

Tim