Note: An asterik '*' denotes that this option is only available

in cFos for CAPI 2.0.

{++} denotes that this command is only available in

'cFos Professional', i.e. when you register 'cFos'.

-----------------------------------------------------------------------

cFos Modem Command Set

-----------------------------------------------------------------------

?       Show a small help screen of AT Commands and important

S-registers.

A       Force Answer

Bn      Select protocol ONLY for outgoing calls

0       X.75 without B3 protocoll, 64000bps     ("ISDNC")

s22=2048, s26=2, s28=2048

1       V.110 async, 38400bps, 8n1              ("ISDNB")

s22=2048, s26=2, s28=2048

2       V.110 async, 19200bps, 8n1              ("ISDNA")

s22=2048, s26=2, s28=2048

3       TELES 128kbps Bundle Protocol

s22=2048, s26=2, s28=2048

4       X.75, Framelen=256, Win.Size=7 (ELINK mode)

s22=framelen, s26=7, s28=256

5       Datex-J (BTX), Framesize=130, Win.Size=7

s22=framelen, s26=7, s28=128

6       same as 5, but switches VT-100 on

7       analogue modem, if possible

8       PPP mode according to RFC1662 (see WHATSNEW)

9       CompuServe via BTX

10      V.110 async, 9600 bps, 8, n, 1

11      V.110 async, 9600 bps, 7, e, 1

12      V.120

{++}            13      X.31

'framelen' refers to the framelen set by -b switch on

commandline; all values are checked not to exceed the

commandline given settings

D..     Dial

F       start a leased line connection

L       dial last number

S       dial stored number (hardcoded, 0=Zaphods BBS)

V       flag connection as SPV

0..9    digits

A..D#*  special "digits"

<nb>s   dial number <nb> and flag connection as SPV

En      Command Echo

0       echo off

1       echo on

Hn      On/Off Hook control

0       on hook, terminate an active connection

1       off hook, reject an incoming call

In      Inquiry

0       product identification and CAPI version and

manufacturer

1       status line output

2       last call statistics

4       last 10 CAPI info dumps

5       some CAPI infos (number of charging infos

and time/date infos) (not CFOSN)

6       some OS-dependant information on cFos

7       PPP statistics

Qn      Quiet mode, result codes display

0       display result codes

1       suppress result code; be Quiet

Sr=n    Set register r to value n (registers are unsigned 16bit ints)

Sr.b=n  Set bit b to n

Sr,b=n  Set bit b to n

Sr?     Query register r

SrH=n   Set high byte of register r to value n

SrL=n   Set low byte of register r to value n

Vn      Verbal mode

0       numeric result codes

1       verbal result codes

Xn      Result code options

0       modem looks (i.e. responds) like a standard

9600 bps modem

1       enables CONNECT <rate>

2       enables RINGING

3       enables RING <incoming caller's id>

4       enables CONNECT[...]/<protocols..>

5       enables ?

6       enables /<cause> on ERROR, NO CARRIER, NO ANSWER

and NO DIALTONE

7       enables /V42B message after CONNECT

8       enables /PPP message after CONNECT

Zn      Restore profile from number n (out of the file PROFILE).

This will further reregister the application at the CAPI.

If no number is given, 0 will be default.

Fn      \

Kn       |

Ln       |

Mn       |  ignored for compatibility

Nn       |

O        |

P        |

T       /

-----------------------------------------------------------------------

cFos extended Modem Command Set

-----------------------------------------------------------------------

*       &Ak=nnn Answer to these MSNs (CAPI 2.0). See CAPI_20.DOC

{++}    &Bn     Number of Channels to use for 'cFos' Channel Bundling, for

next dial command

&Cn     DCD override

0       DCD always high

1       DCD follows connection state: high=connection

active, low=no connection active

&Dn     Status line

0       disable status line (default)

1       enable status line (only if port is initialized)

2       enable status line (only if carrier is high)

3       always display status (cFos/Win)

&En     EAZ: n = '0' ... '9': changes register 18. Default is

EAZ 0.

&F      Reset to Factory defaults and reset. Does not clear

the accumulated charges

&G      Accumulated costs management

0       Display accumulated costs in currency units

1       Clear accumulated costs

&H<str> This sets the hangup string to "+++<str><cr>". E.g. this

enables setting the hangup  sequence  to  "+++ATHE1<cr>"

for the pcANYWHERE "Hayes compatible modem". Default  is

<str>   =   "ATH0",   i.e.   the   hangup-sequence    is

"+++ATH0<cr>".

&I..    Listen to services; changes register 14; takes

'A' (Analog or Audio) and/or 'D' (Data or Digital) as

parameter. Use register S44 to select mode for analogue

calls.

&Kn     Enable/Disable inband-negotiation / V.42bis)

0       disable both

1       enable negotiation and V.42bis

2       enable negotiation without V.42bis

&Lnnn   Listen to what EAZ's; changes register 13;

takes multiple digits or '*' for all. Default is EAZ 1.

&Mn     Select bundling mode.

0       static bundling

1       dynamic bundling

&Onnn   Outdial Number. This number will be used as prefix for

all phone numbers dialed with the ATD command. Default is

none.

* {++}  &Pnnn   Called Subaddress (CAPI 2.0). Default is none.

&Rn     Reregister the application if all ports are idle; use

&R1 to force reregister (and lockout of all active

connections!)

&Sn=..  Set string register

&Sn?    Query string register

&T      TRANX: set the clock to the actual time/date (gotten from

the last connection attempt).

* {++}  &Unnn   Calling Subaddress (CAPI 2.0 and X.31)

&Vn     View profile (n=0..2)

&Wn     Write profile to number n (into a file named PROFILE). If

no number is given, it defaults to 0.

{++}    &Xn     n=1, X.31 activate, n=0, X.31 deactivate

-----------------------------------------------------------------------

cFos Modem Register Chart

-----------------------------------------------------------------------

Reg#      Description                                     Default

S0      Auto Answer on (>=1) / off (==0)                0

S1      Ring count                                      0

S2

S3      ASCII Carriage Return (only used to detect      13

the end of a modem command)

S4      ASCII Line Feed (only used in modem command     10

evaluation)

S5      ASCII Backspace (only used to detect a          8

backspace in modem command evaluation)

S6      Column of callerid display (only if s10.1 == 1, 61

at&d1 or at&d2; used together with s11 (line))

S7      Number of seconds modem waits for a carrier     60

S8

S9      Bit-mapped register:                            32737

.0      enable any further result code

.1      enable CONNECT <rate>, otherwise

CONNECT 9600 is always returned

.2      enable RINGING

.3      enable RING followed by <number>, style depends

on S10.3

.4      enable /X75.. after CONNECT

.5      enable ERROR/<cause>

.6      enable NO CARRIER/<cause> and

NO CARRIER/CHARGE=..

.7      enable /V42B after CONNECT

.8      enable /PPP after CONNECT

S10     Bit-mapped register:                            Default: 0

.0      enable CONNECT.../ID=<callerid>

.1      disable full status line, just display the callerid

on the left side

.2      display numerical values for service indicator /

add. si instead of names

.3      display the callerid, requested eaz, si/add si on

a RING like RING 1234/EAZ1/07/00, in spite of the

default method: RING<cr>ID=1234/EAZ1/07/00

.4      enable +++ATH0<CR> hangup sequence in BIOS emulator

mode (and disable hangup on baudrate change)

.5      (obsolete)

.6      disable multiple CallerID's

{++}            .7      display the baudrate specified in register S47

on CONNECT messages

.8      reject calls with no CallerID

{++}            .9      enable 'Audible RING': each RING message

activates a small ringer

.10     (reserved)

.11     1=disable disconnect on port close (cFos/Win)

.12     handle DTR like: 0=COM.SYS, 1=SIO (cFos/2)

on port open/close and set_DCB IOCTL; see

README.OS2 for more details.

{++}            .13     ignore '?' and 'R' modifiers on incoming

cFos Channel Bundling calls.

S11     Status line line number (0 - screen length-1)   0

Note: only displayed if AT&D1 or AT&D2

S12     Nummer of 1/18 sec. to wait before CONNECT      50

message.

S13     Serviced EAZ Mask                               2

Every bit from bit0.. bit9 enables RING on

this EAZ ('0' - '9')

ex:     0000000010b     answer on '1'

0100000000b     answer on '8'

0001111111b     answer on '0' - '6'

1111111111b     answer all calls

This register is also modified by the AT&L... command.

S14     Serviced SI Mask                                0x80

Every bit enables a service 'cFos' listens to

Bit 0    videophone

Bit 1    telephone

Bit 2    a/b-services

Bit 3    X.21-services

Bit 4    telefax (Group 4)

Bit 5    BTX (64 kbits/s)

Bit 7    data transmission (64 kbits/s)

Bit 8    X.25-services

Bit 9    teletex 64

Bit 10   mixed mode

Bit 15   BTX (new standard)

S15     B-Channel Identification (ffs. 1.TR.6,          0x83

3.2.3.4.4.7)

S16     Source Service Indicator                        7

S17     Source Additional Service Indicator             0

S18     Source EAZ (character: '0'=48.. '9'=57)         '0'

Sent with connect_req message; maybe this has

to be changed on PBX systems.

This register is also modified by an AT&En

command.

S19     Inactivity timer: modem disconnects if          0

no traffic in N minutes; 0==disabled

S20     B-Channel, level 2 protocol                     1

1       X.75 SLP Basis Operation Mode,

mit Implementation Rules nach T.90

2       Transparent-HDLC mit Bit-Stuffing,

Frame-Erkennung und CRC-Check

3       Bittransparent

4       SNA-SDLC

5       X.75 Btx

7       LAPD

8       V.110 mit transparentem B2 protokoll

9       V.110 SNA-SDLC

10      V.110 X.75 SLP nach T.90

S21     B-Channel, level 3 protocol                     4

1       T70 NL fr Leitungsvermittlung (CSPDN)

2       ISO 8208 (DTE/DTE)

3       Level 3 according to T.90, Appendix II

4       transparent

S22     SELECT_B2_PROTOCOL_REQ: Data-length             2048

S23     SELECT_B2_PROTOCOL_REQ: Link-Address-A          3

S24     SELECT_B2_PROTOCOL_REQ: Link-Address-B          1

S25     SELECT_B2_PROTOCOL_REQ: Modulo-Mode             8

S26     SELECT_B2_PROTOCOL_REQ: Window-Size             2

S27     SELECT_B2_PROTOCOL_REQ: V.110 user rate         64

S28     Maximum SEND B2-Framesize                       2048

S29     B3-Protocol-Overhead in Bytes                   0

b3 == 1 (t70nl): 2

b3 == 2 (iso8208): 3

b3 == 4 (transparent): 0

S30     SELECT_B3_PROTOCOL_REQ: LIC                     0

S31     SELECT_B3_PROTOCOL_REQ: HIC                     0

S32     SELECT_B3_PROTOCOL_REQ: LTC                     1

S33     SELECT_B3_PROTOCOL_REQ: HTC                     1

S34     SELECT_B3_PROTOCOL_REQ: LOC                     0

S35     SELECT_B3_PROTOCOL_REQ: HOC                     0

S36     SELECT_B3_PROTOCOL_REQ: Modulo-Mode             8

*       S37     SELECT_B3_PROTOCOL_REQ: Window-Size             2

(CAPI 2.0 only)

S39     Numbering Type/Plan                             0x80

S40     Controller                                      0

S41     Info-Mask-low                                   63

S42     Info-Mask-high                                  0

S43     Default V.110 user rate & Protocol              64

(CAPI 1.1 only)

S44     Audio Mode for incoming calls                   0

0       Voice mode

1       Modem mode

{++} *  S45     X.25 / X.31 delivery confirmation, CAPI 2.0     0

{++} *  S46     X.25 / X.31 group/channel, CAPI 2.0             0

{++}    S47     Baudrate to return on CONNECT, if S10.7=1       9600

{++}    S48     Suppress the first <s48> RINGs.                 0

{++}    S49     number of digits of caller id to ignore         0

(from right to left) when called with CCB

S50     \

S51      |

S52      |   Default Additional Service Indicator and B2-Protocol

S53      |   for every EAZ:

S54      |   If the incoming AddSI is 0 then we get the AddSI

S55      |   from the low byte of register 50+EAZ; the B2-Protocol

S56      |   is got from the high byte.

S57      |   (CAPI 1.1 only)

S58      |

S59     /

S60     Charge limit, main port     \

S61     Charge limit, aux. port      |

S62     Time limit, main port (secs) |

S63     Time limit, aux. port (secs) |

S64     Lower cps limit              | {++} see CFOS.DOC, chapter 12

S65     Upper cps limit              |

S66     Charge simulation timer      |

S67     Pre-off disconnect time      |

S68     Connect tries                |

S69     Delay between dyn.CCB tries /

{++}    S70     Cost of one charging unit (Pfennig, Groschen,Cent,...)

{++}    S71     Amount of money: charging units * S70 (Pfennig, Groschen,..)

{++}    S72     Amount of money: charging units * S70 (DM, Schilling,..)

S75     internal use, do not change

S76     protocol selection for incoming calls without service

signalling: 0 = use X.75, 1 = use outgoing protocols

(default: 0)

S77     What to do, if the string "01910" is found      0

in the number of an outgoing call:

0       nothing changes (default)

1       act as if ATB5 was set

2       act as if ATB6 was set

3       act as if ATB9 was set

S99     internal use, do not change

-----------------------------------------------------------------------

The following registers, marked with (*) are reserved for cFos

for CAPI 2.0 (currently under development) and cannot be used yet.

*      S100  CAPI 2.0  B1 Protocol                             0

0        64 kBit/s, HDLC framing (CAPI 2.0 default)

1        64 kBit/s, bit transparent

2        V.110 async, w/ start/stop byte framing

3        V.110 sync, w/ HDLC framing

4        T.30 for FAX group 3

5        64 kBit/s inverted w/ HDLC framing

6        56 kBit/s, bit transparent

*      S101  CAPI 2.0  B2 Protocol                             0

0        ISO 7776 (X.75 SLP) (CAPI 2.0 default)

1        Transparent

2        SDLC

3        LAPD, Q.921 for D channel X.25

4        T.30 for FAX group 3

5        Point to Point Protocol (PPP)

6        Transparent (ignoring framing errors of B1)

*      S102  CAPI 2.0  B3 Protocol                             0

0        Transparent (CAPI 2.0 default)

1        T.90NL w/ compatibility to T.70NL

2        ISO 8208 (X.25 DTE-DTE)

3        X.25 DCE

4        T.30 for FAX group 3

*      S103  CAPI 2.0  B1 Protocol Config, Rate                0

*      S104  CAPI 2.0  B1 Protocol Config, Bits per Char       8

*      S105  CAPI 2.0  B1 Protocol Config, Parity              0

0=no, 1=odd, 2=even

*      S106  CAPI 2.0  B1 Protocol Config, Stop Bits           0

0=1 stop bit, 1=2 stop bits

*      S107  CAPI 2.0  CIP mask, low                           4

Bit 0       don't provide CIP value

(all other bits must be 0)

Bit 1       speech

Bit 2       unrestricted digital information

Bit 3       restricted digital information

Bit 4       3.1 kHz audio

Bit 5       7 kHz audio

Bit 6       video

Bit 7       packet mode

Bit 8       56 kBit/s rate adaption

Bit 9       unrestr. digital info. w/ tones

Bit 10..15  reserved

*      S108  CAPI 2.0  CIP mask, high                          0

Bit 0       telephony

Bit 1       FAX group 2/3

Bit 2       FAX group 4, class 1

Bit 3       teletex, basic & mixed mode

Bit 4       teletex, basic & processable mode

Bit 5       teletex, basic mode

Bit 6       international interworking f. videotex

Bit 7       telex

Bit 8       X.400

Bit 9       X.200

Bit 10      7 kHz Telephony

Bit 11      video telephony, first connection

Bit 12      video telephony, second connection

Bit 13..15  reserved

*      S109  CAPI 2.0  Info mask, low                        127

Bit 0       Cause info

Bit 1       Date/Time

Bit 2       Display

Bit 3       User-user info

Bit 4       Call progression

Bit 5       Facility

Bit 6       Charging info

Bit 7..15   reserved

*      S110  CAPI 2.0  Info mask, high                         0

Bit 0..15   reserved

*      S111  CAPI 2.0  CIP value                               2

*{++}  S112  CAPI 2.0  Modem-Support, speaker mode             0

Bit 0       disable retrain

Bit 1       disable calling tone

Bit 3..2    guard tone

00   none

01   1800 hz

10   550 hz

Bit 5..4    loudspeaker

00   off

01   on, while dialing & negotiation

10   always on

Bit 7..6    speaker volume

00   silent

01   normal low

10   normal high

11   maximum

*{++}  S113  CAPI 2.0  Modem Support: Max. Connect Rate        0

0=adaptive

-----------------------------------------------------------------------

cFos Modem String Register Chart

-----------------------------------------------------------------------

*      &S0   CAPI 2.0  Bearer Capability, BC

string register of 14 bytes

*      &S1   CAPI 2.0  Low Level Compatibility, LLC

string register of 17 bytes

*      &S2   CAPI 2.0  High Level Compatibility, HLC

string register of 5 bytes

{++}    &S3   X.31  Call User Data

string register of 32 bytes

-----------------------------------------------------------------------

cFos Numerical Result Codes

-----------------------------------------------------------------------

0     OK

1     CONNECT

2     RING

3     NO CARRIER

4     ERROR

5     ( not used )

6     NO DIALTONE

7     BUSY

8     NO ANSWER

9     ( not used )

10     ( not used )

11     RINGING