'cFos' PPP support                     'cFos' ISDN Driver Family

----------------------------------------------------------------

Installation:

-------------

'cFos/Win'  and  'cFos/2'  have  now  built-in  synchronous  PPP

support. The PPP mode is loaded  automatically  when  'cFos'  is

loaded. (The -P switch of elder versions is  no  longer needed).

E.g. with such a Windows SYSTEM.INI section:

[cFos]

param=COM3 COM4

help=c:\cfos\cfos.hlp

capi=c:\windows\capi.dll

capi20=c:\windows\capi20.dll

key=c:\cfos\cfos.key

profile=c:\cfos\profile

factory=c:\cfos\factory

cfosdw=c:\cfos\cfosdw.exe

drv=c:\cfos\cfos.drv

oldcomm=C:\WINDOWS\SYSTEM\XOMM.DRV

(Use SETUP.EXE to install 'cFos/Win'.)

Or with such an OS/2 CONFIG.SYS entry:

device=c:\cfos\cfos.sys COM3 COM4

The MTU (maximum transmit unit) is 2048 bytes IP data.  The  MRU

is not limited,  i.e.  the  received  block  must  fit  in  your

receiver buffer.

Some explanation on how it works:

---------------------------------

PPP is a block oriented protocol. Thus, it is important for  the

receiver to know where a block starts and where it ends.

With analogue modems, which do not transmit blocks, but transmit

a byte stream, PPP is done as "asynchronous PPP", i.e.  the  PPP

blocks are sent in a manner that the receiver  can  recover  the

beginning of the block and it's length.

Under ISDN this may be omitted. The  RFC1662  describes  how  to

send PPP data over ISDN: the async PPP blocks are  converted  to

sync  blocks  and  are  sent  in  HDLC  blocks,  i.e.  the  CAPI

B2-protocol is HDLC.  Received  data  is  coded  as  defined  in

RFC1662 and sent to the receiving application.

'cFos' now makes this available  to  you.  Just  use  "ATB8"  to

enable PPP conversion and HDLC and call out with  "ATD...".  The

CONNECT string will be something like "CONNECT 64000/HDLC/PPP".

Case studies:

-------------

Using sync PPP with 'cFos/2' and the Internet Dialer:

Set the connection type to PPP, use a "B8" in  the  init

string and set  the  other  parameters  to  values  your

provider tells you. Use the "Dial" button to  call  out.

The connection is established  when  the  dialer  writes

"default route assigned through a.b.c.d".  You  may  use

any of the TCP/IP programs then.

Using sync PPP with 'cFos/Win' and Trumpet Winsock:

Start  TCPMAN  and   select   "File"/"Setup".   Activate

"Internal PPP" and set the COM port to  a  'cFos'  port.

Use any baudrate you like. Set "Online Status"  to  "DCD

check". After  altering  the  setup,  exit  Trumpet  and

re-enter it to activate the new settings.

Use "Dialer"/"Manual login"  to  get  direct  access  to

'cFos'. You may verify if you're really on a 'cFos' port

by typing "ATI". The cFos copyright will be displayed.

Use "ATB8" to switch to PPP mode  and  use  "ATD..."  to

dial up your provider. If you get a  "CONNECT"  message,

press Escape. Now Trumpet negotiates  the  PPP  session.

You may start your TCP/IP applications now.

Note: Trumpet Winsock does not support CHAP, only PAP.

Other end's that worked with cFos:

----------------------------------

-- ASCEND MAX

-- ascend Pipeline, Access Router Software Release 4.5

Common problems:

----------------

This is a collection of problems we  had  while  we  tested  the

'cFos' PPP support.

If you don't get a working connection, try changing  the

setting  of  "VJ  Compression",  resp.   "Van   Jacobson

Compression". Under OS/2 Internet Dialler this helped me

connect to a CISCO router.

If you cannot get the  connection  running,  check  with

your provider, how they verify your access. Do they  use

your Caller-ID? Do they use a manual login? Do they  use

PAP or CHAP? Note that Trumpet does  not  support  CHAP.

PAP  may  be  activated  in  Trumpet  under  "File"/"PPP

Options".

Does your provider use a CISCO router?  Does  he  use  a

software release before 10.0? If so, ask him  to  update

to a newer release, because the earlier versions seem to

have problems with ISDN.

Does your provider run anything other  but  "raw  HDLC"?

They could use, for example:

-- async PPP over X.75: use ATB0 to connect to him.

-- sync PPP over HDLC with UI  frames:  send  us  a

mail. We will include support in 'cFos' for this

type of PPP connection. Try to get another  type

of connection from your provider. See  below  on

how to mail to us.

-- sync PPP over HDLC with CISCO-headers: mail  us,

see above.

-- other, even weirder, stuff: mail us, see above.

Troubleshooting help:

---------------------

The ATI2 provides you with information for the last session. You

see how may bytes/blocks were sent  and  received  and  why  the

connection was disconnected.

The ATI7 display gives information about the last session, if it

was PPP (used ATB8). It displays  block  and  error  counts  for

various events.

FCS errors:

Should be zero. If this number is  big  compared

to the number of sent frames, contact the author

of your Internet software.

BIT errors and TXB errors:

Should be zero. If not, please contact  us,  see

below.

EmptyFrameCnt and SilentDiscardCnt:

Just for information. Does not count as an error.

TX1buf overflows and TX2buf overflows:

Overflow of a type of  transmitter  buffer.  The

MTU of 1500 bytes was exceeded.  Configure  your

software to use smaller MTU's.

RXbuf overflows:

The receiver buffer was too full (or too  small)

to hold an incoming data block. The sending side

has to decrease her MTU size.

Mailing us:

-----------

If you want to mail us some error, please  include  logfiles  of

BOTH the sending and receiving side. Fixing a problem with  only

one side of log output is quite tricky, so  please  try  to  get

hold of both logs.

Send mails to chris@zaphod.infox.com (Internet) or

chris lueders, 2:2453/30 (Fido).

E-O-F

-----

Thanx for using 'cFos'.