OS/2

Introduction

This OS/2 device driver software documentation is for all of the communication

boards in the DigiBoard PC/X and MC/X series.  The software is a fully

interrupt-driven, OS/2 API call compatible device driver, and will support up

to 32 serial communication channels.

Board Differences

The DigiBoard PC/4 and PC/8 boards are nearly identical, the PC/4 merely having

four fewer channels.  The DIP switch and jumper setting procedures are the same

for these boards, as is the device driver software installation.

The PC/16 has no DIP switches to set; its addresses are "burned" into PAL

(Programmable Array Logic) chips. The device driver software installation

is the same as for the PC/4 and PC/8, with the exception that the Per-Card

parameters are required. (The default settings are for the PC/4 or PC/8,

and won't work with the PC/16.)

Hardware Installation

Before proceeding with the software installation instructions for OS/2, you

must first configure your board(s) for the correct I/O port addresses and

interrupt request (IRQ) line(s), and install the board(s) in your computer

system, according to the hardware installation instructions in the Installation

Guide for your board, and the instructions on the following pages.

The device driver allows you to specify I/O address and interrupt parameters

for each board installed; however, for single-board installations, the four

and eight-port boards may be installed with a set of pre-configured addresses

and interrupts, and the driver can be loaded without specifying these para-

meters. The I/O addresses and interrupt parameters must be specified for each

PC/16 board installed.

PC/X Boards

PC/4 and PC/8 Boards

The following page shows the correct DIP switch and jumper settings for the

PC/4 and PC/8 boards in single-board installations (if you have just taken

your board out of the box, it may already be correctly set_in this case, use

the following diagrams to verify the configuration). For configurations other

than the ones given here, refer to the Installation Guide for your board.

Set PC/4 and PC/8 boards up according to the diagrams on the following page.

DS1: Status port set to 140h:  Switch  1 2 3 4 5 6 7 8 9 10

on: x - x - x x x x x x

off: - x - x - - - - - -

DIP Switch Settings for I/O Port Addresses

Switch  1 2 3 4 5 6 7 8

DS2: Port 1 set to 100h:     on: x - x x x x x x

off: - x - - - - - -

DS3: Port 2 set to 108h:     on: x - x x x x - x

off: - x - - - - x -

DS4: Port 3 set to 110h:     on: x - x x x - x x

off: - x - - - x - -

DS5: Port 4 set to 118h:     on: x - x x x - - x

off: - x - - - x x -

DS6: Port 5 set to 120h:     on: x - x x - x x x

off: - x - - x - - -

DS7: Port 6 set to 128h:     on: x - x x - x - x

off: - x - - x - x -

DS8: Port 7 set to 130h:     on: x - x x - - x x

off: - x - - x x - -

DS9: Port 8 set to 138h:     on: x - x x - - - x

off: - x - - x x x -

IRQ Selection for PC/4 & PC/8 Boards.

IRQ   |x| | | | | |   |x|x|x|x|x|x|x|x| | |      | X X |   | X X |

3    |x| | | | | |   |x|x|x|x|x|x|x|x|x|x|        P2        P3

|              | | | | | | | | |x|x|     Jumper middle 2 pins

J85            J1      -->       J10

PC/16 Boards

The Status Port and I/O Port address selections are made in a different manner

for the PC/16 board.  Instead of DIP switches, the addresses are programmed

into special integrated circuits called PALs (for Programmable Array Logic).

The PC/16 board has two PAL chips which determine its I/O Port and Status

Port addresses.

The PC/16 must have PAL chips numbered 40000246 (in socket U58) and 4000247

(in socket U46) for the device driver software to work with OS/2.  If for

some reason you have different PAL chips, call DigiBoard Customer Service at

(612) 943-9020.

---------------------------------------- __

|                                      |-|

|                                      | |

|            Pal Chips                 | |

|              u46                     |-|

|______________u58______             __| |

|____________|   |

Three jumpers must be installed on the PC/16 board to complete the hardware

configuration.  To select IRQ3, place a jumper on J2, as shown below.  Other

IRQs may be selected (see the PC/X and MC/X Installation Guide).

IRQ   | |x| | | | |

3    | |x| | | | |

|

J2

A jumper must also be installed across pins 2 & 3 (the bottom two pins) of J7.

See the figure below:

| |

|x|

J7 |x|

Finally, be sure that there is a jumper installed on the daisy-chain connector,

P3, located in the upper right-hand corner of the board.

P3 |XX|

Custom Installation

Choosing Your Own Addresses and Interrupts

If you are installing multiple PC/X boards, or if you wish to use address and

interrupt parameters other than the default selections, you can custom-

configure your board(s) with parameters of your choice. See the guidelines

below for setting up your own I/O Port and Status Port addresses.

I/O Port and Status Port Address for 4 and 8-Port Boards

Each PC/X board must have its own unique I/O Port and Status Port addresses, as

well as a unique IRQ selection. Each range of addresses you choose must not be

used by any other device in your computer.

When loading the OS/2 device driver, only the Status Port address is specified.

The driver expects to find the I/O Ports in the 64 bytes (40 hex) preceding the

Status Port (each I/O Port uses eight contiguous bytes). Thus, if the Status

Port address is set to 140h, the I/O Ports must begin at 100h. If the Status

Port is addressed at 2A0h, the I/O Ports must begin at 260h. See the Install-

ation and Reference Manual for your board for details on how to set I/O Port

and Status Port addresses.

Here are some legal address combinations for the four and eight-port PC/X

boards.

Status            Serial Port

Port               (Channel)

1    2    3    4    5*   6*   7*   8*

140h 100h 108h 110h 118h 120h 128h 130h 138h

188h 148h 150h 158h 160h 168h 170h 178h 180h

250h 210h 218h 220h 228h 230h 238h 240h 248h

2A0h 260h 268h 270h 278h 280h 288h 290h 298h

*8-port boards only

This manual shows the suggested DIP switch and jumper settings for installing

one board. A Status Port address of 140h is chosen, resulting in I/O Port

addresses of 100h, 108h, 110h, 118h, 120h, 128h, 130h and 138h. If you choose

to use these settings, set your DIP switches as shown in the figures. If you

are installing more than one board, you'll have to pick more addresses as de-

scribed above.

When setting the DIP switches on the PC/4 and PC/8 boards, switches 8, 9 and

10 of switch bank DS1 and switch 8 of switch banks DS2_DS9 (DS2_DS5 for the

PC/4) must be set to "ON" to enable the ports.

Choosing an Interrupt

Each PC/X board installed must have its own unique Interrupt Request line

(IRQ), and the IRQ you choose must not be used by any other device in your

computer. The IRQ choices are: 2, 3, 4, 5, 6, and 7.  Each board uses a single

IRQ for all of its channels. See the Installation Guide for your board for

information on selecting the IRQ. Note that while the PC/X board can support

two separate IRQ lines on one board, the OS/2 driver supports only one IRQ

per board.

Remember which addresses and interrupts you choose for each board. You will

have to supply this information when loading the device driver.

MC/X Boards

MC/X boards are configured through the POS (Programmable Option Select)

feature of Micro Channel computers.  To install and configure your MC/X board,

follow the instructions in the PC/X and MC/X Installation Guide.

1.  Boot your system from the IBM Reference Diskette.

2.  Make a working copy of the reference diskette by selecting the "Backup the

Reference Diskette" option.

3.  Select the "Copy an Option Diskette" option, and follow the instructions on

the screen to copy the ADF (Adapter Description File) files from the

DigiWARE ADF Files diskette to the working copy of the reference diskette

just created.

4.  Turn off the computer and install the MC/X board in an available slot in

your computer. Follow the instructions in the MC/X Installation Guide.

5.  Boot the system from the working copy of the IBM reference diskette made in

Step 3.  Run the Automatic Configuration.

6.  Select "View Configuration".  You will see something like the following:

View Configuration

Slot 3 - DigiBoard MC/8

UART Address Range . . . . . . . . . . . . . 0xDB80 -> 0xDBC0

Com2 Status  . . . . . . . . . . . . . . . . Disabled

Interrupt Select . . . . . . . . . . . . . . INT 5

Make a note of the UART Address Range and the Interrupt Select.  This is the

information you will need to set up the OS/2 Driver.  Also verify that "Com2

Status" is set to "Disabled".  If it is not, you can change it by selecting

the "Set Configuration" menu.

The UART Address Range provides only the address of the first port on the

board, and the address of the status port. The following list provides all

of the addresses needed to configure the OS/2 device driver:

MC/4, MC/8:

0xDB80 * 0xDBC0

DB80h, DB88h, DB90h, DB98h,

DBA0h, DBA8h, DBB0h, DBB8h.

0xDC00 * 0xDC40

DC00h, DC08h, DC10h, DC18h,

DC20h, DC28h, DC30h, DC38h.

0xBB80 * 0xBBC0

BB80h, BB88h, BB90h, BB98h,

BBA0h, BBA8h, BBB0h, BBB8h.

0xBC00 * 0xBC40

BC00h, BC08h, BC10h, BC18h,

BC20h, BC28h, BC30h, BC38h.

0xAB80 * 0xABC0

AB80h, AB88h, AB90h, AB98h,

ABA0h, ABA8h, ABB0h, ABB8h.

0xAC00 * 0xAC40

AC00h, AC08h, AC10h, AC18h,

AC20h, AC28h, AC30h, AC38h.

0xCB80 * 0xCBC0

CB80h, CB88h, CB90h, CB98h,

CBA0h, CBA8h, CBB0h, CBB8h.

If you are installing an MC/4 board, only the first four addresses in each

list are pertinent.

MC/16:

0x3000 * 0x307F

3000h, 3008h, 3010h, 3018h, 3020h, 3028h, 3030h, 3038h,

3040h, 3048h, 3050h, 3058h, 3060h, 3068h, 3070h, 3078h.

0x4000 * 0x407F

4000h, 4008h, 4010h, 4018h, 4020h, 4028h, 4030h, 4038h,

4040h, 4048h, 4050h, 4058h, 4060h, 4068h, 4070h, 4078h.

0x4100 * 0x417F

4100h, 4108h, 4110h, 4118h, 4120h, 4128h, 4130h, 4138h,

4140h, 4148h, 4150h, 4158h, 4160h, 4168h, 4170h, 4178h.

0x4200 * 0x427F

4200h, 4208h, 4210h, 4218h, 4220h, 4228h, 4230h, 4238h,

4240h, 4248h, 4250h, 4258h, 4260h, 4268h, 4270h, 4278h.

0x4300 * 0x437F

4300h, 4308h, 4310h, 4318h, 4320h, 4328h, 4330h, 4338h,

4340h, 4348h, 4350h, 4358h, 4360h, 4368h, 4370h, 4378h.

0x5000 * 0x507F

5000h, 5008h, 5010h, 5018h, 5020h, 5028h, 5030h, 5038h,

5040h, 5048h, 5050h, 5058h, 5060h, 5068h, 5070h, 5078h.

0x6000 * 0x607F

6000h, 6008h, 6010h, 6018h, 6020h, 6028h, 6030h, 6038h,

6040h, 6048h, 6050h, 6058h, 6060h, 6068h, 6070h, 6078h.

0x7000 * 0x707F

7000h, 7008h, 7010h, 7018h, 7020h, 7028h, 7030h, 7038h,

7040h, 7048h, 7050h, 7058h, 7060h, 7068h, 7070h, 7078h.

Software Installation

To install the OS/2 device driver, simply copy the file DGX.SYS from the OS/2

device driver diskette onto your hard disk.  Then use a text editor to add the

line  DEVICE=DGX.SYS [parameters]

to the CONFIG.SYS file. Note that if DGX.SYS is not in the root directory, you

will have to give its full pathname. (e.g. DEVICE=C:\BIN\DGX.SYS). If you are

installing a single four or eight-port board with the default parameters, the

parameters portion may be omitted.

For MC/X boards, address and IRQ information is taken directly from the

computer's POS (Programmable Option Select) registers, so the parameters

portion may be omitted if you wish to use the default device names, baud

rate and data type.

DigiBoard PC/16 users must enter the line above with Per-Card Parameters

as follows:

DEVICE=DGX.SYS /P:140 /C:16 /I:IRQ

where IRQ is the Interrupt Request line you have selected for the board.

Default Device Names

With one board in the system, the default names for the devices are COM3,

COM4, COM5, COM6, COM7, COM8, COM9, COM10...

Default modes for each of the channels follow the OS/2 standards of 1200 Baud,

7 data bits, 1 stop bit, even parity, and RTS control-mode.

Command Line Arguments

There are both global as well as per-card arguments that can be specified on

the device driver command line.  If any arguments are given, the defaults in

the data segment of the driver will be ignored.  Use a comma ( , ) to separate

sets of arguments for each card in your system.  Each card in your system must

be fully described by these arguments.

Global Arguments (PC/X and MC/X Boards)

The following Global Arguments set parameters for all channels on all boards

controlled by the device driver. To change parameters for individual ports,

use the utility DMODE.EXE provided on the distribution diskette with the

driver. DMODE.EXE is discussed beginning on page 45 of this manual.

/n:device_names

Sets up the naming algorithm to be used to generate device names.  This con-

sists of an optional new stem followed by the starting device number. Without

the new stem, the stem name "COM" will be used.  Quotes are mandatory to

set a new stem.  Without a number, "3" is assumed.

Examples:

/n:"digi"0	DIGI0, DIGI1, DIGI2 ...

/n:4 			COM4, COM5, COM6 ...

/n:"digi"	DIGI3, DIGI4, DIGI5 ...

/b:baud_rate

OS/2 sets all serial devices to 1200 baud at initialization time.  Use this

option to set a different baud rate.

Example:

/b:9600		Set all channels to 9600 baud.

/d:data_type

OS/2 sets all serial devices to even parity, 7 data bits and 1 stop bit at

initialization time.  Use this option to override those values.  The syntax is

Parity Data Stop (without spaces), where Parity is E, O or N for "Even",

"Odd" or "None"; Data is 8, 7, 6 or 5; and Stop is 1 or 2.  All three values

MUST be provided.

Example:

/d:N81		Set all channels to no parity, 8 data bits, 1 stop bit.

Per-Card Arguments (PC/X Boards Only)

Per-Card arguments MUST be in complete sets identifying the Status Port address

and IRQ number.  The preceding global arguments may be given without

interfering with the default Per-Card settings in the driver file.

Each board in your system must be fully described by these Per-Card arguments.

Use a comma to separate the board sets from one another.

Also note that the channel numbers (COM3, COM4, etc.) will be assigned to the

boards in the same order in which the per-card arguments are given.

/p:Status Port Address

Set the Status Port address. This argument specifies the top of the range

of I/O Port addresses that will be used by a PC/X board.  This address should

correspond to the dip-switch setting for the Status Port.  Individual channel

port addresses should then begin 40 (hex) bytes below this address, occupying

a contiguous block.

Example:

/p:280	(set Status Port address to 280h)

I/O Port for Channel 1 = 240h

.

.

.

I/O Port for Channel 8 = 278h

PC/16 users must specify a Status port address of 140h, as

follows:

/p:140

/i:IRQ

Sets the IRQ line number to one of the legal options for the PC/X board.

Example:

/i:5		(set card to IRQ 5 decimal)

/c:channels

Sets the number of channels (either 4, 8 or 16). This is required for four and

sixteen-port boards. If omitted, eight ports is assumed.

Examples:

/c:4		(PC/4)

/c:16		(PC/16)

Command Line Example

This two-card example uses the preceding GLOBAL examples, and sets the two

eight-channel boards to:

Board 1:	Status Port=140

IRQ 4

Board 2:	Status Port=188

IRQ 3

A command line for the above parameters would read:

device=dgx.sys /n:"digi"0 /b:9600 /d:N81 /p:140 /i:4, /p:188 /i:3

Troubleshooting

If, after installation, the device driver software comes up with a problem

message, first check that your configuration matches the DIP switch settings

on the board. Next, try powering down the system and then bringing it back

up.  If it still fails, call DigiBoard Technical Support.

In general, a good way to test the basic functionality of the driver is to

hook up a terminal to one of the ports (COM3 for example), set it up for the

correct Baud rate, Handshake, and Data type characteristics, and then simply

try to re-direct output to it from the command prompt.  For example:

C:\>dir > com3

If the terminal is set up correctly (the above defaults are typical) you

should get the directory listing coming out on it. If there is no output,

or if the above command hangs, then you may need some technical support.

It is possible that the board is not functioning properly.

DMODE I/O Port Configuration Utility

DMODE is DigiBoard's I/O channel configuration utility. DMODE functions like

the OS/2 MODE command, but is more flexible, and provides some added features.

*  Supports communications device names other than COM1-COM8 (e.g.

COM9, DIGI2, etc).

*  Timeout, handshake modes, etc. can be specified without affecting baud rate,

data type, etc.

*  Any single parameter may be changed without causing all others to be reset

to default values.

*  Maintains the familiar MODE syntax.

DMODE arguments may be entered in any order, and in many cases may be

abbreviated.  For example, a baud rate of 19,200 may be entered simply as "19",

since within the context of the DMODE command, "19" cannot be interpreted as

anything else. Arguments may be separated by spaces or commas, and unspecified

parameters are left unchanged.  DMODE returns the settings of all parameters

after any requested changes have been made. If no parameters are given, DMODE

simply returns the current settings.

The syntax of DMODE is: dmode port_name [parameters]

Port names and parameters are not case sensitive, so upper and lower case

letters may be freely mixed without effect on the operation of the DMODE

command. The following parameters are accepted:

Baud rates can be specified with 2 to 5 digit designations, according

to the following table:

Accepted Values           Baud Rate

11   110                    110

15   150                    150

30   300                    300

60   600                    600

12   120   1200            1200

24   240   2400            2400

48   480   4800            4800

96   960   9600            9600

19   192   1920   19200   19200

Character Length

Characters may be 5, 6, 7 or 8 bits long. Enter the appropriate number in the

DMODE arguments.

Parity	   Enter the desired parity checking method with a single letter E, O,

N, M or S for Even, Odd, None, Mark or Space.

Stop Bits  Enter the desired number of stop bits.  Recognized values are 1, 2

and 1.5.

Flags	   DMODE supports the following flags in the same way as the OS/2

MODE command.  Refer to your OS/2 documentation for explanations

of the flags. To set a flag, enter the mnemonic for the flag,

followed by an equals sign (=), and the desired value (ON, OFF,

etc.).

Read Timeout Flag

TO=ON      Read timeout in effect

TO=OFF     No timeout

XON/XOFF Handshake Flag

XON=ON	XON/XOFF handshaking enabled

XON=OFF	XON/XOFF handshaking disabled

Input DSR Sensitivity

IDSR=ON	Enables Input DSR Sensitivity

IDSR=OFF	Disables Input DSR Sensitivity

Output DSR Sensitivity

ODSR=ON	Enables Output DSR Sensitivity

ODSR=OFF	Disables Output DSR Sensitivity

Output Handshake on CTS

OCTS=ON	Enables CTS output flow control

OCTS=OFF	Disables CTS output flow control

DTR Signal Control

DTR=ON	Sets DTR to "ON" or HIGH

DTR=OFF	Sets DTR to "OFF" or LOW

DTR=HS	DTR used for output flow control

RTS Signal Control

RTS=ON	Sets RTS to "ON" or HIGH

RTS=OFF	Sets RTS to "OFF" or LOW

RTS=HS	RTS used for output flow control

RTS=TOG	Toggles RTS from LOW to HIGH, or from HIGH to LOW

Alt-Pin Processing

ALT=ON	Internally "swaps" DSR and DCD signals (see

explanation below)

ALT=OFF	DSR and DCD are not swapped

DigiBoard products use 10-pin RJ-45 connectors in order to support

all of the signals normally available on a 9-wire RS-232 connection.

However, many commercially available RJ-45 connectors have only

8 pins.  8-pin RJ-45 plugs will fit in a 10-pin jack, but the first and

last pins (numbers 1 and 10) are not connected.  DigiBoard uses

these pins for the modem control signals RI (Ring Indicator, pin 1)

and DCD (Data Carrier Detect, pin 10).  While Ring Indicator is

rarely a concern (since most modems have auto-answer capability),

Data Carrier Detect is often monitored to insure that the modem has

established, and is maintaining, a valid connection with a remote

modem.

To facilitate the use of 8-pin RJ-45 connectors in modem

applications, a special DMODE flag, ALT, has been provided.  ALT

instructs the device driver to monitor pin 2 for DCD instead of pin

10.  The DSR (Data Set Ready) signal, normally on pin 2 is switched

to pin 10. Since most programs don't monitor DSR, this signal can

be safely ignored. See the Installation Guide for your board for cabling

diagrams and pin assignment charts for DB-25 and RJ-45 connectors.

Examples

dmode digi12

Returns the current settings of device digi12.  No changes are made.

dmode com5 2

Configures COM5 for 2 stop bits.  All other parameters remain unaffected.

dmode com5:19 rts=ON

Configures COM5 for a baud rate of 19,200 and sets RTS to "ON" or HIGH.

dmode com5 8

Configures COM5 for 8 data bits.  No other parameters are affected.

dmode comdev83 IDSR=OFF, TO=off e 7 12

Sets COMDEV83 to disable Input DSR Sensitivity, disable input timeout,

select even parity, 7 data bits and a baud rate of 1200.

dmode com5 19 ALT=ON

Sets up COM5 for a baud rate of 19,200, and enables Alt-Pin Processing.

dmode com5 e alt=off

Sets COM5 for even parity checking and disables Alt-Pin processing.