������������������������������������������������������������������������������

�                                                                            �

�                     ������������������������������                         �

�                     �      AVGA3   BULLETIN      �                         �

�                     �   ����������������������   �                         �

�                     ������������������������������                         �

�     All files in diskette 1 remain orginal format and contains, please     �

�     reference to user's manual for installation instructions.              �

�     Diskette 2 contains advanced Windows and OS/2 display drivers with     �

�     self-extracting format. Following chapters describe decompression      �

�     and installation instructions.                                         �

�                                                                            �

�     <F1> : Chapter 1 : How to extract diskette 2                           �

�     <F2> : Chapter 2 : Windows display drivers & setres                    �

�     <F3> : Chapter 3 : OS/2 2.0 display drivers                            �

�                                                                            �

�                                                                            �

�                                                                            �

�                                                                            �

�                                                                            �

�                                                                            �

�                                                                            �

�                                                              Version :1.1  �

������������������������������������������������������������������������������

Chapter  1

HOW TO EXTRACT DISKETTE 2

- If you want extracted files saved in diskettes, two 1.2MB diskettes or

1.44MB diskettes will be needed.

. For Windows drivers diskette,make 3 sub-directorys: WIN30,SETRES,SYSTEM

For example :  insert a blank diskette into driver A

B: MD WIN30   [Enter]

B: MD SYSTEM  [Enter]

B: MD SETRES  [Enter]

. For OS/2 2.0 drivers diskette,take another diskette and make 2 sub-

directorys : SETRES,SYSTEM

For example :  insert another blank diskette into driver B

B: MD SYSTEM  [Enter]

B: MD SETRES  [Enter]

. Copy WIN31.EXE to root-directory   of Windows drivers diskette

Copy WIN30.EXE to WIN30 directory  of Windows drivers diskette

Copy MSETRES.EXE to SETRES directory of Windows drivers diskette

Copy MSYSTEM.EXE to SYSTEM directory of Windows drivers diskette

For example :  insert master diskette into driver A and Windows

drivers diskette into driver B

COPY A:\WIN31.EXE B:\              [Enter]

COPY A:\WIN30.EXE B:\WIN30\        [Enter]

COPY A:\MSETRES.EXE B:\SETRES\     [Enter]

COPY A:\MSYSTEM.EXE B:\SYSTEM\     [Enter]

. Copy OS2.EXE to root-directory of OS/2 drivers diskette

Copy OSETRES.EXE to SETRES directory of OS/2 drivers diskette

Copy OSYSTEM.EXE to SYSTEM directory of OS/2 drivers diskette

For example :  leave master diskette in driver A and insert OS/2

driver diskette into B

COPY A:\OS2.EXE B:\                [Enter]

COPY A:\OSETRES.EXE B:\SETRES\     [Enter]

COPY A:\OSYSTEM.EXE B:\SYSTEM\     [Enter]

. Every execution-files is a self-extracting files, extract it before

Windows & OS/2 drivers installation.

For example :

insert Windows drivers diskette to driver A and insert

OS/2 drivers diskette to driver B

A:\WIN31.EXE           [Enter]

A:\WIN30.EXE           [Enter]

A:\SYSTEM\MSYSTEM.EXE  [Enter]

A:\SETRES\MSETRES.EXE  [Enter]

B:\OS2.EXE             [Enter]

B:\SYSTEM\OSYSTEM.EXE  [Enter]

B:\SETRES\OSETRES.EXE  [Enter]

- Windows and OS/2 drivers are extracted and saved into two different

diskettes, please reference to Chapter 2 and Chapter 3 for detail insta-

llation.

Chapter  2

WINDOWS DISPLAY DRIVERS & SETRES

Installing Windows 3.1 display drivers

To install the Windows 3.1 drivers from the DOS prompt,

proceed as follows:

1.Insure that Windows 3.1 already installed on your computer.

2.From your Windows directory,at the DOS prompt,type SETUP and press

[Enter] to run the Windows SETUP.EXE program. Follow the

instructions on the screen.When you come to the screen which lists

the hardware and software components such as display adapter

(e.g. VGA,CGA,etc.),keyboard type,mouse type,etc.,go to the

Display seclection by using cursor keys to move the highlighted

bar and press [Enter].

3.From the next memu listing of display options,scroll to the bottom

of the list,and highlight the following text:

Other (Requires disk provided by a hardware manufacturer)

Press[Enter],and when prompted,insert the Windows display driver

disketter into A and type

A:\[Enter]

4.You will see the list of drivers and their assoicated resolutions,

such as:

Cirrus 54xx v1.1,1024x768x16

Cirrus 54xx v1.1,640x480x16

Cirrus 54xx v1.1,640x480x256

Cirrus 54xx v1.1,640x480x64K

Cirrus 54xx v1.1,800x600x16

Cirrus 54xx v1.1,800x600x256

5.Highlight by moving the cursor to the desired display driver, and

then press [Enter].

6.Continue with the remainder of the SETUP procedire.

To install the Windows3.1 drivers from within Windows, proceed as

follows:

1.Insure that Windows3.1 is already installed on your computer

and start Windows.

2.From the Main Window of the Program Manager run the Windows 3.1

Setup program.

3.Select Change Systems Settings...from the Options memu of the

Setup program.

4.Click on the down arrow at the right side of the Display:

Line.Scroll to the end of the list of available display drivers

and select Other display (Requires disk from OEM)...

5.Insert the Windows display driver diskette into drive A and type

A:\[Enter] as the pathname,then click on OK.

6.You will see the list of available drivers and their associated

resolutions,such as:

Cirrus 54xx v1.1,1024x768x16

Cirrus 54xx v1.1,640x480x16

Cirrus 54xx v1.1,640x480x256

Cirrus 54xx v1.1,640x480x64K

Cirrus 54xx v1.1,800x600x16

Cirrus 54xx v1.1,800x600x256

7.Highlight by moving the cursor to the desired display driver,and

then click on OK.

8.Continue with the remainder of the Setup procedure.The change will

not take effect until Windows is restarted.

Installing Window 3.0 display drivers

To installing the Windows 3.0 drivers,proceed as follows:

1.Insure that Windows 3.0 is already installed on your computer.

2.From your Windows directory,at the DOS prompt,type SETUP and press

[Enter] to run the Windows SETUP.EXE program. Follow the

instructions on the screen.When you come to the screen which lists

the hardware and software components such as display adapter

(e.g. VGA,CGA,etc.),keyboard type,mouse type,etc.,go to the

Display seclection by using cursor keys to move the highlighted

bar and press [Enter].

3.From the next memu listing of display options,scroll to the bottom

of the list,and highlight the following text:

Other (Requires disk provided by a hardware manufacturer)

Press[Enter],and when prompted,insert the Windows display driver

diskette into A and type

A:\WIN30[Enter]

You will see the list of drivers and their associated resolution,

such as:

Cirrus 54xx v1.1,Multi-res 16 colors

Cirrus 54xx v1.1,Multi-res 256 colors

Highlight by moving the currsor to the desired display driver, and

the press[Enter].

When Windows SETUP asks you to insert the Cirrus Logic GD54xx

Windows 3.x Fonts Diskette,leave the same disk in the drive and

enter the same disk drive as you did above,but instead reference the

SYSTEM directory. For example,type the following:A:\SYSTEM[Enter]

Later you will be asked to insert the Cirrus Logic GD54xx Windows 3.0

Driver Diskette,this time use the WIN30 directory. For example,type the

following:

A:\WIN30[Enter].SETUP will complete the installation.

Configuring the drivers for Windows 3.0

The GD54xx Multi-resolution color drivers require that a section be

present in your SYSTEM.INI file. The Windows 3.0 SETUP program must be

used to select the default configuration of the 16 or 256 color drivers.

After exiting the SETUP program,edit the SYSTEM.INI file to change the

resolution manually. Any text editor may be used,including EDLIN, or

EDIT that came with your version of DOS,or NOTEPAD found in the accessories

group of your Windows 3.0 installation. Find the section [CLVGA}. The

[CLVGA] section will not exist when you first setup Windows 3.0.

You must add one to the end of the SYSTEM.INI file.

Type in the apporpriate section from the following table.

Resolution        16 Color Drivers     256 Color Drivers

640x480           [CLVGA]              [CLVGA]

VIDEOMODE=18         VIDEOMODE=95

800x600           [CLVGA]              [CLVGA]

VIDEOMODE=106        VIDEOMODE=92

1024x768          [CLVGA]              [CLVGA]

VIDEOMODE=93         VIDEOMODE=96

You will find that it is easiest to use the SetRES utility that comes

with your Cirrus Logic VGA to configure resolutions and color selections.

Run the INSTALL utility on the Windows driver disk to copy the drives and

the SetRES utility to your hard drive.

Then run SetRes from within Windows to configure your installation.

Chapter  3

OS/2 DISPLAY DRIVERS

This installation guide assumes the user to be familiar with the mouse

interface of the OS/2 2.0 Desktop Shell environment.  If additional

instructions are needed, please consult the OS/2 Tutorial in the Information

folder.

To install OS/2 2.0 PM Display Driver and Virtual Device Driver:

Save a copy of the display.dll in \os2\dll.

If you are currently running OS/2 2.0, open a DOS session by

double clicking on a DOS Full Screen icon from the OS/2 Command

Prompts folder. The Command Prompts icon is in the OS/2 System

folder.

Move to the \os2\dll directory by typing 'CD \OS2\DLL'

at the DOS prompt.

Type 'COPY DISPLAY.DLL IBMVGA.DLL /B' at the DOS prompt

to make a backup copy of display.dll.

If you are currently running MS DOS, follow the above steps for

a DOS session to make a backup copy of the display.dll.

Boot OS/2 to continue with the installation.

Invoke OS/2's Device Driver Install utility to copy device drivers from

the Driver and Utilities for OS/2 diskette to the hard disk.

Insert the Drivers and Utilities for OS/2 diskette in drive A:.

Double click on the Device Driver Install icon in the System

Setup folder.  The System Setup icon is in the OS/2 System

folder.

Click on the Install button from the OS/2 2.0 Device Driver

Installation pop up window.  The Select Device Drivers pop up

window will display a list of device drivers such as:

Cirrus Logic VGA - 800x600x16 on 36 MHz monitor

Cirrus Logic VGA - 800x600x16 on 40 MHz monitor

Cirrus Logic VGA - 800x600x16 on 50 MHz monitor

Cirrus Logic VGA - 1024x768x16 on 64 MHz monitor

Cirrus Logic VGA - 1024x768x16 interlaced on 44.9 MHz

monitor

To assist in determining what capabilities your monitor

has, refer to the following list:

36 MHz monitor: Super VGA, Extended Super VGA.

ie: NEC 2A, NEC II

40 MHz monitor: Multi-freq.

ie: NEC 3D

50 MHz monitor: Extended multi-freq, Super

multi-freq, Extended Super multi-freq.

ie: NEC 3FGx, NEC 4D, Sony CDP-1304

44.9 MHz monitor: 8514-compatible, Super VGA,

Extended Super VGA, Multi-freq.

ie: IBM 8514, NEC 2A, NEC II

64 MHz monitor: Extended multi-freq.

ie: NEC 3FGx, NEC 4D, Sony CDP-1304

Select the device driver to be installed from the list and

click on the OK button.  Device drivers are copied to their

corresponding directories.

Click on the Exit button, then the Yes button.  If the current

display.dll is in use, a dialog box will pop up to notify the

user that changes will be in effect on the next reboot.  Click

the OK button to exit the installation utility.

Modify the config.sys file in the root directory so that OS/2 will

install the correct device driver on the next reboot.

Click on the OS/2 System Editor icon from the Productivity

folder which resides in the OS/2 System folder.

Select Open from the File menu.  Open config.sys from the root

directory.

Search for the command line which contains vvga.sys.  For

example, 'DEVICE=C:\OS2\MDOS\VVGA.SYS'.

Select Find from the Edit menu.

Type 'VVGA.SYS' in the Find: edit box.

Uncheck Case sensitive option if it is checked.

Check Wrap option so a tick mark appears in the check

box.

Click on the Find button.

Once the command line is located, select the Cancel button.

Highlight the command line with the mouse, then select Copy

from the Edit menu.

Click at the end of the command line, press 'ENTER' key to

insert a new line below the current command line.

Select Paste from the Edit menu to make a copy of the command

line.

Type 'REM ' at the beginning of the original command line.

Change vvga.sys in the pasted command line to CLVSVGA.SYS.

Select Save from the File menu and save the changes.

Double click on the system icon to exit the editor.

Shutdown the system and reboot for the changes to take effect.

To install Windows 3.0 driver for OS/2 2.0:

This section assumes that the WIN-OS/2 Support has been installed.

If that option has not been selected at the initial OS/2 installation,

please use the Selective Install utility that is in the System Setup

folder to first install WIN-OS/2 SupporT.

If you do not currently have a WIN-OS/2 session running, start one.

Open a WIN-OS/2 session by double clicking on a WIN-OS/2

icon from the OS/2 Command Prompts folder. The Command Prompts

icon is in the OS/2 System folder.

Run the install program to copy the WIN-OS/2 Drivers and SetRES Utility

to the hard disk.

In the Program Manager, choose File Run.

Specify the diskette drive where the Drivers and Utilities for

OS/2 Diskette is inserted, and run the install program.

Type 'A:\INSTALL'

When prompted,specify to copy the files to the \os2\mdos\winos2

directory.

Set Resolution may now be run by selecting the SetRes icon.

Use the SetRes utility to select either 16 or 256 colors and

the desired resolution for future Win-OS/2 sessions.

For help on the SetRes utility, select Help from the program

menu.