NOTE: READ THIS DOCUMENT IN ITS ENTIRETY!

****

9/6/98:

This doc now updated to include an email sent us by

Daniella Engert <d.engert@odn.de> with what could be

a very simple solution for some users.

Also Note:

IBM's Warp4 FixPak #8  has this note in a readme:

"We have temporarily removed the AIC7879.ADD driver from

this fixpak." and "we suggest you obtain the latest version from

the ibm ddk driver site or from adaptec.com."

Lots of luck with Adaptec.com, folks!

(It's therefore implied that other changes are forthcomming

and who knows ..maybe they'll even solve the absurd

'Same name ..different driver' inconvenience!   In the

meantime, the included drivers do work.

*****************

(ref: This aic7895c.zip contains an email'd doc from IBM re:

instructions / installation of Warp using the Adaptec AIC7895 and

related scsi controllers)

NOTE: This orig doc describes frustrations in installing the OS/2 Warp

driver for the Adaptec AIC7895 Dual Channel onboard scsi controller.

READ THE LAST SECTION FOR IBM'S RESPONSE TO ME, GIVING INSTRUCTIONS

FOR PROPERLY INSTALLING THIS DRIVER!  Ie, don't get frustrated by

this first section

The Adaptec AIC7895 onboard scsi controller may be used by motherboard

mfgr's in ways that are not supported by Adapted. (Eg, the Intel DK440LX

and similar Intel boards)

Consequently, the drivers must be supplied by the motherboard mfgr, and

supported by them.

Lots of luck with Intel, btw...

Nonetheless,  this enclosed driver is for the 7895 onboard controller, even

though it uses the same name as a previous version(s) of the driver.

(Real smart move, Adaptec and IBM !!  Blah!!!!!  That the name is identical

to an incorrect driver found on the cdrom for Warp3 and W4 is the primary

reason for problems).   Both drivers are called:  AIC7870.ADD  but only the

133k sized one can handle the AIC7895 controller.

Easy to tell them apart:  the updated 7895's is 130k+   ...prev. versions are

under 40k.

This driver works without problems on my DK440LX Intel mobo.

Source:

1) Intel's nightmare of a 'support' site ... from which this important

and good driver has seemingly been removed.  Alas, the MS/Intel Consortium

from Hell is alive and well ..  They seem to have no prob. making M$ stuff accessible.

2) Email rec'd from IBM Developer site.

3) IBM driver group

Note: there are problems in installing Warp3, Warp4 with this controller:

All methods I've tried (incl 'remote lan' , modifying config.sys to the max, etc)

have not resolved it for Warp4:   a new Warp4 install under all circumstances

will overwrite this new driver with that form the CD and error out.

(You need this driver to work your cdrom, so it's got to be on the IBM

Install floppy #2 of 3  ...aka: Disk_1   (Thanks, ibm for maintaining a

totally nonsensical-to-the-gen-public approach to identifying disks)

Even an rspinst.exe install that we attempted using 'remote floppy install'

disks that did work inso far as starting the process  bombed out at a

attempt by the install to read the 'now improperly overwritten' aic7870.add

driver ..ARRRRRRGHH

The improper overwriting occurs at about disk_7 's  copying, and there

is NO opportunity to halt the process to put the correct (enclosed) version

onto the install drive.  (Making the driver +R does not work, naturally)

A response file we carefully edited from the sample.rsp  didnt work either,

... no rspinst.exe would even begin.

Our solution was desperate:  we simply restored a tape backup of a Warp 4

installation on another machine (a 486) to the P2 PCI machine.  we expected

total failure: was rewarded with total success after merely changing the

controller driver by means of a floppy boot (using Warp 3 Util. disks !)

and replacement of the AIC7870.ADD with the proper version.  Fortunately,

OS/2 's entire knowledge about the machine's bus type seems to be within

the kernel, making this possible.

*******

We received the following from IBM as instructions for proper installing of Warp4

and this driver:

Thank you for your mail.  I've been researching this problem/error, and

believe

the information/files below should fix this issue.

First, you'll need to use AIC7870.ADD (renamed to AICNEW.ADD), from the

Adaptec

driver site at: http://www.adaptec.com/support/overview/scsiha.html

(You'll need

to use the AHA-3940 Family drivers which support the AIC-7895 chipset)

to replace the driver on the diskette, and possibly after installation

to

replace the driver installed from the Warp v4 CD-ROM.

Next, go to IBM's website at:

http://service.software.ibm.com/os2ddpak/html/FCF6C3F9EF462CCC8625644900

6F9599.

html

This document is also copied below:

Follow this Warp Connect and Warp V4 procedure for installing OS/2

whenever the

Adaptec 7800 Family Manager Set v2.1 for OS/2 driver is required for the

boot

drive.

1.Type cdinst from CDROM drive to create the boot diskettes.

2.Remove the following files from Disk 1

aha152x.add

aha154x.add

aha164x.add

aha174x.add

aic7770.add

aic7870.add

3.Rename the new Adaptec driver to AICNEW.ADD and copy to Disk 1.

4.Note: Follow this step only if installing Warp V4

Remove the file called bundle from Disk 2 and copy aicnew.add to

Disk 2.

5.Edit config.sys (In Disk 1) and remark out the basedev statements

for all of the above deleted drivers.

6.Add a new Basedev statement in config.sys

BASEDEV=AICNEW.ADD

7.In config.sys make sure the following statement is present

SET SAVECONNECT=1.

By default, this statement is inserted by cdinst.

8.At the end of installation, aicnew.add gets copied to the root

partition of the boot drive, and the config.sys will have a

corresponding BASEDEV statement. You can choose to copy this driver

to the c:\os2\boot directory. Do not rename this driver back to

AIC7870.ADD.

-----

NOTE - YOU MIGHT NEED TO BOOT FROM DISKETTE AFTER INSTALLATION AND

VERIFY

AICNEW.ADD IS IN C:\OS2\BOOT.  IF SO, HE WILL NEED TO USE TEDIT (FROM

THE DISKETTE) TO VERIFY C:\CONFIG.SYS DOESN'T LOAD AIC7870.ADD AND HAS

AICNEW.ADD AS THE FIRST LINE OF CONFIG.SYS.

Regards,

<name withheld>

Developers Website

******** end of IBM's email to us

6/1/98:

IBM released drivers dated 5/15/98

we have included prev AIC7870.ADD drivers but named as <DriverDate>.ADD

in case prev versions might be to your advantage. Just rename them

to be AIC7870.ADD as might suite your purposes.

As driver devel. seems to be continueing, we shall continue to include the

prev versions with any package subsequent to this one, and any new

version of ' 7800fam.exe '  that might  contain the latest updates.

jonander@idt.net  &

becka@doctormail.com

*******

Following is from Daniella Engert, as ref'd at top of this doc:

Hi Jon,

On Sat, 05 Sep 1998 22:56:36 -0500, jon t.k. anderson wrote:

>Can it be that simple??

In our company, we got 2 new machines with a GigaByte GA6BXDS board (dual

PentiumII

with a Adaptec dual channel UW SCSI chip on it); my colleagues was equipped

with

one PII-350, mine with two PII-266. There were a couple of disks with the

latest

Adaptec drivers (3.0 or 3.01 if memory serves me right). At first we got no

machine

installed with OS/2 (hans, traps, anything you like...) until my colleage

found

this /!PCIHW tip in the README of the driver disks. It essentially switches

the

method, how the drivers try to find the Adaptec chips. By default, the drivers

access the PCI configuration hardware directly, with this switch they use the PCI

BIOS.

ciao,

Dani

***** end of addendum